3GPP TSG-RAN WG4 Meeting #112bis				                  R4-2415585

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	HPUE Tx requirement in IoT NTN

Agenda item:	6.8.2.3.1

Document for:	Discussion

# Background

In RAN#103, a new WID: enhanced requirements and test methodology for NR and IoT NTN has been approved [1]. From core requirement, the target of this WID is to support HPUE for both NR and IoT NTN and also introduce the 3MHz channel BW into the NTN system.

Supporting HPUE was discussed in RAN4#112, where the agreements are captured in [2]. In this contribution, we further express our views regarding the HPUE in IoT NTN system based on those agreements.

# HPUE Architecture for IoT NTN

For the UPUE architecture, the following agreement has been made [2]:

- Agreement:

  - For PC2 (NR NTN), 1 Tx and 2 Tx, are to be considered for requirements.

  - For PC2 IoT NTN, at least 1Tx shall be considered

  - For other PCs (both IoT NTN and NR NTN), FFS

Currently, only single Tx has been considered for legacy IoT devices. Therefore, it is reasonable to prioritize the single Tx architecture to define the core requirement for PC2 as well, which will minimize the impact on the RF implementation of IoT devices. However, 2Tx can still be studied in parallel so that RAN4 can check its feasibility for use in IoT devices.

Proposal 1: RAN4 can focus on the single Tx architecture for PC2 IoT NTN but further study the feasibility of 2Tx architecture.

# A-MPR and Emission limit for IoT NTN

Unlike NR UEs, no HPUE is defined for eMTC and NB-IoT. Therefore, there is a lack of HPUE TN requirements as a reference for IoT NTN, and the corresponding MPR and AMPR values of the HPUE need to be re-evaluated based on reasonable implementations of IoT devices.

Observation 1: Due to the lack of TN requirement as a reference, the MPR and AMPR values of the HPUE need to be re-evaluated based on reasonable implementations of IoT devices.

Another critical aspect of IoT NTN, which has already been encountered even for PC3, is to meet several ETSI emission limits. A couple of problematic scenarios have been illustrated in Fig.1 and Fig. 2 for b255 and b254 for PC3, respectively. It can be expected that those scenarios will be even more challenging for HPUEs. For the out-of-band emission limit, a guard band on the edge of the band might be helpful. For example, around 200 kHz guard band is needed for b255 to meet the OOB emission limit from ETSI EN 301 681, as shown in Fig. 1. However, the in-band emission limit would be tricky to resolve, as shown for the b254 v.s. ETSI EN 301 441 in Fig. 2. Based on the replied LS from ETSI [3][4], some actions are ongoing in ETSI to create a new harmonized standard for IoT NTN operation. It is suggested 3GPP wait for the ETSI progress before we capture any ETSI limits in 3GPP HPUE for IoT NTN. For other regulatory requirements applicable to IoT NTN, 3GPP should also scrutinize them to determine if the existing solution (guard band or AMPR) would be sufficient for HPUE.

Fig. 1. The comparison of ETSI OOB emission limit in ETSI EN 301 681 vs. 3GPP SEM for NB-IoT in b255.

Fig. 2. The comparison of ETSI IB and OOB emission limit in ETSI EN 301 441 vs. 3GPP SEM for NB-IoT in b254.

Proposal 2: RAN4 shall carefully study the corresponding emission limit from regulators and specify the method to ensure the 3GPP HPUE IoT NTN does not violate them.

Proposal 3: RAN4 may wait for ETSI to specify the new harmonized standard for HPUE IoT NTN before capturing any emission limit from ETSI.

Since some of the emission limits have already been challenging for PC3, meeting those limits with an even higher power class has become more challenging. Fully relying on the A-MPR may not be a practical solution, but guard band or guard RB can be considered in some cases, especially for out-of-band emission.

Proposal 4: In addition to the AMPR, guard band/RB may also be considered for HPUE IoT NTN to meet the emission limit.

# SAR solution

For the SAR solution, the following agreements have been made in RAN4#112[2]:

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

For NTN systems in handheld devices, it is possible that such a system would be used for emergency cases, and therefore, the link budget is critical. Therefore, solutions other than P-MPR may need to be supported. For normal TN handsets, the duty cycle is a well-specified approach so that the network can lower the duty cycle and reduce the device's P-MPR value. Therefore, a similar approach can be applied in NTN as well.

Observation 2: the NTN system in handheld devices might be used for emergency cases; therefore, the link budget is critical. Therefore, relying only on a P-MPR-based solution to meet the SAR limits is not sufficient.

Proposal 5: other solutions than P-MPR need to be specified for HPUE in NTN to ensure a sufficient uplink budget.

Proposal 6: RAN4 shall check if the duty cycle-based approach for SAR can be applied in NTN system.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: Due to the lack of TN requirement as a reference, the MPR and AMPR values of the HPUE need to be re-evaluated based on reasonable implementations of IoT devices.

Observation 2: the NTN system in handheld devices might be used for emergency cases; therefore, the link budget is critical. Therefore, relying only on a P-MPR-based solution to meet the SAR limits is not sufficient.

Proposal 1: RAN4 can focus on the single Tx architecture for PC2 IoT NTN but further study the feasibility of 2Tx architecture.

Proposal 2: RAN4 shall carefully study the corresponding emission limit from regulators and specify the method to ensure the 3GPP HPUE IoT NTN does not violate them.

Proposal 3: RAN4 may wait for ETSI to specify the new harmonized standard for HPUE IoT NTN before capturing any emission limit from ETSI.

Proposal 4: In addition to the AMPR, guard band/RB may also be considered for HPUE IoT NTN to meet the emission limit.

Proposal 5: other solutions than P-MPR need to be specified for HPUE in NTN to ensure a sufficient uplink budget.

Proposal 6: RAN4 shall check if the duty cycle-based approach for SAR can be applied in NTN system.

# References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2414452	WF on UE RF requirements for NTN HPUE 	Samsung

- R4-2317932	LS reply on 3GPP NTN requirements and potential impact on specification work in ETSI TC SES	ETSI TC SES

- R4-2411017	Blocking requirements for extended L band 	ETSI TC ERM