3GPP TSG-RAN WG4 Meeting #112bis		                 R4-2415586

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	UE RF requirement for NTN Redcap and eRedcap

Agenda item:	6.26.2.1

Document for:	Approval

# Background

In RAN4#112, the RF requirement for NTN Redcap in Rel-19 has been further discussed [1], where the following agreements have been made:

- HD-FDD refsens for 2 Rx: n256: 0.5 dB tightening

- Further study NR NTN UE simultaneous operation with GNSS considering

- UL capacity enhancements: As starting point, no UE RF impact at least from inter-symbol OCC scheme

In addition, the potential impact on maximum output power were also discussed during the meeting.

In this contribution, we provide our view on the remaining missing RF requirement for NTN Redcap and eRedcap in Rel-19.

# REFSENS for HD-FDD

In RAN4#112, it has been agreed that a 0.5 dB tightening factor is adopted for n256 for HD-FDD operation, which results in the REFSENS for 5MHz becoming -100 dBm for HD-FDD, and it aligns with the principle of how we derived the TN Redcap REFSENS.

In addition, In RAN4#111, it has also been agreed:

- [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112.

The 0.5 dB tightening on n254 and n256 followed the same principle as in the TN case, which was originally proposed in [3], and the principle is summarized in the table below:

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

In our view, the same approach should be taken as in the TN case so that the RF front end of TN Redcap and NTN Redcap can be largely reused to facilitate the cost and time-to-market of Redcap/eRedcap NTN system.

Observation 1: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

From the coverage perspective, it is our understanding that the uplink coverage is the bottleneck for both legacy TN Redcap and legacy NTN networks. Therefore, further stringent REFSENS requirements may not significantly benefit network coverage.

Observation 2: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Therefore, it is proposed that the same approach can also be used to derive the REFSENS for NTN Redcap HD-FDD, where RAN4 shall confirm 0.5 dB tightening for n254 and adopt 0 dB tightening for n255.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n255 and n254.

# GNSS operation

In RAN4#111, it is identified that the GNSS operating in the frequency range 1559~1610MHz and may have difficulty operating simultaneously with frequency band n255 due to the NTN UL/DL interference and n254 due to the UL interference [4]. In addition, a few bands (e.g., [n251]) are currently under discussion in 3GPP RAN4, which may also impact the GNSS receiver due to the very close spectrum allocation. An illustration of NTN spectrum 3GPP v.s. GPS L1 receiver frequency is presented in Fig. 1.

Fig. 1. An illustration of NTN spectrum 3GPP and GPS L1 receiver frequency.

Observation 3: It is challenging for GNSS and NR NTN to operate simultaneously in some frequency bands due to the extremely closed frequency allocation.

On the other hand, it is explicitly stated in the WI that the simultaneous GNSS and NR-NTN operation are supported in RedCap/eRedCap UE [5]. Such a restriction was not required explicitly for normal NR NTN UEs, and it was explicitly not needed at all for IoT NTN UEs. Considering the complexity of Redcap UE is somewhere between normal NR UE and IoT UEs, it is a bit unclear what the implication of having this requirement for Redcap NTN devices is.

Considering the Redcap NTN is a lower device than normal NR NTN, it is also a question of whether the Redcap NTN can fully support the Doppler and TA pre-compensation in the same way as normal NR UEs. Therefore, RAN4 shall investigate how to handle the scenario in which the GNSS can’t operate simultaneously with NR communication.

Proposal 2: RAN4 shall further study the impact of GNSS not operating simultaneously with NR NTN.

Per our knowledge, no measurement gap has ever been defined for NR NTN operation, only for IoT NTN. Therefore, there is currently no approach from 3GPP that can be used for NR NTN UE to mitigate such interference.

Observation 4: No measurement gap has been defined for NR NTN operation.

To resolve this issue without affecting the core requirements defined in RAN4, a possible tool to be used is the in-device coexistence (IDC) mechanism. IDC is a mechanism to solve any coexistence issues within the UE that can not be solved by UE implementation, and UE requests some action from the network. Actually, GNSS reception and LTE UL interference within the UE for some bands was one of the use cases when IDC was first studied and specified back in rel-10 or so. Then, in subsequent releases, many other use cases were identified and defined, including those for NR.

Observation 5: GNSS reception and LTE UL interference within the UE for some bands was one of the use cases when IDC was first studied and specified back in rel-10 or so.

With IDC solution, NR NTN UE can send an indication to the network along with some more information like interferer type, etc, and then the network either applies FDM (handover to another frequency or allocates resources away from interfering source) or TDM (time multiplex resources between two sources) between the GNSS and NR NTN. Therefore, adding the GNSS receiver as an IDC event for the NTN system in 3GPP can ensure the protection of the GNSS receiver with NR NTN operation for Redcap NTN UE.

Proposal 3: It is proposed that GNSS reception be added as an event for IDC to protect the GNSS operation with the NR NTN option for Redcap NTN UE.

With the IDC solution, a universal method can be created for all the existing and future NTN bands that may potentially jeopardize the GNSS operation. Meanwhile, the proposed approach can minimize the impact on the RAN4 RF requirement, where no core requirement changes are needed, but it only needs to ensure that no IDC event is triggered during the conformance test.

Observation 6: no change in the RF requirement is expected by using IDC IDC-based approach but rather ensure that the IDC event shall not be triggered during the conformance test by designing the test configuration properly.

Proposal 4: RAN4 can further discuss how to ensure that no IDC event would be triggered in the conformance test if the IDC-based approach is agreed upon to solve the coexistence between GNSS and NR NTN.

# Maximum output power

Another open issue from RAN4#111 is whether to consider an increased PC3 power level for NRN Redcap UE for HD-FDD operation. In our view, the output power for each power class is carefully defined and justified. The HD-FDD front end is intended to provide a more efficient frontend solution than FD-FDD with reduced insertion loss but is not designed to support higher output power. Therefore, there is no reason to change the maximum output power of the power class, and a new power class would be needed if any change of maximum output power would be needed.

Proposal 5: Keep the output power level of PC3 for Redcap NTN.

# Conclusion

In this paper, we presented our view on the RF requirement for NTN Redcap and eRedcap, where the following observations and proposals are made:

Observation 1: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

Observation 2: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Observation 3: It is challenging for GNSS and NR NTN to operate simultaneously in some frequency bands due to the extremely closed frequency allocation.

Observation 4: No measurement gap has been defined for NR NTN operation.

Observation 5: GNSS reception and LTE UL interference within the UE for some bands was one of the use cases when IDC was first studied and specified back in rel-10 or so.

Observation 6: no change in the RF requirement is expected by using IDC IDC-based approach but rather ensure that the IDC event shall not be triggered during the conformance test by designing the test configuration properly.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n255 and n254.

Proposal 2: RAN4 shall further study the impact of GNSS not operating simultaneously with NR NTN.

Proposal 3: It is proposed that GNSS reception be added as an event for IDC to protect the GNSS operation with the NR NTN option for Redcap NTN UE.

Proposal 4: RAN4 can further discuss how to ensure that no IDC event would be triggered in the conformance test if the IDC-based approach is agreed upon to solve the coexistence between GNSS and NR NTN.

Proposal 5: Keep the output power level of PC3 for Redcap NTN.

# References

- R4-2413411  Topic summary for [112][311] NR_NTN_Ph3_UE_RF	Moderator (Qualcomm)

- R4-2410574   WF on Rel-19 phase 3 NR-NTN UE RF requirements	Qualcomm

- R4-2200442   RedCap UE HD-FDD REFSENS requirements   Apple

- R4-2409336   Discussion on the other potential impacts for Rel-19 NTN RedCap UE    Huawei, HiSilicon

- RP-234078  New WID: Non-Terrestrial Networks (NTN) for NR Phase 3