3GPP TSG-RAN WG4 Meeting#112bis		R4-2415589

Hefei, China, Oct 14th -19th, 2024

Agenda item:	6.21.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on RF requirements impact for intermediate node (UE)

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

Discussion

TX requirements

Agreement in RAN4#112: - Requirements potentially needed: - Maximum output power - Transmitted signal quality (Frequency error, EVM) - Occupied bandwidth - Spectrum emission mask - Transmitter Intermodulation - ACLR - Operating band unwanted emissions - Transmitter spurious emissions - FFS whether requirements are needed or not: - Output power dynamics - Transmit ON/OFF power - Transmit ON/OFF time mask - IBE - Requirements not needed: - Transmission times

Minimum output power

The purpose of defining the minimum power is to ensure the performance and SNR prevent from being drowned by noise. If the intermediate node (UE) supports power control, which means transmission power is not fixed and is dynamically adjusted. As is all known to us, receiver performance of device is worse than normal UE. In our understanding, the intermediate node (UE) needs to ensure the performance and SNR to compensate for the poor performance brought by the device in output dynamic power range. Hence, minimum output power is needed to define for the intermediate node (UE) if the intermediate node (UE) supports power control. In addition, the power level of minimum output power needs to be further discussed.

Proposal 1: Minimum output power is needed to define for the intermediate node (UE) if the intermediate node (UE) supports power control. The power level of minimum output power needs to be further discussed.

Transmit off power

It is observed that excessive transmitter off power may increase the Rise over Thermal (RoT), thus reducing the cell coverage area of other UEs. If the off power of UE is too high, it means that the system floor noise of the whole cell has been raised. In order to meet a certain SNR requirement, the terminal receiving service will have to raise the transmission power, which in turn will cause higher noise for other UEs. As a result, the system capacity of the whole cell decreases. Whatever in factory or indoor, there must be multiple readers (the intermediate node (UE)). In our understanding, transmit off power should be defined to avoid increasing the system floor noise and save power consumption.

Proposal 2: Transmit off power is needed to define for the intermediate node (UE).The power level of transmit off power needs to be further discussed.

Transmit ON/OFF time mask

For the transmit ON/OFF time mask, the output power of the terminal during the rising edge/falling edge period when the transmission is off and on is mainly investigated to ensure the correct power output; Sending the wrong power will interfere with other channels or increase the transmission error of uplink channel. However, power control accuracy may not need to be like that of normal UE. Hence, transmit on/off time mask maybe is not needed for the intermediate node (UE).

Proposal 3: Transmit on/off time mask maybe not be necessary for the intermediate node (UE).

In-band emissions

According to the definition of in-band emission (IBE), IBE includes three parts: general, IQ image and carrier leakage. In our understanding, IQ image and carrier leakage should exist, but generally maybe rely on RAN1’s design for channel design. But anyway, in-band emission needs to be considered.

Proposal 4: In-band emission needs to be considered.

RX requirements

Agreement in RAN4#112: - Requirements potentially needed: - Reference sensitivity power level - Maximum input power - Receiver intermodulation - FFS whether requirements are needed or not: - ACS - In-band blocking - Out-of-band blocking - Narrow band blocking - Receiver spurious response - Rx spurious emission - Requirements not needed: - ICS

Similarly, better reception performance of the reader (the intermediate node (UE) can compensate for the poor transmission of performance of device. Hence, related RX requirements could be considered. Specific requirements level needs to be relied on co-existence study and regulatory requirements.

Proposal 5: The following requirement needs to be considered:

- ACS

- In-band blocking

- Out-of-band blocking

- Narrow band blocking

- Receiver spurious response

- RX spurious emission

Conclusions

In this paper, we provide our views on RF requirements impact of intermediate node (UE), we have made the following proposals.

Proposal 1: Minimum output power is needed to define for the intermediate node (UE) if the intermediate node (UE) supports power control. The power level of minimum output power needs to be further discussed.

Proposal 2: Transmit off power is needed to define for the intermediate node (UE).The power level of transmit off power needs to be further discussed.

Proposal 3: Transmit on/off time mask maybe not be necessary for the intermediate node (UE).

Proposal 4: In-band emission needs to be considered.

Proposal 5: The following requirement needs to be considered:

- ACS

- In-band blocking

- Out-of-band blocking

- Narrow band blocking

- Receiver spurious response

- RX spurious emission

Reference

- R4-2414305, WF on impacts of A-IoT  on RF requirements, Huawei, TSG RAN WG4 meeting #112