3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415596

Hefei, Anhui, China 14th – 18th  October, 2024

Agenda Item:	6.21.2.2

Source: 	OPPO

Title: 	on co-existence evaluations

Document for:	Approval

- Introduction

In RAN4#112 meeting, the co-existence evaluations have been fully carried out. In this paper we try to give further analysis based on the co-existence simulation results.

- Discussion

For the D2T2 scenario, simulation for case 5-1 and 5-2 have been carried out. Based on our simulation result, it has been shown below for case 5-1,

It can be observed from the above captured agreements, the different understanding at least starts from whether type 4a UE can support NR-CA. We have looked back to the WID and previous agreements, from the WID [2], it has captured that “Note: UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE).”  In such case we refer to the UE types captured in WF [3] and the agreements are captured as below:

Table 1 SINR degradation for case 5-1

| case | Aggressor | victim | SINR degradation @5% in dB | SINR degradation @50% in dB |
| --- | --- | --- | --- | --- |
| f | NR UL | reader | 0.01 | 0.7 |
| g | reader | NR UL | 0 | 0 |
| h | NR UL | device | 0 | 0.03 |

From table 1 above, it can be observed that for NR UL as victim, the SINR degradation for 5% and 50% are all 0dB. The NR UL victim is base station RX and as the in-channel selectivity has been assumed as full, also considering the base stations are all located outdoor, the simulation result is quite clear that no degradation will be observed.

Observation 1: For case 5-1-g, no degradation is observed.

For NR UL as aggressor, the SINR degradation for reader is 0.01dB at 5% and 0.7dB at 50%. The SINR degradation for device is 0dB for 5% and 0.03dB for 50%. From device perspective, the 0.03dB SINR degradation can be ignored. While from reader perspective, the 0.7dB SINR degradation needs further study to see if acceptable for reader throughput.

Observation 2: For case 5-1-h, the 0.03dB SINR degradation can be ignored.

Table 2 SINR degradation for case 5-2

| case | Aggressor | victim | 10% UE indoor |  | 100% UE indoor |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SINR degradation @5% in dB | SINR degradation @50% in dB | SINR degradation @5% in dB | SINR degradation @50% in dB |
| g | reader | NR UL | 0.04 | 2.87 | 0.03 | 0.48 |
| h | NR UL | device | 6.32 | 11.76 | 16.41 | 25.14 |

Above table 2 has shown the degradation for 5-2-h is quite large. Even for 10% UE indoor, the SINR degradation at 5% is 6.32dB and at 50% is 11.76dB. For 100% the SINR degradation goes even higher as at 5% is 16.41dB and at 50% is 25.14dB. The number for 10% UE indoor is quite large since there are only 3 UEs UL at for each UE the indoor probability is 10%. However, once the UE is located indoor, the interference is quite large and hence the SINR degradation is also large even with only 10% UE indoor.

Observation 3: For 10% UE indoor, the SINR degradation is still large.

From the A-IoT industry development perspective, limiting the legacy UE deployment is for sure not what we want. In such case, it might be useful to inform RAN1 that some further interference cancellation mechanism might be introduced.

3	Conclusions

In this paper, we give further discussion on Type 4 UE behaviour and capability, the observations and proposals are as below:

Observation 1: For case 5-1-g, no degradation is observed.

Observation 2: For case 5-1-h, the 0.03dB SINR degradation can be ignored.

Observation 3: For 10% UE indoor, the SINR degradation is still large.

4 References