3GPP TSG-RAN WG4 Meeting#112bis		R4-2415608

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.21.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on co-existence evaluation results for Ambient IoT and NR

Document for:	Discussion

Introduction

In RAN4#112, co-existence calibration results and assumption for formal simulation were reached and a WF was approved [1]. In this paper, we present updated simulation results and provide some views about the co-existence aspects.

Discussion

Deployment scenario and simulation cases

In RAN4#112 meeting, below cases were reached to be simulated in formal simulation.

| Deployment scenario No. (Case No.) | Topology | spectrum |
| --- | --- | --- |
| 1-1(a/b/c/d) | D1T1-A2- NR UE only outdoor | R2D: DL |
|  |  | CW2D and D2R: DL |
| 1-2(a/b/c/d) | D1T1-A2- NR UE indoor | R2D: DL |
|  |  | CW2D and D2R: DL |
| 2-1(e/f/c/d) | D1T1-B- NR UE only outdoor | R2D: DL |
|  |  | CW2D and D2R: UL |
| 2-2(e/f/c/d) | D1T1-B- NR UE indoor | R2D: DL |
|  |  | CW2D and D2R: UL |
| 3 (Optional)(a/b/g/h) | D1T1-A2- NR UE indoor | R2D: UL |
|  |  | CW2D and D2R: DL |
| 4 (Optional)(e/f/g/h) | D1T1-B- NR UE indoor | R2D: UL |
|  |  | CW2D and D2R: UL |
| 5-1(e/f/g/h) | D2T2-A2- NR UE only outdoor | R2D: UL |
|  |  | CW2D and D2R: UL |
| 5-2(e/f/g/h) | D2T2-A2- NR UE indoor | R2D: UL |
|  |  | CW2D and D2R: UL |
| 6-1(a/b/g/h) | D2T2-B- NR UE only outdoor | R2D: UL |
|  |  | CW2D and D2R: DL |
| 6-2(a/b/g/h) | D2T2-B- NR UE indoor | R2D: UL |
|  |  | CW2D and D2R: DL |

Simulation results

D1T1-A2 simulation results

In this scenario, R2D is DL spectrum, D2R and CW2D is DL spectrum. The device type is device1, the TX BS IBE is 30dBc, the A-IoT BS TX power is 33dBm. We have not used the Maximum d2D-in. The CW cancellation capability is 138dB, which is used only at D2R receiver, and no CW interference at R2D receiver since R2D and CW will not exist at the same time. In addition, no CW inference at NR UE receiver. Throughput for NR is referred to TR 38.803 Section 5.2.7, equations approximate the throughput over a channel with a given SINR.

The D1T1-A2 simulation results for the scenarios of legacy UE only outdoor and legacy UE indoor are shown in the below figures:

Scenario 1-1: Legacy UE only outdoor

Figure1: case a (device vs NR DL)                                       Figure 2: case b (NR DL vs reader)

Figure3: case c (Reader vs NR DL)                                       Figure 4: case d (NR DL vs device)

According to the simulation results, the SINR degradation of ACI is 0dB @5% and @50% compared to baseline SINR for case a and case b in figure1 and figure2. The SINR degradation of ACI is 0.0091dB @5% and 0.0048dB @50% for case c in figure3. The SINR degradation of ACI is 0.0327dB @5% and 0.0695dB @50% for case d in figure4. It is observed that there is basically no interference for case a/b/c/d because the SINR degradation of ACI is less than 1dB than baseline SINR.

Observation 1: For D1T1-A2 in scenario 1-1, there is no co-existence impact between A-IoT and NR when Legacy UE is only outdoor.

Scenario 1-2: 10% Legacy UE indoor and 90% Legacy UE outdoor

Figure 5: case a (device vs NR DL)                                    Figure 6: case b (NR DL vs reader)

Figure 7: case c (Reader vs NR DL)                           Figure 8: case d (NR DL vs device)

According to the simulation results, the SINR degradation of ACI is 0dB @5% and @50% compared to baseline SINR for case a and case b in figure5 and figure6. The SINR degradation of ACI is 0.3981dB @5% and 0.126dB @50% for case c in figure7. The SINR degradation of ACI is 0.0304dB @5% and 0.0709dB @50% for case d in figure8. It is observed that there is basically no interference for case a/b/c/d because the SINR degradation of ACI is less than 1dB than baseline SINR.

Observation 2: For D1T1-A2 in scenario 1-2, there is no co-existence impact between A-IoT and NR when 10% Legacy UE is indoor and 90% Legacy UE is outdoor.

However, in real scenario, how to judge the percentage of NR UE is indoor, 10% legacy UE is indoor is OK means that one NR UE or others? It seems that there is no difference between 10% legacy UE and 100% legacy UE is indoor.

Observation 3: How to judge the percentage of 10% Legacy UE in real scenario?

Scenario 1-2: 100% Legacy UE indoor

Figure 9: case a (device vs NR DL)                                                           Figure 10: case b (NR DL vs reader)

Figure 11: case c (Reader vs NR DL)                                               Figure 12: case d (NR DL vs device)

According to the simulation results, the SINR degradation of ACI is 0.016dB @5% and 0.01dB@50% compared to baseline SINR for case a in figure9. The SINR degradation of ACI is 0 dB @5% and 0 dB @50% for case b in figure10. The SINR degradation of ACI is 21.407dB @5% and 15.4807dB @50% for case c in figure11 and the throughput loss is 0%@5% and 82.9%@50%. The throughput loss is 0% means that 5% of NR UE of SINR is lower than -10dB which cause throughput is 0. Obviously, when NR UE is 100% indoor, there is obviously SINR degradation since it suffered interference from A-IoT system.

According to the above analysis, there is severe co-existence impact between A-IoT and NR when Legacy UE is only indoor, which means NR UE hardly cannot work when NR UE is indoor. How to solve this issue? Maybe we can use guard RB(s) between NR UE and A-IoT system to avoid this interference.

Observation 4: For D1T1-A2 in scenario 1-2, there is severe co-existence impact between A-IoT and NR when 100% Legacy UE is indoor. The SINR degradation is much higher than 1dB. And the throughput loss is 82.9%@50%.

Proposal 1: Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when 100% legacy UE is indoor for D1T1-A2.

In addition, according to figure 9 and figure 11, we can see that there about 30% baseline SINR is lower than -2.5dB for NR DL, which means these NR UE cannot work because the distance between NR BS and indoor NR UE is too far. Do we need to set the Maximum d2D-in or can we default that NR UE has no signal indoors?

Proposal 2: Whether maximum d2D-in be set or not needs to be discussed.

D1T1-B simulation results

In this scenario, R2D is DL spectrum, D2R and CW2D is UL spectrum. The device type is device1, the TX BS IBE is 30dBc, the A-IoT BS TX power is 33dBm. We have not used the Maximum d2D-in. The CW cancellation capability is 155dB which is used only at D2R receiver, and there is no CW interference at R2D receiver, since R2D and CW will not exist at the same time. In addition, no CW inference at NR UE receiver. Throughput for NR is referred to TR 38.803 Section 5.2.7, equations approximate the throughput over a channel with a given SINR.

The D1T1-B simulation results for the scenarios of legacy UE only outdoor and legacy UE indoor are shown in the below figures.

Scenario 2-1: Legacy UE only outdoor

Figure13： Case e( device vs NR UL )                                   Figure14： Case f( NR UL  vs Reader )

Figure15： case c(Reader vs NR DL)                                                           Figure16： Case d( NR DL vs device)

According to the simulation results, the SINR degradation of ACI is 0dB @5% and 0dB@50% compared to baseline SINR for case e in figure13. The SINR degradation of ACI is 0.2277 dB @5% and 0.0532 dB @50% for case f in figure14. The SINR degradation of ACI is 0.0193dB @5% and 0.0042dB @50% for case c in figure15. The SINR degradation of ACI is 0.0333dB @5% and 0.0673dB @50% for case d in figure16. For case e/f/c/d, the SINR degradation of ACI is less than 1dB compared to baseline SINR. It is observed that there is basically no co-existence impact for A-IoT and NR when NR UE is only outdoor.

Observation 5: For D1T1-B in scenario 2-1, there is no co-existence impact between A-IoT and NR when Legacy UE is only outdoor.

Scenario 2-2: 10% Legacy UE indoor and 90% Legacy UE outdoor

Figure17： Case e( device vs NR UL )                                            Figure18： Case f( NR UL  vs Reader )

Figure19： case c(Reader vs NR DL)                                                           Figure20： Case d ( NR DL vs device)

According to the simulation results, the SINR degradation of ACI is 0dB @5% and 0dB@50% compared to baseline SINR for case e in figure17. The SINR degradation of ACI is 13.3715dB @5% and 6.065dB @50% for case f in figure18, The SINR degradation of ACI is 0.3133dB @5% and 0.1643dB @50% for case c in figure19. The SINR degradation of ACI is 0.0323dB @5% and 0.074dB @50% for case d in figure20. Obviously, we can see that in case f in figure18, the SINR degradation is large since it suffered interference from A-IoT system. According to the figure 21, the SNR is 2.2dB when A-IoT TX BLER is 10%. And we can see that the outage percentage is 88% consider SINR level with 10% BLER. It means that 88% of Readers cannot receive the signal of D2R because it suffered severe interference from NR UE. How to solve this issue? Similarly, guard RB(s) needs to be considered.

Figure21: A-IoT TX BLER vs SNR

Observation 6: For D1T1-B in scenario 2-2, there is severe co-existence impact between A-IoT and NR when 10% Legacy UE is indoor. The SINR degradation is much higher than 1dB. And the outage percentage is 88% consider SINR level with 10% BLER.

Observation 7: For D1T1-B in scenario 2-1, A-IoT and NR for in-band deployment can be co-existed successfully when legacy UE is only outdoor.

Scenario 2-2: 100% Legacy UE indoor

Figure22： Case e( device vs NR UL )                                            Figure23： Case f( NR UL  vs Reader )

Figure24： case c(Reader vs NR DL)                                                           Figure25： Case d ( NR DL vs device)

According to the simulation results, the SINR degradation of ACI is 0dB @5% and 0dB@50% compared to baseline SINR for case e in figure22. The SINR degradation of ACI is 28.4993dB @5% and 24.306dB @50% for case f in figure23, The SINR degradation of ACI is 21.3042dB @5% and 15.4753dB @50% for case c in figure24. The SINR degradation of ACI is 0.312dB @5% and 0.0685dB @50% for case d in figure25. Obviously, we can see that in case f in figure23, the SINR degradation is large since it suffered interference from NR system. According to the figure 21, the SNR is 2.2dB when A-IoT TX BLER is 10%. And we can see that the outage percentage is 87% considering SINR level with 10% BLER. In addition, we can see that in case c in figure24, the SINR degradation is large since it suffered interference from A-IoT system. The throughput loss is 0%@0% and 80.78%@50%. The throughput loss is 0% means that 5% of NR UE of SINR is lower than -10dB which causes throughput is 0.

According to the above analysis, there is severe co-existence impact between A-IoT and NR when Legacy UE is only indoor, which means NR UE hardly cannot work and Reader cannot work when NR UE is indoor. Similarly, how to solve this issue? Maybe we can use guard RB(s) between NR UE and A-IoT system to avoid this interference.

Observation 8: For D1T1-B in scenario 2-2, there is more severe co-existence impact between A-IoT and NR when 100% Legacy UE is indoor compared to10% legacy UE is indoor. The SINR degradation is much higher than 1dB.

Observation 9: For D1T1-B in scenario 2-2, A-IoT and NR for in-band deployment cannot co-exist successfully when legacy UE is indoor, whatever the percentage of NR UE is indoor.

Proposal 3: Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when legacy UE is indoor whatever the percentage.

In addition, D2T2-B with R2D is DL spectrum and CW2D/D2R are UL spectrum simulation results show more severe impact compared to D1T1-A2 with R2D/CW2D/D2R are DL spectrum simulation results when some NR UE  are indoor. It is observed that R2D/CW2D/D2R is DL spectrum for D1T1 with better performance compared to R2D is DL spectrum and CW2D/D2R is UL spectrum when some NR UE is indoor.

Observation 10: R2D/CW2D/D2R is DL spectrum for D1T1 with better performance compared to R2D is DL spectrum and CW2D/D2R is UL spectrum when some NR UE is indoor.

And we can see that, as to in-band deployment for A-IoT and NR, there is no co-existence problem when only outdoor UE. If NR UE is indoor, NR UE or reader may not work. Hence, related solutions for co-existence need to be considered.

Observation 11: In-band deployment for A-IoT and NR for D1T1, there is no co-existence between A-IoT and NR problem when NR UE is only outdoor.

Proposal 4: Related solutions for co-existence between A-IoT and NR for in-band deployment need to be considered when NR UE is indoor.

Conclusions

In this paper, we provide our views on co-existence between A-IoT and NR/UE, we have made the following observations and proposals.

Observation 1: For D1T1-A2 in scenario 1-1, there is no co-existence impact between A-IoT and NR when Legacy UE is only outdoor.

Proposal 1: Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when 100% legacy UE is indoor for D1T1-A2.

Proposal 2: Whether maximum d2D-in be set or not needs to be discussed.

Observation 2: For D1T1-A2 in scenario 1-2, there is no co-existence impact between A-IoT and NR when 10% Legacy UE is indoor and 90% Legacy UE is outdoor.

Observation 3: How to judge the percentage of 10% Legacy UE in real scenario?

Observation 4: For D1T1-A2 in scenario 1-2, there is severe co-existence impact between A-IoT and NR when 100% Legacy UE is indoor. The SINR degradation is much higher than 1dB. And the throughput loss is 82.9%@50%.

Observation 5: For D1T1-B in scenario 2-1, there is no co-existence impact between A-IoT and NR when Legacy UE is only outdoor.

Observation 6: For D1T1-B in scenario 2-2, there is severe co-existence impact between A-IoT and NR when 10% Legacy UE is indoor. The SINR degradation is much higher than 1dB. And the outage percentage is 88% consider SINR level with 10% BLER.

Observation 7: For D1T1-B in scenario 2-1, A-IoT and NR for in-band deployment can co-exist successfully when legacy UE is only outdoor.

Observation 8: For D1T1-B in scenario 2-2, there is more severe co-existence impact between A-IoT and NR when 100% Legacy UE is indoor compared to10% legacy UE is indoor. The SINR degradation is much higher than 1dB.

Observation 9: For D1T1-B in scenario 2-2, A-IoT and NR for in-band deployment cannot co-exist successfully when legacy UE is indoor, whatever the percentage of NR UE is indoor.

Proposal 3: Guard RB(s) between NR and A-IoT needs to be considered for co-existence between A-IoT and NR when legacy UE is indoor whatever the percentage.

Observation 10: R2D/CW2D/D2R is DL spectrum for D1T1 with better performance compared to R2D is DL spectrum and CW2D/D2R is UL spectrum when some NR UE is indoor.

Observation 11: In-band deployment for A-IoT and NR for D1T1, there is no co-existence between A-IoT and NR problem when NR UE is only outdoor.

Proposal 4: Related solutions for co-existence between A-IoT and NR for in-band deployment need to be considered when NR UE is indoor.

Reference

- R4-2414304, WF on co-existence evaluation for ambient IoT, CMCC, INC.TSG-RAN WG4#112 meeting