3GPP TSG-RAN WG4 Meeting#112bis		R4-2415610

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.23.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on RX requirements of REFSENS for LP-WUR

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

REFSENS requirements

For below 6GHz, the REFSENS level can be calculated by the equation below.

Sensitivity (dBm) = -174dBm (KT)+NF+10log (RX BW)-Diversity gain+ SNR+IM

IM value for REFSENS

As we know, we need to consider the Implementation Margin (IM) because handheld UE/CPE and so on terminals have so many antenna combinations. Now existing NR UE use 2.5dB for IM value. In our understanding, LP-WUR also is applied in handheld UE and other form factor terminals but still have so many antenna combinations. Thus, we could reuse 2.5dB IM value for LP-WUR. In addition, there is no obvious difference between OOK-based and OFDM-based receivers with regard to antenna combinations.

Proposal 1: IM value could use 2.5dB for OOK-based and OFDM-based receivers.

NF GAP between LR and MR for REFSENS (assume MR as 9dB)

Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8

As to noise figure (NF), we could refer to NF part in RAN1 evaluation. RAN1’s conclusion in RAN1#116bis about NF is that all three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported. Based on the above information, NF can be define as delta NF on top of MR. I.e., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR but NF of MR is 9dB.

Proposal 2: NF can be defined as delta NF on top of MR (e.g., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR (assume MR NF is 9dB).

MDR value for REFSENS test case

Agreements: - FFS the MDR for conformance testing should be aligned with the value for RF requirement.

In last meeting, some companies proposed that using higher MDR (e.g. 5%) for conformance testing compared with 1% MDR for core requirements in order to save testing cost and complexity. If we define different values for core requirements and testability requirements, especially lower MDR for conformance testing. Assume the MDR value of conformance testing is 5% and the MDR value of core requirements is 1%.In such conditions, even though the terminals pass the conformance testing, how can we ensure that the terminals must meet the core requirements? If we can’t guarantee that the terminals meet core requirements, the definition of core requirements will be meaningless. In our understanding, the MDR value for conformance testing should be aligned with the value for RF requirements.

Proposal 3: The MDR value for conformance testing should be aligned with the value for RF requirements.

Conclusions

In this paper, we provide our views on RX requirements of REFSENS, we have made the following proposals.

Proposal 1: IM value could use 2.5dB for OOK-based and OFDM-based receiver.

Proposal 2: NF can be defined as delta NF on top of MR (e.g., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR (assume MR NF is 9dB).

Proposal 3: The MDR value for conformance testing should be aligned with the value for RF requirements.

- Reference

- R4-2414309, WF on NR LP-WUS UE requirements, vivo, TSG-RAN WG4 112 meeting