3GPP TSG-<TSG/WG4> Meeting #112bis	R4-2415612

# Hefei, China , Oct 14th – 18th , 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 15.27.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.101-1 on ΔT_IB and ΔR_IB tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NewRAT-Core |  |  |  |  |  | Date: |  |  | 2024-10-5 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce a template-base approach to simplify the tables |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introdcue a template {a,b} for a band combination consisting of band nX and nY to representing dBs respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In newer releases these tables become long and inconvenient |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.2.3, 6.2C.2, 7.3A.3, 7.3C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

##### 6.2A.4.2.3 ΔTIB,c for Inter-band CA (two bands)

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c (dB) | Inter-band CA combination | ΔTIB,c (dB) | Inter-band CA combination | ΔTIB,c (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n3-n77 | {0.6, 0.8} | CA_n3-n78 | {0.6, 0.8} | CA_n3-n79 | {0.3, 0.8} |
| CA_n8_n75 | {0.3, 0} | CA n8-n78 | {0.6, 0.8} | CA_n8-n79 | {0.3, 0.8} |
| CA n28-n75 | {0.3, 0} | CA_n28-n78 | {0.5, 0.8} | CA_n41-n781 | {0.3, 0.8} |
| CA_n75-n78 | {0, 0.8} | CA_n76-n78 | {0, 0.8} | CA n77-n79 | {0.5, 0.5} |
| CA_n78-n79 | {0.5 or 1.52, 0.5 or 1.52} |  |  |  |  |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |

<End of Change>

----- Next change ----

<Start of Change>

### 6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | ΔTIB,c (dB) | Band combination for SUL | ΔTIB,c (dB) | Band combination for SUL | ΔTIB,c (dB) |
| --- | --- | --- | --- | --- | --- |
| SUL_n78-n80 | {0.8, 0.6} | SUL_n78-n81 | {0.8, 0.6} | SUL_n78-n82 | {0.8, 0.6} |
| SUL_n78-n83 | {0.8, 5} | SUL_n78-n84 | {0.8, 0.3} | SUL_n78-n86 | {0.8, 0.6} |

<End of Change>

----- Next change ----

<Start of Change>

##### 7.3A.3.2.1	ΔRIB,c for two bands

Table 7.3A.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination | ΔRIB,c (dB) | Inter-band CA combination | ΔRIB,c (dB) | Inter-band CA combination | ΔRIB,c (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_n3-n77 | {0.2, 0.5} | CA_n3-n78 | {0.2, 0.5} | CA_n3-n79 | {0, 0.5} |
| CA_n8_n78 | {0.2, 0.5} | CA n8-n79 | {0, 0.5} | CA_n28-n75 | {0.2, 0} |
| CA n28-n78 | {0.2, 0.5} | CA_n41-n781 | {0, 0.5} | CA_n75-n78 | {0, 0.5} |
| CA_n76-n78 | {0, 0.5} |  |  |  |  |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. |  |  |  |  |  |

<End of Change>

----- Next change ----

<Start of Change>

##### 7.3C.3.2.1	ΔRIB,c  for two bands

Table 7.3C.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | ΔRIB,c (dB) | Band combination for SUL | ΔRIB,c (dB) | Band combination for SUL | ΔRIB,c (dB) |
| --- | --- | --- | --- | --- | --- |
| SUL_n78-n80 | {0.5, 0} | SUL_n78-n81 | {0.5, 0} | SUL_n78-n82 | {0.5, 0} |
| SUL_n78-n83 | {0.5, 0} | SUL_n78-n84 | {0.5, 0} | SUL_n78-n86 | {0.5, 0} |

<End of Change>