3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415624

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM core requirements in AIML Beam management

Source: 	Huawei, HiSilicon

Agenda item:	6.17.4.1

Document for:	Discussion

Introduction

RRM core requirements for AI/ML based beam management are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the earlies discussion, the RRM requirements are mainly considered for data collection, model inference and LCM related requirements. Our views on requirements for model inference are provided in the companion paper on testability and interoperability issues in AI/ML based beam management.

In this paper, we will provide our views on data collection and LCM related requirements for AI/ML based BM.

Discussion

Data collection

Data collection is involved in model inference, model monitoring and model training. The reporting of data collection is different for UE-sided model and for NW-sided model.

- NW-sided model

In RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are reached related to NW-sided model.

According to RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are achieved. Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

It is observed that the purpose of the beam report initiated by network will not be specified in RAN1. If the purpose will also not be specified by other WGs, RAN4 can focus on the common requirement of potential new reporting or enhanced reporting, without differentiating the purpose of the reporting.

Based on the agreements, the reporting is the beam related information, e.g., L1-RSRPs and beam index. RAN4 can use the existing core requirement as the starting point.

Proposal 1: For data collection in NW-sided model, use the existing core requirement for beam related information reporting as the starting point.

- UE-sided model

According to RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are achieved. Agreement - For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. - where the set of beams is Set A, i.e., the beams for UE prediction. Agreement - For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement - For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption - For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output Agreement - For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

For UE-sided model, new reporting will be introduced. The content could be the beam information of the predicted beams. Since the content is still under RAN1 study, RAN4 can wait for more RAN1 progress. At current stage, potential requirements are identified for reporting delay and prediction accuracy.

LCM related requirements

In RAN1#118 the following agreements are reached related to performance monitoring.

According to RAN1#116, RAN1#116bis, RAN1#117 and RAN1#118, the following agreements are achieved. Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In LCM procedure, the functionality/model activation/deactivation/fallback to non-AI function are to be considered.

For UE-sided model, if the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., triggered by gNB via RRC/MAC-CE/DCI signaling, the delay of activating/deactivating one function can be defined. Otherwise if these functionality/model changes are autonomously performed inside UE, there may be a need to monitor the function performance, then the requirements would be reflected by performance monitoring requirements.

For model monitoring, two options are identified in RAN1. For Option 1, UE needs to report the measurement. This can reuse the legacy requirement of measurement reporting. For Option 2, UE will calculate the performance metric(s) then report. Since the metric is still under RAN1 study, RAN4 can wait for more progress before starting the study of the requirement of metric reporting.

Proposal 2: If measurement reporting is used for the calculation of performance metric at NW during model monitoring, reuse RAN4 legacy requirement for measurement reporting.

Proposal 3: If metric reporting is introduced by RAN1 for model monitoring, RAN4 will study the related requirement.

Conclusions

In this paper we provided provide our views on data collection and LCM related requirements for AI/ML based beam management.

Proposal 1: For data collection in NW-sided model, use the existing core requirement for beam related information reporting as the starting point.

Proposal 2: If measurement reporting is used for the calculation of performance metric at NW during model monitoring, reuse RAN4 legacy requirement for measurement reporting.

Proposal 3: If metric reporting is introduced by RAN1 for model monitoring, RAN4 will study the related requirement.

Reference

- R4-2414323, WF on AI/ML, Qualcomm Incorporated.