3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415628

# Hefei, China, October 14th – 18th, 2024

Title: 	Discussion on introduction of new FR2 UE power class

Source: 	Huawei, HiSilicon

Agenda item:	4.4.1

Document for:	Discussion

- Background

As per the way forward [1] approved in last RAN4#112bis meeting, evaluations on the feasibility of the targeted scenarios based on the given simulation assumptions need to be conducted. In this contribution, we share our evaluation results and corresponding views on introduction of this new FR2 UE power class.

- Discussion

DL evaluations

    - FTP traffic model

As per simulation assumptions [1], full buffer traffic model is agreed, but as per the characteristic of mmWave, only the UEs under the active beam can be served at one time, so UPT traffic model is more suitable. The following are evaluation results for FTP traffic model 3 (4Mb payload every 200ms):

Figure 1: Simulation for DL with FTP traffic model 3 (4Mb payload every 200ms)

Table 1: TP@5% and 95% percentile for DL with FTP traffic model

| CBW | 100 MHz |  |  | 200MHz |  |  | 400MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISD | 100m | 150m | 200m | 100m | 150m | 200m | 100m | 150m | 200m |
| TP@5% Percentile | 341.987 | 257.082 | 160.330 | 882.248 | 602.201 | 391.605 | 1867.569 | 1151.233 | 716.934 |
| TP@95% Percentile | 684.359 | 696.985 | 665.264 | 1538.467 | 1541.115 | 1502.338 | 3303.164 | 3299.711 | 3260.778 |

From the above simulation results, we can know:

Observation 1: In the cell edge, for DL, both max throughput 300Mbps and 500Mbps can’t be achieved under the FR2 network with ISD larger than 150m for 100MHz transmission bandwidth.

Observation 2: In the cell edge, for DL, higher throughput than 300Mbps and 500Mbps can be well achieved under the FR2 network with ISD = 150m for channel bandwidth 200MHz and 400MHz.

From Observation 1 and Observation 2, we can know that larger channel bandwidth than 100MHz is needed to support those scenarios with higher DL throughput than 300Mbps.

    - Full buffer traffic model

As comparison, we also share the evaluation results for full buffer traffic model:

Figure 2: Simulation for DL with Full buffer traffic model

Table 2: TP@5% and 95% percentile for DL with Full buffer traffic model

| CBW | 100MHz |  | 200MHz |  | 400MHz |  |
| --- | --- | --- | --- | --- | --- | --- |
| ISD | 150m | 200m | 150m | 200m | 150m | 200m |
| TP@5% Percentile | 15.971 | 13.940 | 31.581 | 26.794 | 60.153 | 48.321 |
| TP@95% Percentile | 87.526 | 88.199 | 169.011 | 174.754 | 329.976 | 333.682 |

From the above simulation results, we can know that lower throughput can only be achieved for full buffer traffic model compared to FTP traffic model, considering that 10 UEs share the full channel bandwidth. The total cell capacity is similar with FTP traffic model throughput per user.

UL evaluations

    - FTP traffic model

Considering the network is still DL heavy service with TDD pattern DDDSU, so we use FTP traffic Model 3 with 1Mb payload every 200ms for evaluation.

Figure 3: Simulation for UL with FTP traffic model (1Mb payload every 200ms, DDDSU)

Table 3: TP@5%, 50% and 95% percentile for UL with FTP traffic model

| CBW | 100 MHz |  |  | 200MHz |  |  | 400MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISD | 100m | 150m | 200m | 100m | 150m | 200m | 100m | 150m | 200m |
| TP@5% Percentile | 5.486 | 0.103 | 0 | 20.780 | 0.283 | 0 | 33.915 | 0.217 | 0 |
| TP@50% Percentile | 48.325 | 7.268 | 3.547 | 150.834 | 18.1614 | 3.791 | 291.049 | 45.2779 | 2.9749 |
| TP@95% Percentile | 124.229 | 83.394 | 21.067 | 278.198 | 198.222 | 53.852 | 523.614 | 378.276 | 95.216 |

From the above simulation results, we can know that relative low throughput can only be obtained for ISD larger than 100m at the cell edge, but under FR1+FR2 CA network deployment, FR1 carrier can be used for UL transmission when there is UL power and coverage limit for FR2. The max UL throughput for FR1 with DDDSU are 380Mbps(64QAM) or 506Mbps（256QAM）for 2 layer transmission and channel bandwidth 200 MHz, so the additional throughput can be achieved by FR1 besides the throughput obtained by FR2 shown above, it means that still higher UL throughput can be achieved.

Observation 3: FR1 can be used for UL transmission when there is UL power and coverage limit for FR2.

At the same time, higher throughput can be achieved at the cell middle and centre area by using higher channel bandwidth, such as 400MHz or higher channel bandwidth by using CA.

Observation 4: Higher throughput can be achieved at the cell middle and centre area for higher channel bandwidth by using CA.

When there is no UL power and/or coverage limit for FR2, for some UL heavy specific service, TDD pattern DSUUU can be used. We conducted the corresponding evaluations for FTP traffic Model 3 with 1Mb payload every 200ms and have the following evaluation results.

Figure 4: Simulation for UL with FTP traffic model (1Mb payload every 200ms, DSUUU)

Table 4: TP@5%, 50% and 95% percentile for UL with FTP traffic model

| CBW | 100 MHz |  |  | 200MHz |  |  | 400MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISD | 100m | 150m | 200m | 100m | 150m | 200m | 100m | 150m | 200m |
| TP@5% Percentile | 84.651 | 7.649 | 0.030 | 125.555 | 7.306 | 0 | 157.847 | 2.491 | 0 |
| TP@50% Percentile | 359.226 | 179.808 | 12.718 | 642.866 | 315.903 | 12.213 | 1072.853 | 431.006 | 5.493 |
| TP@95% Percentile | 537.201 | 456.673 | 293.261 | 1040.378 | 852.692 | 529.682 | 1909.199 | 1616.888 | 818.635 |

We can know that relative higher throughput can be achieved for ISD = 100m with larger channel bandwidth than 100MHz. But considering 50% percentile, enough high throughput can be achieved for 200MHz and 400MHz channel bandwidth for ISD = 150m.

Observation 5: TDD pattern DSUUU can be considered for some UL heavy specific service with ISD = 100m and 150m; FR1 can be used for UL besides FR1 for larger ISD than 150m.

    - Full buffer traffic model

For the UL full buffer traffic model, we have the following evaluation results:

Figure 5: Simulation for UL with Full buffer traffic model (1Mb payload every 200ms, DDDSU)

Table 5: TP@5%, 50% and 95% percentile for UL with Full buffer traffic model with DDDSU TDD pattern

| CBW | 100 MHz |  |  | 200MHz |  |  | 400MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISD | 100m | 150m | 200m | 100m | 150m | 200m | 100m | 150m | 200m |
| TP@5% Percentile | 1.791 | 0.606 | 0.288 | 2.787 | 0.871 | 0.329 | 4.095 | 1.383 | 0.009 |
| TP@50% Percentile | 7.865 | 6.992 | 4.776 | 13.610 | 11.021 | 6.532 | 22.415 | 16.150 | 8.350 |
| TP@95% Percentile | 16.488 | 14.956 | 12.840 | 29.081 | 25.128 | 20.638 | 50.927 | 41.101 | 32.218 |

From the above simulation results, generally low throughput is observed for scenario with UL full traffic model, but still FR1 carrier can be used for UL higher throughput transmission.

As stated for DL, full buffer traffic model is not typical for FR2, but we also provide the results for comparison. Same as UL FTP traffic model, for some UL heavy specific service transmission with NO FR2 UL coverage limit, TDD pattern DSUUU also can be considered.

Figure 6: Simulation for UL with Full buffer traffic model (1Mb payload every 200ms, DSUUU)

Table 6: TP@5%, 50% and 95% percentile for UL with Full buffer traffic model and DSUUU TDD pattern

| CBW | 100 MHz |  |  | 200 MHz |  |  | 400 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISD | 100m | 150m | 200m | 100m | 150m | 200m | 100m | 150m | 200m |
| TP@5% Percentile | 6.045 | 1.910 | 0.860 | 9.554 | 2.510 | 0.965 | 13.583 | 4.082 | 0.026 |
| TP@50% Percentile | 24.886 | 21.752 | 14.840 | 43.003 | 33.937 | 20.389 | 70.530 | 50.184 | 25.643 |
| TP@95% Percentile | 50.685 | 46.223 | 38.885 | 89.829 | 77.863 | 62.644 | 157.267 | 124.938 | 97.521 |

Similar observation can be obtained as for UL FTP traffic model, i.e. FR1 can be used to achieved higher UL throughput.

From the above DL and UL evaluations results, we have the following observations and proposals:

Observation 6: It is feasible to use the assumed UE architecture with single panel, dual polarization, 2x1 array, min peak EIRP of 16.4 dBm for the targeted use cases under the FR2 network environment which mainly consists of LOS channel.

Proposal 1: Introduce FR2 power class 8 with UE architecture assumptions of single panel, dual polarization, 2x1 array, min peak EIRP of 16.4 dBm.

- Conclusion

As per our evaluation results shown above, our observations and proposals are summarized as following:

Observation 1: In the cell edge, for DL, both max throughput 300Mbps and 500Mbps can’t be achieved under the FR2 network with ISD larger than 150m for 100MHz transmission bandwidth.

Observation 2: In the cell edge, for DL, higher throughput than 300Mbps and 500Mbps can be well achieved under the FR2 network with ISD = 150m for channel bandwidth 200MHz and 400MHz.

Observation 3: FR1 can be used for UL transmission when there is UL power and coverage limit for FR2.

Observation 4: Higher throughput can be achieved at the cell middle and centre area for higher channel bandwidth by using CA.

Observation 5: TDD pattern DSUUU can be considered for some UL heavy specific service with ISD = 100m and 150m; FR1 can be used for UL besides FR1 for larger ISD than 150m.

Observation 6: It is feasible to use the assumed UE architecture with single panel, dual polarization, 2x1 array, min peak EIRP of 16.4 dBm for the targeted use cases under the FR2 network environment which mainly consists of LOS channel.

Proposal 1: Introduce FR2 power class 8 with UE architecture assumptions of single panel, dual polarization, 2x1 array, min peak EIRP of 16.4 dBm.

- Annex system simulation assumptions

Table 1: FR2 new PC8 UL/DL System Simulation Assumptions

| Parameters | Value |
| --- | --- |
| Scenario | UMa |
| Site layout |  |
| UE deployment | 100% Outdoor |
| Pathloss model | TR 38.901 Table 7.4.1-1: Pathloss models |
| LOS Probability | TR 38.901 Table 7.4.2-1 LOS probability |
| Traffic type | UL/DL: Full buffer DL: FTP Model 3 (0.5MB payload every 200ms) UL: FTP Model 3 (1Mb payload every 200ms) |
| Scheduling type | Proportional Fair |
| CQI/MCS table | 256QAM |
| BS antenna heights (m) | 25 |
| UT antenna heights (m) | 1.5 |
| h_E (m) | 1 |
| Number of UE | 10 per TRxP |
| SCS( kHz) | 120 |
| Carrier frequency (GHz) | 26 |
| Occupied channel bandwidth for PDSCH Per CC (MHz) | [100 for comparison], 200, 400 |
| Number of CC | 1 |
| occupied PRB Per CC | [66],132, 264 |
| ISD(m) | 150,100,200 |
| max layer | 2 |
| TDD pattern | DDDSU with S = 10:2:2 |
| DL Symbols with PDCCH | 1 |
| DM-RS configuration | Type 1: 1+1 (DL: Multiplexing with PDSCH, UL: Non- multiplexing with PUSCH) |
| xOverhead | 6 |
| Max transmission number | 4 |
| UL Codebook type | ULCB |
| DL codebook type | SRS based codebook selection |
| BS antenna radiation pattern [NOTE 1] | UPA, [m,n,p,mg,ng,mp,np] = [16,16,2,1,1, 1, 1] [dh,dv] = [0.5λ, 0.5λ], beam_theta = [90:5:155]° beam_phi = [-55:5:55]° polar_angle = [45, -45] element gain_pattern: General3D = { theta_3db = 65.0°, phi_3db = 65.0°, sla_v = 30.0dB, a_m = 30.0dB, max_element_gain = 8.0dBi } |
| UE antenna radiation pattern [NOTE 1] | UPA = { [m, n, p, mg, ng, mp, np] = [2, 1, 2, 1, 1, 1, 1], [dh,dv] = [NA, 0.5λ,], beam_theta = [90.0]°, beam_phi = [0.0]° } polar_angle = [0, 90]° element gain pattern: General3D = { theta_3db = 90.0°, phi_3db = 90.0°, sla_v = 25.0dB, a_m_db = 25.0dB, max_element_gain = 5.0dBi } |
| UE EIRP (dBm) | 16.4 |
| BS EIRP (dBm) | 70 |
| Thermal noise density (dBm/Hz) | -174 |
| BS Noise figure (dB) | 7 |
| UE Noise figure (dB) | 10 |
| UE velocity | 3km/h |
| UL Power control parameter | P0 = -86, alpha = 0.9 |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Criteria for analog beam selection for serving TRxP | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. |
| Criteria for analog beam selection for interfering TRxP | Based on the analog beam selection according to scheduling results of non-serving TRxP |
| NOTE 1: Refer to ITU R-REP-M.2412-2017-PDF-E Table 9 and Table 11 respectively |  |

- Reference

[1] R4-2414420 WF on introduction of new FR2 Power Class 8, RAN4#112, Huawei, HiSilicon

[2] R4-2415629 Updated simulation assumptions for introduction of FR2 Power Class 8, RAN4#112bis, Huawei, HiSilicon

[3] R4-2415630 CR on introduction of RF requirements for FR2 PC8 in TS 38.101-2, RAN4#112bis, Huawei, HiSilicon