3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415635

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on BS MMSE-IRC requirements with inter-cell interference

Source: 	Huawei, HiSilicon

Agenda item:	6.16.3

Document for:	Discussion

Background

In last meeting, WF R4-2413570 [1] was approved, one objective is related to BS MMSE-IRC requirements under interference scenario, this contribution provides our views.

- Discussions

Test scenarios

One proposal raised in last meeting is considering the scenario with different TDD patterns between BS and how to model the corresponding interference profile. However, such scenario outsides the WI scope according to the latest WI. Besides, we don’t think there is potential baseband processing difference between same and different TDD patterns. Alignment on TDD patterns is enough to verify the BS MMSE-IRC receiver.

Proposal 1: Not consider different TDD patterns.

Interference modelling criteria

There are two options for interference definition criteria: DIP based modelling with SINR based requirements definition and INR based modelling with SNR based requirements definition:

- Candidate options on interference modelling criteria: - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based modelling - Candidate options on DIP definition: - Option 1: Reuse definition of DIP in sub-clause B.6.1 of TS 36.104 - Candidate options on SINR definition: - Option 1:

Such two criteria are equivalent to each other and have no impact on performance. Considering INR based modelling with SNR based requirements definition criteria has been used in UE MMSE-IRC performance requirements definition. We slightly prefer to be aligned with UE side and consider INR based modelling with SNR based requirements definition.

Proposal 2: Use INR based modelling with SNR based requirements definition.

Interference profile

Based on our simulation results shown in clause 3, the performance gain of MMSE-IRC over baseline MMSE is considerable, which means current LTE modelling model is a good choice to verify MMSE-IRC receiver. Considering there is no SI assigned for this topic, we concern there is no enough time to explore new interference profiles by SLS. Hence we propose to reuse LTE interference profiles.

- Candidate options on interference modelling: - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - Other options not precluded - On interference power level: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - FFS how to account for HPUE in existing LTE profiles.

Proposal 3: RAN4 to reuse LTE interference profiles for NR MMSE-IRC BS requirements definition.

Number of interferers

One issue is number of interference cells. Candidate options are:

- Candidate options: - Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna - Other options not precluded

In LTE, only 1 interferer was considered for 2Rx MMSE-IRC requirements definition, the reason is that 2 interferers make the 2Rx MMSE-IRC performance gain over MMSE unobvious. Based on our simulation results in clause 3, the gain for 2Rx is over 1.9dB for DIP set1 and over 3.5dB for DIP set2, which is enough. Hence, we propose to consider 2 interference cells for 2Rx.

Proposal 4: RAN4 to consider 2 interference cells for 2Rx/4Rx/8Rx

Channel bandwidth

We have following options for channel bandwidth:

- Candidate options for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz - Candidate options for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz

We prefer to only consider minimum bandwidth for each SCS, i.e. 5MHz SCS for 15kHz SCS and 10MHz for 30kHz SCS. In Rel-15, all bandwidth for each SCS have been verified, it’s expected to reduce the test cases for bandwidth for future Release. It is mentioned that many Rel-18 requirements only cover minimum bandwidth such as ATG and enhanced DMRS, the approach of which can also be applied to Rel-19 MMSE-IRC requirements with interference cell considering bandwidth have negligible impact on performance.

Proposal 5: RAN4 to only consider minimum bandwidth for each SCS, i.e. 5MHz for 15kHz SCS and 10MHz for 30kHz.

Antenna configuration

We have following options for antenna configuration:

- Candidate options: - Option 1: Cover 2/4/8 Rx with 1Tx - Option 2: Consider only 4RX and 8RX gNB - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers - Option 5: Prioritize 1Tx

For Tx, we prefer to only consider 1Tx since we already propose to reuse LTE profiles, otherwise, we have to run SLS to study the new interference profiles for 2Tx. For Rx, based on our simulation results, all performance gain is larger than 2dB even for 1T2R, which is enough, hence we prefer to cover 2Rx,4Rx and 8Rx.

Proposal 6: RAN4 to consider 1T2R, 1T4R, 1T8R for requirements definition.

Propagation condition and MCS

We prefer to select one propagation condition and one MCS to reduce the test cost. Based on our simulation results in clause 3, MCS16 is a good choice to guarantee the reasonable SNR gain and SNR operating range. Hence, we slightly prefer MCS16, as for propagation conditions, we prefer TDLB100-400, which is usual configuration for MCS16. As for propagation conditions for interference cell, we prefer TDLC300-100.

Proposal 7: Only choose one MCS and one propagation conditions for requirements definition.

Proposal 8:  RAN4 to use MCS16, TDLB100-400 for serving cell and TDLC300-100 for interference cells

Waveform

We have following options for waveform:

Waveform - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

Based on our understanding, DFT-s-OFDM is rarely used in the real deployment, considering the timeline, we prefer to only focus on CP-OFDM.

Proposal 9: RAN4 to only consider CP-OFDM for requirements definition.

DMRS configuration

We have following options for DMRS configuration:

- Candidate options: - Option 1: DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); 0. DM-RS sequence generation; NID0=0, group hopping and sequence hopping are disabled. - Other options not precluded

Generally the proposed DMRS configuration looks reasonable, for the DMRS sequence generation, serving cells and interference cells shall have different DMRS initial seed to guarantee the orthogonality of DMRS sequence. Hence, we propose the following:

nSCID=0 for all cells, NID0=0 for serving cell, NID0=1 for interference cell1 and NID0=2 for interference cell2.

Proposal 10: RAN4 to configure nSCID=0 for all cells, NID0=0 for serving cell, NID0=1 for interference cell1 and NID0=2 for interference cell2.

Others

Same as legacy PUSCH requirements, we prefer to cover both PUSCH type A and type B to cover all base station types and use 70% of maximum throughput as metric.

Proposal 11: RAN4 to cover both PUSCH type A and type B and use 70% of maximum throughput as metric.

- Simulation results

In this clause we provide our initial simulation results to check the MMSE-IRC performance gain over MMSE-MRC under following assumptions:

Table 3-1: Simulation assumptions

| Assumptions |  | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | 5MHz/15kHz | 5MHz/15kHz | 5MHz/15kHz |
| Duplex mode |  | FDD | FDD | FDD |
| MCS |  | 2,16,20 | 16QAM random symbols | 16QAM random symbols |
| Propagation conditions: |  | TDLC300-100 Low for MCS2 TDLB100-400 Low for MCS16 TDLA30-10 Low for MCS20 | TDLC300-100 Low | TDLC300-100 Low |
| Antenna configuration |  | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R |
| Interference model |  |  | DIP set1: DIP1=-1.11dB DIP set2: DIP1=-0.43dB | DIP set1: DIP2=-10.91dB DIP set2: DIP2=-13.78dB |
| DM-RS | DM-RS configuration type | 1 | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB | -3 dB |
|  | DM-RS port | {0} | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Receiver assumption |  | MMSE-MRC, MMSE-IRC | N/A | N/A |

The simulation results are captured in Table 3-2:

Table 3-2: Simulation results

| DIP Set | MCS | Antenna configuration | MMSE-IRC (SNR(dB)@70% of maxTP) | MMSE (SNR(dB)@70% of maxTP) | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| DIP Set1 | MCS2 | 1T2R | 0.5 | 2.8 | 2.3 |
|  |  | 1T4R | -3.3 | 0.0 | 3.3 |
|  |  | 1T8R | -5.3 | -1.9 | 3.4 |
|  | MCS16 | 1T2R | 15.0 | 16.9 | 1.9 |
|  |  | 1T4R | 8.8 | 12.3 | 3.5 |
|  |  | 1T8R | 5.4 | 8.9 | 3.5 |
|  | MCS20 | 1T2R | 15.3 | 17.3 | 2.0 |
|  |  | 1T4R | 10.2 | 13.5 | 3.3 |
|  |  | 1T8R | 7.3 | 10.5 | 3.2 |
| DIP Set2 | MCS2 | 1T2R | 3.0 | 6.9 | 3.9 |
|  |  | 1T4R | 0.2 | 4.2 | 4.0 |
|  |  | 1T8R | -0.8 | 2.4 | 3.2 |
|  | MCS16 | 1T2R | 17.4 | 21.1 | 3.7 |
|  |  | 1T4R | 10.0 | 16.3 | 6.2 |
|  |  | 1T8R | 6.1 | 12.9 | 6.8 |
|  | MCS20 | 1T2R | 17.4 | 21.2 | 3.8 |
|  |  | 1T4R | 11.5 | 17.3 | 5.8 |
|  |  | 1T8R | 7.9 | 14.0 | 6.1 |

- Conclusion

This contribution provides our views on BS MMSE-IRC requirements. The proposals and observations are:

Proposal 1: Not consider different TDD patterns.

Proposal 2: Use INR based modelling with SNR based requirements definition.

Proposal 3: RAN4 to reuse LTE interference profiles for NR MMSE-IRC BS requirements definition.

Proposal 4: RAN4 to consider 2 interference cells for 2Rx/4Rx/8Rx

Proposal 5: RAN4 to only consider minimum bandwidth for each SCS, i.e. 5MHz for 15kHz SCS and 10MHz for 30kHz.

Proposal 6: RAN4 to consider 1T2R, 1T4R, 1T8R for requirements definition.

Proposal 7: Only choose one MCS and one propagation conditions for requirements definition.

Proposal 8:  RAN4 to use MCS16, TDLB100-400 for serving cell and TDLC300-100 for interference cells

Proposal 9: RAN4 to only consider CP-OFDM for requirements definition.

Proposal 10: RAN4 to configure nSCID=0 for all cells, NID0=0 for serving cell, NID0=1 for interference cell1 and NID0=2 for interference cell2.

Proposal 11: RAN4 to cover both PUSCH type A and type B and use 70% of maximum throughput as metric.

- Reference

[1]   R4-2413570 WF on NR demodulation performance: Phase 5.

[2]   Revised WID on NR demodulation performance: Phase 5. China Telecom, NTT DOCOMO