3GPP TSG-RAN WG4 Meeting # 112 bis	R4-2415636

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on spatial channeling modelling methodology

Source: 	Huawei, HiSilicon

Agenda item:	6.14.2

Document for:	Discussion

Background

In last meeting, WF R4-2413570 [1] was approved, one objective is related to 8Rx requirements under interference scenario, this contribution provides our views.

- Discussions

Extended TDL channel

In last meeting, two proposals for extended TDL channel were raised. One is based on existing TDL channel, with the changing that the correlation matrix per tap is derived as in [2], it is the most common channel modelling method. The pro of this approach is that each tap has different spatial component, which is more close to the real environment. Besides, variables cASD, cASA can be controlled to meet different test purposes. For example, if we are performing demod test, we can set high cASD and cASA values to make channel have more rays to support higher Rank. E.g. Rank8. If we are performing PMI test, we can set low cASD and cASA values to make channel more directional to have more precoding gain.

According to [2] ([17] noted in the above first yellow sentence), following assumptions are made: For simplicity, channel modelling is analyzed on 2D plane. Multiple clusters are modelled, each cluster has AOD (Angle of departure), ASD (Angle spread of departure), AOA(Angle of arrival) and ASA(Angle spread of arrival). BS antenna configuration: Two dual-polarized slant +45° and -45° elements are spatially separated by the distance d, where d is 0.5λ. BS antenna pattern: Aθ=-min12θθ3dB2,Am - Where is defined as the angle between the direction of interest and the bore sight of the antenna, is the 3dB beam width in degrees, and Am is the maximum attenuation. For a 3 sector scenario is 70 degrees, 20dB, maximum directional gain of an antenna element GE,max =8dBi UE antenna configuration: two dual-polarized slant +90° and -90° elements are spatially separated by the distance d, where d is 0.5λ. UE antenna pattern: omni-directional antenna. Figure 2-1: Proposed TDL based channel modelling The correlation matrix for each cluster can be derived from following steps: Step1: Calculate antenna correlation between BS side for one polarization. For ith cluster, correlation coefficient of antenna m and n is defined as following: ρBS,m,ni=φAODi-12ASDφAODi+12ASDej2πλm-ndcosφA(φ)PAS(φ)dφφAODi-12ASDφAODi+12ASDA(φ)PAS(φ)dφ Where φAODi is angle of departure of ith cluster. ASD is angle spread, A(φ) is antenna gain at φ, PAS(φ) is power angular spectrum for each cluster. Then we can derive correlation matrix Ai for ith cluster at BS side: Am,ni=ρBS,m,ni Step2: Calculate antenna correlation between UE side for one polarization. Similar as BS side, for ith cluster, correlation coefficient of antenna m and n can be derived as following: ρUE,m,ni=φAOAi-12ASAφAOAi+12ASAej2πλm-ndcosφA(φ)PAS(φ)dφφAOAi-12ASAφAOAi+12ASAA(φ)PAS(φ)dφ Where φAOAi is angle of arrival of ith cluster. ASA is angle spread, A(φ) is antenna gain at φ, PAS(φ) is power angular spectrum for each cluster. Then we can derive correlation matrix Bi for ith cluster at: UE side: Bm,ni=ρUE,m,ni Step3: Calculate polarization matrix The channel polarization matrix is given as following: S=SvvSvhShvShh Where Svv=Shh=1, Svh=Shv=10-XPR20, XPR can be selected from typical values defined in CDL parameter table in 38.827. The polarization matrixes at the BS and UE are the following respectively PBS=cosθ1,BScosθ2,BSsinθ1,BSsinθ2,BS PUE=cosθ1,UEcosθ2,UEsinθ1,BSsinθ2,UE Where θ1,BS, θ2,BS are the polarization of the antennas of BS and θ1,UE, θ2,UE are the polarization of the antennas of UE. The total channel is the matrix product of BS polarization, channel polarization and UE polarization for the downlink case: Then polarization correlation matrix is given as follows: Step4: Calculate the total channel correlation matrix Channel correlation matrix for ith cluster is given as follows, which is same approach as clause B.2.3.2.1 in 38.101-4 Rspat,i=PAi⨂Γ⨂BiPT Noted that all above analysis are based on 2D, which can easily be extended to 3D by applying another angular variable. AOA, AOD, XPR can be reused from AOA(Azimuth angle of Arrival) and AOD(Azimuth of Departure) of CDL table defined in 38.827/38.901, or can be redefined. PAS can be replaced by reusing table 7.5-3 of 38.901(copied as Table 2-1) or can be redefined by using Laplace distribution or uniform. Table 2-1 Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m. 1,2; ± 0.0447. 3,4; ± 0.1413. 5,6; ± 0.2492. 7,8; ± 0.3715. 9,10; ± 0.5129. 11,12; ± 0.6797. 13,14; ± 0.8844. 15,16; ± 1.1481. 17,18; ± 1.5195. 19,20; ± 2.1551. For information, Laplace distribution is defined as following: Pφ,σ,φ=N0-2φ-φσ φ is AOD/AOA of the given cluster, σ is RMS angle spread. Finally, No is the normalization constant: N0-1=-π+φπ+φexp-2φ-φσdφ

We compare the PDSCH requirements with follow PMI vs random PMI when cASD and cASA equal to 2°and 20°in figure 2-2. It can be observed that performance of random PMI gets worse as angular spread increases. Therefore, PMI requirement can be better verified with low cASD and cASA values.

Figure 2-1: Follow PMI vs Random for different angular spread

We compare the PDSCH requirements with random PMI requirements with rank4, in Figure 2-3, it can be observed that PDSCH requirements with small cASD,cASA is much worse than high cASD,cASA. Therefore, PDSCH requirement can be better verified with high cASD and cASA values.

Figure 2-2: PDSCH requirements with Rank4 for different cASD and cASA

Besides, this approach has one pro that it has much less random variables, which is more easily for simulation alignment compared CDL channel.

Observation 1: The proposed extended channel model in [2] has following pros:

Two variables cASD and cASA is configurable to meet different test purposes. I.e. PDSCH requirements and PMI requirements

Close to real environment

Easy to align simulation results.

We propose to capture this extended channel model and corresponding observation and simulation in the TR.

Proposal 1: Capture the extended channel model and corresponding observation and simulation in the TR

CDL channel

Channel model

In last meeting, RAN4 agreed to prioritise UMa-CDL-C for initial comparison and alignment and use following UE antenna configuration:

Agreement: UE Antenna Configuration, for DL, for 38.827 : Cross Pol (1, n/2, 2), i.e. [ + + + +]. Where ‘n’ is number of Rx with half lambda spacing, with omni antenna element radiation pattern.

However, UMa-CDL-C channel has extreme small angular spread of ZOA and ZOD(See Clause 4), especially for ZOD, there is no angular spread, which causes rank deficiency, our simulation shows that 8 layers can’t be supported by Uma-CDL-C.

Observation 2: UMa-CDL-C has extreme small angular spread of ZOA and ZOD, which has no enough spatial freedom to support 8 layers.

As we pointed out in last meeting, the selected channel model shall have enough spatial freedom to support high layers. CDL-A channel in 38.901 is a good choice. We did some initial simulation results for Rank8 shown in Figure 2-2. For simulation with analog beamforming, we assumed antenna configuration with (M,N,Me,Ne,P)=(1,1,8,8,2), CSI ports {3000,3001,3002,3003} corresponds to all antenna elements one polarization, CSI ports{3004,3005,3006,3007} corresponds to all antenna elements with another polarization.

- 1st Strongest beam direction: ZOD:100°, AOD°-7.27°:

- 2nd Strongest beam direction: ZOD: 100°, AOD:-21.82°

- 3rd Strongest beam direction: ZOD:100°, AOD: -36.36°

- 4th Strongest beam direction: ZOD: 100°, AOD: -50.91°

Noted that this BB to AE mapping is proposed by some companies in the contribution and email, not officially defined.

It can be observed that CDL-A defined in 38.901 has enough spatial freedom has reasonable SNR range for 8layers.

Figure 2-2: Initial simulation results for CDL-A channel with/without analog beamforming.

Observation 3: CDL-A channel defined in 38.901 has good spatial freedom to support 8layers with reasonable SNR range.

If other companies have the same observations, it’s proposed to modify the channel parameters of UMa-CDL-C to support high layers. For example, replace the parameters of UMa-CDL-C by parameters of CDL-A model defined in 38.901.

Proposal 2: RAN4 shall modify the channel parameters of UMa-CDL-C to support high layers. For example, replace the parameters of UMa-CDL-C by parameters of CDL-A model defined in 38.901.

Analog beamforming

As pointed in the email discussion, according to 38.827, it is not clear how to map BB ports to antenna elements and how to implement beamforming for 8x8 MIMO. RAN4 shall further discuss whether it is necessary to implement analog beamforming in the demod and PMI test. If yes, RAN4 should discuss realistic map between BB ports and antenna elements and beamforming approach for demod/PMI test and capture it clearly in the TR and WF.

Proposal 3: RAN4 shall discuss whether it is necessary to implement analog beamforming in the demod and PMI test.

Proposal 4: If it is agreed to use analog beamforming, RAN4 should discuss realistic map between BB ports and antenna elements and beamforming approach for demod/PMI test

- Conclusion

Observation 1: The proposed extended channel model in [2] has following pros:

Two variables cASD and cASA can be controlled to meet different test purposes. I.e. PDSCH requirements and PMI requirements

Closed to real environment

Make it easiler to align simulation results.

Proposal 1: Capture the extended channel model and corresponding observation and simulation in the TR

Observation 2: UMa-CDL-C has extreme small angular spread of ZOA and ZOD, which has no enough spatial freedom to support 8 layers.

Observation 3: CDL-A channel defined in 38.901 has good spatial freedom to support 8layers with reasonable SNR range.

Proposal 2: RAN4 shall modify the channel parameters of UMa-CDL-C to support high layers. For example, replace the parameters of UMa-CDL-C by parameters of CDL-A model defined in 38.901.

Proposal 3: RAN4 shall discuss whether it is necessary to implement analog beamforming in the demod and PMI test.

Proposal 4: If it is agreed to use analog beamforming, RAN4 should discuss realistic map between BB ports and antenna elements and beamforming approach for demod/PMI test

- Reference

[1]  R4-2413606 Way Forward for [112][327] FS_NR_demod_SCM

[2]  R4-2412762 Views on spatial channel model. Huawei, HiSilicon

- Appendix

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | Z | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |