3GPP TSG-RAN WG4 Meeting#112-bis		R4-2415641

Hefei, China, Oct 14th -18rd, 2024

Agenda item:	6.1.1.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on SRS antenna switching requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for SRS antenna switching and ΔTRxSRS

Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Way forward: Further discuss the following options. Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

As we all known, if the frequency is higher, the insertion loss of front-end devices and PCB traces will be higher. However, according to our practical experience, ∆TRxSRS value of n104 is higher than n79 by 0.5dB. In our understanding, there is no obvious difference between n79 and n104 for ∆TRxSRS value. In addition, according to the average value of ∆TRxSRS , ∆TRxSRS value of n104 is not more than 0.5dB higher than n79. On the whole, there is no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 1: Support option1.There is no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

ΔTRXSRS value

Issue 2-2-1: Proposed ∆TRxSRS values Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n41, n77, n78; 4.0; 3.5; 4.5. - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

A conclusion was reached at RAN-P #104 meeting that RAN1 should study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. SRS antenna switching to support 4T6R is precluded. According to the above conclusion, we could first consider ΔTRXSRS for 1T6R, 2T6R for PC3. It is observed that 2T6R can be divided into 1T4R and 1T2R. Thus we can get the value of ΔTRXSRS for 2T6R is 3 dB. For 1T6R, we can see the difference with 8RX (in Figure1 and Figure2 in Annex) is the length of PCB (0.5dB), thus we can get ΔTRXSRS of 1T6R is 3.5dB. Similarly, we did not see the difference between the 1T6R-2T6R architecture and1T6R. And so on, we can get the following table.

Proposal 2: Adopt the value of ΔTRXSRS in Table 1 for 1T6R, 2T6R for PC3.

Table 1: ΔTRXSRS for 1T6R, 2T6R for PC3

| UE capability | Bands whose FUL_high is lower than the FUL_low of n79 (dB） | Bands whose FUL_high is higher than the FUL_low of n79 (dB） |
| --- | --- | --- |
| 1T6R/1T6R-2T6R | 3.5 | 5 |
| 2T6R | 3 | 4.5 |

According to the above analyse, in our understanding, there is no need to introduce the additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 3: Support option10, we accept that adopt the average values of ∆TRxSRS from company proposals without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: Support option1.There is no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 2: Adopt the value of ΔTRXSRS in Table 1 for 1T6R, 2T6R for PC3.

Table 1: ΔTRXSRS for 1T6R, 2T6R for PC3

| UE capability | Bands whose FUL_high is lower than the FUL_low of n79 (dB） | Bands whose FUL_high is higher than the FUL_low of n79 (dB） |
| --- | --- | --- |
| 1T6R/1T6R-2T6R | 3.5 | 5 |
| 2T6R | 3 | 4.5 |

Proposal 3: Support option10, we accept that adopt the average values of ∆TRxSRS from company proposals without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

- Reference

- R4-2414285, WF on requirements for 6RX, AT&T.TSG-RAN4#112 meeting.

- Annex

Figure 1: an example of 1T6R architecture                         Figure 2: an example of 1T8R architecture