3GPP TSG-RAN WG4 Meeting#112bis							         		R4-2415645

Hefei, China, 14 – 18 October 2024

Title: 	Preliminary input on DualPA PC1.5 non-contiguous ULCA MPR based on measurements

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.1.1.1.2 Intra-band contiguous and non-contiguous UL CA with PC1.5 [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112, we discussed PC1.5 intra-band non-contiguous ULCA in [1] and the emissions issues related to PSD imbalance for ULCA in [2] and, our proposal to evaluate MPR for up to 6dB PSD imbalance was captured in way forward [3]. In this contribution, we provide some preliminary measurements results that compare DualPA PC1.5 non-contiguous ULCA required backoff with the DualPA PC2 case, while also assessing the impact of supporting some level of PSD imbalance as proposed in [4].

# Discussion

## 2.1 Way forward inputs

The way forward [2] captured our input on PSD imbalance in [1] in the following agreement related to non-contiguous ULCA in italics.

Online agreement:

- MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

- PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

Furthermore, it also discussed ULCA PCmax aspects similar to what was done in previous meetings within the following agreements in italic.

Adhoc agreement:

- For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC

  - Whether or how to reflect the above agreement in spec is FFS

Online agreement with adhoc modification:

- For intra-band UL NC CA with dual PA architecture, the upper bound of Pcmax is 29dBm

In this contribution, we further evaluate the impact of up to 6dB PSD imbalance on the PC1.5 non-contiguous ULCA MPR based on measurements.

## 2.2 Measurement description.

Note that this evaluation is not yet meant to provide a MPR value proposals as:

- It has a limited set of waveforms: only CP-OFDM and DFT-s-OFDM QPSK are evaluated with a set of corner waveforms.

- It does not account for limitations of ET or APT implementations at large back-offs (these would require an assessment at lower voltages as the back-off increases resulting in a “slower” improvement of the IMDs with back-off.

- The measurements focus on assessing two aspects to provide some insight on how PSD imbalance impacts the MPR.

Measurement parameters:

- DualPA PC1.5 measurements:

  - Uses two band n77 PAs calibrated MPR1 at 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveform.

  - Coupled at 10dB and with 3dB post PA losses.

  - The measured output of each PA is summed mathematically for PC1.5 power capability.

- Configurations and power

  - Non-contiguous ULCA with different gaps between allocations centers: 400, 200, 100 and 50MHz to check the impact of frequency distance on RIMD.

  - Only 15kHz SCS QPSK CP-OFDM and a few DFT-S-OFDM waveforms are measured.

  - Each CC is allocated with permutations of 1, 2, 4, 6, 8, 24, 32, 64, 96 and 128RBs to exercise different total allocated BW and different allocated BW per CC

  - Different power balance cases are tested:

    - Equal PSD (=equal power when the same allocation is used in both)

    - 6dB PSD imbalance (2 cases depending on which CC has the lower PSD, in some cases the 6dB PSD imbalance results in equal power)

    - Some equal power corner cases are evaluated to further evaluate the impact of PSD imbalances.

  - Measurements only at the calibration voltage (does not account for ET APT) with back offs up to 9dB.

  - Only IMD3 and IMD5 levels are measured and checked versus the two MPR levels of:

    - IMD3 for -13dBm/MHz (checking that IMD5 passes -30dBm/MHz) SEM

    - IMD3 for -30dBm/MHz spurious emissions

- Overall, ~360 configurations and power levels were tested with close to 2200 measurements resulting in 1 days of continuous automated testing.

Observation:

- Even with such a limited waveform set, the test time is important while the SEM is measured indirectly by focussing on the IMD frequencies.

- To perform a fully representative set of waveforms, CA configurations, allocation types and accounting for PSD imbalances and APT/ET operation at large backoff will require many days of measurements:

- Hopefully, the set may be reduced based on selecting corner cases from simulations,

- Otherwise, patience will be needed for a complete MPR proposal or it is evaluated as a delta versus DualPA PC2 with a limited set of waveforms.

## 2.3 DualPA PC1.5 contiguous ULCA MPR versus existing specification.

To have a first insights on the DualPA PC1.5 MPR, it can be compared with the existing DualPA PC2 case as it is only an increase of the maximum power by 3dB, but PAs have the same power capability and RIMD is already in play. If it was the case one could expect the IMD3/5 power levels to increase by 3dB in power and thus ~1dB of additional MPR would reduce these IMDs back to the desired SEM level.

However, this is not so simple as the DualPA PC2 non-contiguous ULCA was dimensioned to enable an implementation with one PC2 PA and one PC3 PA and allocating the PC2 PA to the CC with >23dBm power and switching the LO accordingly. On top of creating UL interruptions due to LO switching compared to the straight forward implementation with two 26dBm PAs, this also means that one of the PA is less linear (23dBm vs 26dBm but also 30dB MPR1 vs 31dB MPR1) and thus more sensitive to RIMD and consequently higher MPR. This means that we cannot directly compare the dualPA PC1.5 non-contiguous ULCA MPR with the DualPA PC2 MPR.

Observation: DualPA PC1.5 non-contiguous ULCA MPR cannot be directly derived from dualPA PC2 case as the latter is not based on two PC2 PAs.

## 2.4 DualPA PC1.5 non-contiguous ULCA MPR versus PSD imbalance.

In order to compare the required back-off for different PSD imbalance and especially for 6dB imbalance like we have proposed in [4], but also the effect of the separation bandwidth, the relevant results are presented in Table 1 and Table 2:

- Equal PSD case is denoted “=PSD” (this is also equal power for cases with same allocation BW in each CC)

- “+6dB” denotes 6dB PSD imbalance with increased power on CC1 vs CC2.

- “+6dB” denotes 6dB PSD imbalance with increased power on CC2 vs CC1.

- Equal Power case is denoted “=PWR”.

- Table 1 focusses on 1RB+1RB allocation (29dBm can be reached as the same allocation is used in each CC) at different distances for CP-OFDM and DFT-s-OFDM waveforms at equal PSD.

- Table 2 focusses on CP-OFDM waveforms for different PSD and RB BW imbalance with PCmax<29dBm in some cases, and results in deltaPowerClass < 6dB as discussed and proposed in [4]. The distance between the centers of the allocations is kept at 100MHz.

Table 1: DualPA PC1.5 backoff results for worst case CP-OFDM and DFT-s-OFDM allocation versus frequency gap.

| Allocation configuration |  |  | CP-OFDM backoff for IMD3 of |  | DFT-s-OFDM backoff for IMD3 of |  |
| --- | --- | --- | --- | --- | --- | --- |
| #RB1 | #RB2 | Fgap [MHz] | -13dBm/MHz | -30 dBm/MHz | -13 dBm/MHz | -30 dBm/MHz |
| 1 | 1 | 100 | 3 | 7.8 | 2.8 | 7.3 |
| 1 | 1 | 200 | 2.3 | 7.2 | 2.2 | 6.6 |
| 1 | 1 | 400 | 3.1 | 7.8 | 2.8 | 7.1 |
| 1 | 1 | 50 | 3.1 | 7.3 | 2.3 | 6.6 |
| 1 | 1 | 600 | 2.9 | 7.6 | 2.6 | 6.8 |

Observations:

- As expected, the DFT-s-OFDM backoff is slightly lower than for CP-OFDM but of the same order of magnitude within less than 0.5dB difference => it is sufficient to evaluate CP-OFDM only.

- There is little variation versus the gap distance, but it is higher than 0.5dB => different separation bandwidths should be evaluated

- Note that this is evaluated with 1RB+1RB which is more sensitive to IMD asymmetry and memory effects.

Table 2: DualPA PC1.5 backoff results for CP-OFDM allocations versus PSD imbalance.

| configuration |  | backoff for -13dBm/MHz IMD3 |  |  |  |  | backoff for -30dBm/MHz IMD3 |  |  |  |  | deltaP PowerClass [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #RB1 | #RB2 | =PSD | +6dB | -6dB | =PWR | =PSD |  | +6dB | -6dB | =PWR | =PSD |  | +6dB | -6dB | =PWR |
| 1 | 1 | 3 | na | na | na | 7.8 |  | na | na | na | 6 |  | na | na | na |
| 1 | 2 | 2.3 | 3 | 0.8 | na | 7.4 |  | 8.2 | 6 | na | 5.2 |  | 6 | 4.8 | na |
| 1 | 4 | 1.1 | 2.7 | na | na | 7 |  | 8.1 | na | na | 4.5 |  | 6 | na | na |
| 1 | 8 | 0 | 1.8 | na | 2.1 | 5.6 |  | 7.4 | na | 7.3 | 3.9 |  | 5.8 | na | 6 |
| 1 | 32 | 0 | 0 | na | 0 | 1.8 |  | 4 | na | 6 | 3.3 |  | 3.9 | na | 6 |
| 1 | 128 | 0 | 0 | na | 0 | 0 |  | 0 | na | 4 | 3.1 |  | 3.3 | na | 6 |
| 2 | 6 | 1.2 | 2.4 | na | na | 6.9 |  | 7.7 | na | na | 4.8 |  | 6 | na | na |
| 4 | 4 | 2.6 | na | na | na | 8 |  | na | na | na | 6 |  | na | na | na |
| 8 | 24 | 0 | 0.1 | 0 | na | 5.3 |  | 6.2 | 5.4 | na | 4.8 |  | 6 | 5.5 | na |
| 16 | 16 | 0.4 | na | na | na | 6.6 |  | na | na | na | 6 |  | na | na | na |
| 24 | 32 | 0 | 0 | 0 | na | 5.5 |  | 5.7 | 3.5 | na | 5.7 |  | 6 | 4.2 | na |
| 32 | 32 | 0 | na | na | na | 5.8 |  | na | na | na | 6 |  | na | na | na |
| 32 | 96 | 0 | 0 | 0 | na | 3.5 |  | 4.4 | 3.8 | na | 4.8 |  | 6 | 5.5 | na |
| 64 | 64 | 0 | na | na | na | 4.9 |  | na | na | na | 6 |  | na | na | na |

Observations:

- The worst back-off occurs for the equal power case for large BW imbalance between the two CCs. In some cases it can be more than 2dB and 4dB higher than the equal PSD case for -13 and -30dBm/MHz IMD3 respectively (red highlight).

- The worst back-off between equal PSD and 6dB imbalance (bold) is in all cases for the 6dB imbalance with:

  - Up to 1.8dB impact at both -13 and -30dBm/MHz IMD3.

  - Limited impact for large total allocation bandwidth which have the lower MPR (to be verified in some gap cases for ACLR).

- Even allowing for up to 6dB PSD imbalance, deltaP PowerClass can be close to 3dB versus 3dB for some allocations.

The overall conclusion is that accounting for 6dB PSD imbalance in the MPR is viable as, if it has an impact, it is limited for the worst-case allocations. However, supporting equal power has a significant impact for MPR for little value as it further impacts the CC with the larger allocated RB BW.

Proposal on power imbalance:

- Equal power is not supported by the MPR and is not evaluated.

- 6dB PSD imbalance is supported by MPR and evaluated (as proposed in [4]).

## 2.5 Proposals for DualPA PC1.5 non-contiguous ULCA MPR evaluation

Based on the above observations, we make a detailed proposal for the MPR evaluation plan.

Proposal on DualPA PC1.5 non-contiguous ULCA MPR evaluation: Given the complexity of the number of configurations to be measured, the evaluation is performed via:

- Measurement of corner cases in DualPA setup with 10dB cross-coupled PAs.

- Measurements and simulations should account for APT/ET PA behavior at larger back-off.

- 100MHz, 200MHz and 600MHz separation BW classes are evaluated.

- Both Equal PSD and worst case 6dB imbalance are evaluated and DeltaP PowerClass <6dB in some cases accounted for.

- Only CP-OFDM is evaluated as it covers for DFT-s-OFDM case with small margin for:

  - -13dBm/MHz IMD3 (IMD5 < -30dBm/MHz may be verified)

  - -30dBm/MHz IMD3.

  - Additional cases should be assessed when both CC ACLR overlap.

# Conclusion

In this contribution, based on our previous work on the topic we performed back-off measurements for DualPA PC1.5 and various PSD imbalance to assess their differences and make the following proposals for future evaluations.

Proposal on power imbalance:

- Equal power is not supported by the MPR and is not evaluated.

- 6dB PSD imbalance is supported by MPR and evaluated (as proposed in [4]).

Proposal on DualPA PC1.5 non-contiguous ULCA MPR evaluation: Given the complexity of the number of configurations to be measured, the evaluation is performed via:

- Measurement of corner cases in DualPA setup with 10dB cross-coupled PAs.

- Measurements and simulations should account for APT/ET PA behavior at larger back-off.

- 100MHz, 200MHz and 600MHz separation BW classes are evaluated.

- Both Equal PSD and worst case 6dB imbalance are evaluated and DeltaP PowerClass <6dB in some cases accounted for.

- Only CP-OFDM is evaluated as it covers for DFT-s-OFDM case with small margin for:

  - -13dBm/MHz IMD3 (IMD5 < -30dBm/MHz may be verified).

  - -30dBm/MHz IMD3.

  - Additional cases should be assessed when both CC ACLR overlap.

# References

[1] R4-2412277 On PC1.5 intra-band non-contiguous ULCA with dualPA, Skyworks Solutions Inc., RAN4#112

[2] R4-2411302 On equal PSD vs equal power spectral regrowth, Skyworks Solutions Inc., RAN4#112

[3] R4-2414277 WF on HPUE for CA in TN, Samsung, RAN4#112

[4] R4-2415275 On PC1.5 ULCA PCmax and UE behavior under large PSD imbalance, Skyworks Solutions Inc., RAN4#112b