3GPP TSG-RAN WG4 Meeting #112bis	R4-2415673

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	BT, Orange

Title:	Further observations from deployment and MU-MIMO use cases for SCM evaluation

Agenda item:	6.1.4.2

Document for:	Discussion

Introduction

During RAN4#112 a way forward for channel modelling study was approved [1].

In this contribution we provide our observations of spatial characteristics of measured MIMO channels from deployments, focusing on the evolution of spatial preferences of channels in time and frequency. Earlier contributions [2,3] reported basic expected properties of an SCM based on measurements in deployments.

Proposals for MU-MIMO scenarios for SCM are provided in Section 3.

Expected model behaviour based on field measurements

Spatial stability

Modelling spatial properties of the MIMO radio propagation channel in a way that is consistent with properties observed in deployment is essential for reliability of test scenarios for demodulation performance and CSI reporting quality requirements. In [2] we provided several characteristics of MIMO channels observed from measurements in sample commercial deployments.

This section provides further detail on the spatial stability of measured channels. We believe it is essential that an SCM can generate test conditions that are consistent with key physical properties of the propagation environment, such as ones reported here and in [2].

Measurement setup

Measurement conditions were outlined in Section 3.1 of [2] but are repeated in the next paragraph for completeness.

The MIMO channel measurements were conducted using a typical commercial MIMO deployment using a base station with 4 transceiver ports and receiving equipment with 4 ports. Measurements were conducted in a 15MHz channel at 2162.2MHz center frequency, over several locations in a campus-type environment shown in Figure 1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup. The output of the measurements were complex channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. Noise power was estimated at the receiving equipment, while the transmitter power setting was 15 dBm per resource element (15 kHz subcarrier spacing).

Results presented in this contribution are derived from measurements collected at Location A. The location can be characterised as strong line-of-sight profile, yet with several reflections from surrounding buildings

Figure 1. Measurement point locations for the sample MIMO channels from a typical 4T4R base station.

PDP stability

Figure 2 illustrates the power delay profiles (PDP) for a sample channel between single transmitter port and single receiver port (h11). We observe that the PDP is virtually constant over 250ms.While this does not allow us to draw conclusions concerning the spatial preferences of the channel, it does show that the spatial preference is stable for the measurement time, assuming that a changing path/reflection would also change the excess delay of the path.

Figure 2. Sample power-delay profile for channel between transmitter port 1 and receiver port 1 (i.e. h11), showing the evolution of signal reflections in time domain.

Such stable behaviour was observed for all 16 for the 4x4 MIMO channel measured at Location A. Figure 3 provides average PDPs for the full set of individual links. The averaging is over time. Apart from the expected stability of spatial preferences, Figure 3 demonstrates the differences in propagation conditions among polarisations.

The power delay profiles from reflections of the transmitted reference signals exhibit stability across time (~250ms in this specific measurement sample) for static scenarios. Such behaviour is consistent with expectations, and should be reproducible in an SCM channel model.

Figure 3. Average power-delay profiles for the 16 links between 4 transmitter ports and 4 receiver ports for Location A. Figures on the left are for transmitter ports mapped to polarization 1. Figures on the rights are for transmitter ports mapped to polarization 2. Averaging is across circa 250ms of measurement time.

Stability of reflections

The plots in Figure 4 present the phase difference between signals transmitted from two cross- and co-polarized AEs, measured over both frequency and time. It is observed that the phase shift between the two AEs remains virtually constant over time (for each subcarrier). While the phase alone cannot be directly used to determine the direction of departure (DoD), it can at least be concluded that the DoD(s) of the channel are not significantly changing over 250ms.

At this point we would note results in [4], e.g. Figures 7 and 10, demonstrating the effect of TDL channel model lacking the property of long-term stable distinct spatial directions (DoA or DoD) with significant power.

Figure 5. Differences in phases (e.g. phase of h11 minus phase of h13) of signals transmitted from two distinct antenna ports. The transmitting ports 1 and 3 for figure on the left are in a cross-polarized arrangement, while transmitting ports 1 and 2 for figure on the right are co-polarised.

The directions of departure (DoD) exhibit a long-term stable (at least 250ms) spatial preference through differences in phases of respective reference signals.

The SCM channel model shall produce explainable long-term stable spatial characteristics of the channels including departure and arrival angles, and power-delay profiles.

Multi-user MIMO use cases

This section provides proposals for use cases where multiple users or cells are involved in communication.

Single-cell

For throughput test cases for MU-MIMO in a single cell, adopt the test case setting, except for the channel model as in Sec. 5.2.3.1.16 and 5.2.3.2.17 of TS 38.101-4. A CDL-based channel model should be applied using antenna configuration in existing specification.

For PMI reporting quality 6.3.3.1.6 and 6.3.3.2.6 should be adopted as a starting point, even though not an MU-MIMO test setup. Additional cases should include a CDL-based channel model, and a comparison of random Enhanced Type II Codebook precoder as a reference. We note that the need to assess PMI reporting is also highlighted in contribution [4] to this meeting, comparing the performance of Type-I and eType-II codebooks under TDL and CDL modelling approaches.

MNOs are ultimately interested in a minimum system performance involving at least a PMI (CSI) report from multiple UEs, and appropriate baseline processing at the base station, resulting in a reference service performance.

For throughput cases for MU-MIMO evaluation of SCM, adopt the test case settings as in Sec. 5.2.3.1.16 and 5.2.3.2.17 of TS 38.101-4 with additional CDL-based channel model.

For PMI reporting quality in a multi-user setting, 6.3.3.1.6 and 6.3.3.2.6 of TS 38.101-4 with additional CDL-based channel model should be adopted as a starting point, to meaningfully test and compare PMI performance (for both Type-I and eType-II) in multi- and single-user settings, and ultimately for assessment of aspects of system performance involving both throughput and 3GPP PMI/CSI reporting functionality.

Multi-cell (Multi-TRP)

Scenarios involving interaction between multiple transmitting and receiving points already occur in field deployments, and are expected to play an important role in future deployments. Specific cases that we expect are required to be accounted for in 3GPP-standardised features include:

- inter-cell interference at a target UE;

- communication in a multi-TRP context, or other cases with non-collocated base stations (similar to EN-DC, or inter-site CA).

Consider scenarios involving inter-cell interference at a target UE, and/or cases with non-collocated base stations such as Multi-TRP for SCM evaluation.

Conclusion

Observations and proposals for the SCM are summarised below.

- The power delay profiles from reflections of the transmitted reference signals exhibit stability across time (~250ms in this specific measurement sample) for static scenarios. Such behaviour is consistent with expectations, and should be reproducible in an SCM channel model.

The directions of departure (DoD) exhibit a long-term stable (at least 250ms) spatial preference through differences in phases of respective reference signals.

- The SCM channel model shall produce explainable long-term stable spatial characteristics of the channels including departure and arrival angles, and power-delay profiles.

For throughput cases for MU-MIMO evaluation of SCM, adopt the test case settings as in Sec. 5.2.3.1.16 and 5.2.3.2.17 of TS 38.101-4 with additional CDL-based channel model.

For PMI reporting quality in a multi-user setting, 6.3.3.1.6 and 6.3.3.2.6 of TS 38.101-4 with additional CDL-based channel model should be adopted as a starting point, to meaningfully test and compare PMI performance (for both Type-I and eType-II) in multi- and single-user settings, and ultimately for assessment of aspects of system performance involving both throughput and 3GPP PMI/CSI reporting functionality.

Consider scenarios involving inter-cell interference at a target UE, and/or cases with non-collocated base stations such as Multi-TRP for SCM evaluation.

References

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Nokia, RAN4#112, Aug 2024.

- R4-2411557, “Scenarios and requirements for spatial channel modelling”, BT

- R4-2402277, “Discussion on 8Rx general demodulation aspects and spatial channel models”, Nokia, Nokia Shanghai Bell, BT

- R4-2415285, “TDL and CDL comparison with respect to the CSI resolution feedback”, Orange, BT, Telecom Italia, Vodafone, Bouygues Telecom, Fraunhofer, Deutsche Telekom