3GPP TSG RAN WG4 #112bis	R4-2415674

Hefei, China, Oct 14 – 18, 2024

Agenda item:		6.11.3

Source:	Nokia

Title:	On OTA test enhancements - CLTA

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#112 regarding OTA test enhancements. Unfortunately there was not much time for OTA test enhancements so the WF doesn’t include ant agreements but merely lists the topics that were not discussed. There was also an offline meeting regarding the topic held between the interested parties with some agreements and those agreements will be discussed in this paper.

This contribution further elaborates Nokia’s view on wideband horn antennas and the translation with co-location reference antenna. It also presents our view on the aspect of typical deployment and related assumptions.

# Discussion

## Alternative testing solution for the co-location test antenna

This was agreed in the offline meeting:

Agreement: For bands lower than band [n41/3.5GHz], to reuse the existing co-location requirement e.g. co-location reference antenna definition; - How to provide the alternative solution for the colocation testing antenna e.g. wide band horn antenna is FFS For bands above band [n41/3.5GHz], - How to provide the alternative solution for the colocation testing antenna e.g. wide band horn antenna is FFS - No update with the existing requirements for co-location spurious emission requirement and co-location blocking requirements; For FR1 high bands, companies are encouraged to identify the difference with the following two options: 1)AAS BS with colocation reference antenna; 2) AAS BS vs AAS BS;

We have proposed in our earlier contributions that using wideband horn antennas as CLTA’s could be very beneficial concerning cost and test efforts. Single column passive antennas are not readily available for all of the required frequency bands, and they typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. However, there is a need to ensure that wideband horn antennas deliver correct results. To see how wideband horn antennas behave when used as CLTAs with the same setup was tested. The spurious emissions measurements for 3GPP frequency bands were done for two AAS and it was measured with side-by-side distance of 10cm for CLTA and the horn antenna on both sides as depicted in setup A and B, respectively. Three measurements were repeated in order to secure some average result for the spurious emissions. The horn antenna vertical alignment was in the center of the DUT and both +45° and -45° polarizations were measured for each setup. The setups are shown in Figure 1 and Figure 2. Two DUTs were measured with each of them having different results.

Figure 1. Measurement setup for the wideband horn antenna measurements.

Similar setup was used for the CLTAs as depicted in Figure 2.

Figure 2. Measurement setup for the CLTA

The results show that when optimizing the setup to minimize noise, there is no difference in the noise level between a wideband horn antenna and the CLTA. If the DUT has no spurs above the noise level, the results are very similar. When there are spurs visible above the noise level, they are both seen by the wideband horn antenna and the CLTA. However, the wideband horn antenna is able to detect the spurs better in the lower frequencies, most likely due to wider beamwidth, and shows higher levels of spurious emissions. This is beneficial in a way that it shows wideband horn antennas have better performance than CLTAs and they can detect low signals more efficiently. However, in the higher frequencies we can see that horn antennas show weaker spurs than the CLTA. In some cases, this might lead to more failures to comply with the current limits than with CLTAs. The level of the spurious emissions is fluctuating in time, which makes it hard to get justified results. We can see that three measurements are not enough to get correct level result and there could be 10dB or more difference between the measurements. More study with better averaging is needed.

Figure 3. DUT1 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup A

Figure 4. DUT1 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup B.

Figure 5. DUT1 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup A.

Figure 6. DUT1 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup B.

Figure 7. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup A.

Figure 9. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup B.

Figure 8. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup A.

Figure 10. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup B.

As it is visible from the results, wideband horn antennas do not give exactly same results as CLTAs. But at this stage it is impossible to say which one gives more correct results. The spurs tend to fluctuate over time and three measurements are not enough to secure enough averaging to find the correct value of the peak of the spur. More measurements are needed to make a justified decision if wideband horn antennas are suitable to use as CLTAs. One thing to note here is that the SH400 receives different frequency bands in different parts of the antenna. The higher frequencies are received in the back part of the antenna, and it might be needed to move the horn antenna forward according to measured frequency band. During the measurements, the front part of the horn antenna was aligned with the radome of the DUT, which might not give correct results. More averaging is needed to ensure the level of spurs is on correct level. Other option would be to use a predefined spurs that could be altered in power level and frequency. If the power level and the frequency of the spurs were known, then the comparison measurement would have a great meaning.

The power level of the spurs in these measurements are fluctuating in time.

More measurements are needed to ensure the usability of wideband horn antennas as CLTAs.

## Co-location and coupling loss between two AAS BS

It has been discussed in previous RAN4 meetings if the current coupling loss assumption of 30dB between two AAS BS for higher FR1 bands is still valid. It is true that for the higher FR1 bands the most typical deployment would be an AAS BS co-locating with another AAS BS. If we think this assumption only from frequency point of view using the same assumption described in TR25.942 [2], then the coupling loss assumption would be larger for higher frequencies. And the coupling loss for passive antennas in the lower bands would be even higher due to different operating frequencies. In this scenario, we should quite easily find correct values for the coupling loss in higher FR1 frequencies. If we consider the scenario in clause 4.9 in TS38.104 [3] with co-location reference antenna being side-by-side with 10cm distance and consider only the frequency effect for the higher FR1 frequency, we could find more realistic values for the coupling loss. But as the deployment for the higher FR1 bands would most likely be an AAS BS co-locating with another AAS BS, then one crucial element is potentially forgotten in the assumptions. That is the effect of beam steering which happens mostly on the horizontal domain. Beam steering in AAS is a crucial technology that enables improved coverage, increased capacity, enhanced signal quality, dynamic resource allocation, and reduced power consumption. These benefits contribute to a more efficient and reliable wireless network. Considering a side-by-side deployment with horizontal beam steering, where the majority of beam steering occurs and the side lobes are the worst, then the coupling loss might be completely different when the main beam is steered in the direction of the victim BS. Simulations were run with different distances between antennas to study the effect the beam steering has to the coupling between antennas.

Beam steering is a technology of paramount importance for AAS.

Some coupling simulations from TX antenna to combined 8-element RX antenna for the different steered beams were done across 7-7.4GHz frequency. Distance D between the middle points of the antenna elements was changed between 2λ and 8λ as shown in Figure 4.

Figure 4. Simplified picture of the simulation setup for coupling loss.

Figure 5 demonstrates the simulation results for the 7-7.4GHz frequency.

Figure 5. TX total coupling to RX element with different beam steering values

Beam steering has a significant effect on the coupling level.

- It is clearly seen that the coupling is much worse with beam steered to -45° than for 0°. The effect is around 15dB. The effect of increasing the distance from 2λ to 8λ is also visible as for beam steered at 0° for 2λ distance the coupling loss is at ~44dB and for 8λ it is ~60dB. The respective values for -45° are ~30dB and ~44dB. The results clearly show the big effect beam steering has to the coupling between antennas. If we re-consider what would be the co-location scenario for two AAS in the higher FR1 frequencies, the whole beam steering concept together with the side-, back- and grating lobes needs to be considered with respect to the deployment scenario.

Co-location scenarios for higher FR1 frequencies need to consider beam steering.

## Typical deployment scenarios

As stated above, the typical deployment case for higher FR1 bands would be AAS together with AAS. In a perfect environment, new site masts would be built for new products for deployment. But as always, the environment where we live is far from perfect and practical restraints occur. In reality, the existing site masts will be used for new deployments, and we should still assume that side-by-side would be quite typical scenario as the space that is available is very limited. Operators will upgrade their equipment and replace older equipment with new ones but still place them in similar fashion to the sites, which reduces the available space in the site masts. RAN4 must ensure the proper functionality for the co-location between different operators with minimal interference. As the majority of beam steering happens in horizontal domain, this would mean that side-by-side deployment would be affected by the beam steering.

Side-by-side can still be expected as a typical scenario due to site constraints.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- The power level of the spurs in these measurements are fluctuating in time.

- More measurements are needed to ensure the usability of wideband horn antennas as CLTAs.

- Beam steering is a technology of paramount importance for AAS.

- Beam steering has a significant effect on the coupling level.

- Side-by-side can still be expected as a typical scenario due to site constraints

- Co-location scenarios for higher FR1 frequencies need to consider beam steering.

# References

- R4-2413509 “WF for [112][305] NR_BS_RF”, RAN4#112, Maastricht, Netherlands 19th – 23rd August, 2024

- TR 25.942 V18.0.0 “Radio Frequency (RF) system scenarios”

- TS38.104 18.7.0 “5G; NR; Base Station (BS) radio transmission and reception”