3GPP TSG RAN WG4 #112bis	R4-2415676

Hefei, China, Oct 14 – 18, 2024

Agenda item:		6.20.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the existing BS RF TX requirements for SBFD operation.

# Impact on BS TX requirements

## ACLR and OBUE

The following was agreed in RAN4#112:

Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

We do not see the necessity of additional ACLR and/or OBUE would be needed from 3GPP to define. The described scenario where two operators SBFD capable gNB’s would coexist, is a new scenario and a rare one. It has not been discussed in the study item phase and should not be included here. There is no need to define additional 3GPP requirements for this special case. Customer specific proprietary requirement can be used in such case where two SBFD systems are coexisting and reasonable co-location is needed.

No need to specify additional ACLR and/or OBUE requirements.

## Joint measurement for normal DL symbols/slots and SBFD symbols/slots

The following was agreed in RAN4#112:

- FFS on the followings: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Averaged EVM for SBFD symbols and non-SBFD symbols should be measured separately to guarantee signal quality during both symbols. The requirements are valid for both SBFD and non-SBFD symbols as has been agreed already before and both need to be tested. For the averaged EVM for SBFD symbols, we agree that calculation is only based on the data samples where DL’s are allocated within the SBFD symbols and data samples where ULs are allocated within SBFD symbols shall be removed from the calculation. Otherwise, there is no way to calculate averaged EVM as it is defined in the specification, and we might end up in a situation the results are not correct.

Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. There were proposals as listed below.

- No need to specify additional ACLR and/or OBUE requirements.

- Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

- Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

# References

- R4-2413515 “Way Forward for [112][308] NR_duplex_evo_BSRF”, RAN4#112, Maastricht, Netherlands, 19th-23rd August, 2024