3GPP TSG RAN WG4 #112bis	R4-2415678

Hefei, China, Oct 14 – 18, 2024

Agenda item:		6.20.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112 [1] regarding evolution of NR duplex operation. There were some agreements made regarding SBFD general aspects and system parameters. Also, the open items are listed in the WF. The most important topic is related to subband configurations and guardbands. This contribution displays Nokia’s view on SBFD general aspects and system parameters.

# Discussion

## CLI handling impact on RF requirement

The following was agreed in RAN4#112:

- Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Regarding the FFS, our preference is to define the BS RF requirements without considering the potential impact of gNB-to-gNB CLI handling mechanisms, i.e., the FFS should not be considered. First, for the sake of progress we should not risk slowing down the work in RAN4 with revisiting some requirements pending on RAN1 progress. Secondly, the CLI handling schemes discussed in RAN1 are mainly applicable to co-channel CLI and their applicability to adjacent channel CLI is questionable. In fact, RAN1 discussions on gNB-to-gNB CLI handling currently focuses on enabling time/frequency coordination and spatial coordination (beam nulling and beam pairing). All the discussed mechanisms require, at least, i) exchange of CLI-RS configurations across gNBs, ii) identification of aggressor gNBs via measurements, and iii) exchange across gNBs of measurement results and additional information to enable the aggressor-victim coordination. All the above rely on signaling between gNBs over the Xn interface (intra-operator signaling). However, there is no standardized inter-operator interface and therefore exchange of information between operators is not possible. Moreover, the discussed CLI handling schemes are only optional features that are not expected to be supported by all the base stations in the field. Lastly, the impact of each CLI handling on the RF requirements is quite challenging to measure due to the fact that it is quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc. The arguments above support the following proposal:

RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

## Band/channel bandwidth support for SBFD

The following was agreed in RAN4#112:

- Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

For the minimum value of the channel bandwidth X, we think that any value below 50 MHz should not be considered. Below this value, the size of the SBFD guardbands may become excessive with respect to the usable data resource blocks, and it may also become challenging to place reference signals and channels such as SSB and CORESET 0 in the DL subband. Even for 50 MHz channel bandwidth, we think that the use of analog filters is not feasible since a 10MHz passband filter which would be immensely difficult to realize in practice. If guardbands can be kept to minimum without using analog filters, 50MHz might be possible to be realized. As companies stated their disappointment in configuring SBFD requirements only for 100MHz, we can include 50MHz case as well as a compromise. Other frequencies between 50MHz and 100MHz can also be considered if there is a clear need from the operators.

SBFD operation below 50MHz BW would result in a very narrow UL subband, and it introduces some challenges in terms of accomodating reference signals and channels such as SSB and CORESET 0 within the DL subband.

Analog filters require large guardband which makes the operation of SBFD in narrow channel bandwidths either unfeasible or not attractive from performance point of view.

Limit X to be no less than 50MHz.

## Transmission bandwidth configuration for SBFD

The following was agreed in RAN4#112:

- Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed - The existing definition of BS transmission bandwidth configuration (for information only): - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS.

As the transmission bandwidth configuration brings new parameters for SBFD-capable BS, it would be beneficial to add a corresponding figure to clarify the used parameters. Figure 1 presents a proposal that can be used.

Figure 1. Proposal for the figure to be used to clarify SBFD specific transmission bandwidth configuration parameters.

## Subband configurations and guardbands

The following was agreed in RAN4#112:

- Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

We agree with option 1 and propose to have main priority in 100MHz bandwidth with roughly 80:20 DL/UL split together with 30kHz SCS as configurated in following way:

- For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5>

- For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5>

We think RAN4 should narrow down a list of possible carrier bandwidths where requirements for SBFD operation should be specified. 100MHz channel bandwidth in FR1 with 80:20 DL/UL split was assumed in the study item phase (with most companies assuming 5RBs for the guardbands when calculating the RSIC and demonstrating feasibility) and to companies’ views, it seems to be the most viable option if SBFD would be deployed. It was not easy to find any conclusions during the study phase and finding RF requirements that all companies can agree will be even more difficult if we don’t have agreed starting point. RAN4 needs to be extremely careful when opening up the flexibility in the sub-band configurations. This could also lead to fully asymmetric DL bands on the upper or lower part of the band. It is the role of RAN4 to make sure that what we specify in the specification would be bulletproof enough to ensure functionality in the field. There is a reason why RAN4 has not adopted all the flexibility that RAN1 specifies. Testing would need to be planned accordingly to make sure that performance is secured. This would be extremely difficult if additional flexibility is introduced. The amount of different test models could potentially explode. All flexible combinations could not be tested and might lead to improper functionality in the field, which would be a disaster. As there were concerns from companies in only defining configurations for 100MHz channel bandwidth, RAN4 could also include 50MHz channel bandwidth, and other bandwidths in the 50-100MHz interval, following similar principles as for the 100MHz case (roughly 80:20 split, 30kHz SCS):

- For FR1 DUD configuration in 50 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <49, 24, 5>

- For FR1 DU or UD configuration in 50 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <104, 24, 5>

In this configuration, it must be noted that there is no immediate value for NRBs in the DL configurations to match the values in Table 5.3.2-1 in of TS38.104. Therefore, values to not go over the maximum transmission bandwidth configuration have been selected. As the SBFD is all about improving the UL performance, maximum NRBs for UL has been selected following roughly the 80:20 split.

For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidth of 100MHz first and 50 MHz secondly.

For FR1 SBFD requirements, from base station perspective, use the following configuration:

  - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <106, 51, 5>

  - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <217, 51, 5>

On FR2-1, possible bandwidths are 50, 100, 200, and 400 MHz. We have slight preference for focusing on 100 MHz and 200MHz channels with 120 kHz SCS. For this case, the SBFD subband configuration can follow the assumptions agreed by RAN1 in the study item phase, that is < ND, NU, NG > = <25, 14, 1> and < ND, NU, NG > = <52, 26, 1>, respectively

For FR2-1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point:

  - For FR2-1 DUD configuration in 100 MHz channel and 120 kHz SCS: < ND, NU, NG > = <25, 14, 1>

  - For FR2-1 DUD configuration in 200 MHz channel and 120 kHz SCS: < ND, NU, NG > = <52, 26, 1>

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed.

- SBFD operation below 50MHz BW would result in a very narrow UL subband, and it introduces some challenges in terms of accomodating reference signals and channels such as SSB and CORESET 0 within the DL subband.

Analog filters require large guardband which makes the operation of SBFD in narrow channel bandwidths either unfeasible or not attractive from performance point of view.

- RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Limit X to be no less than 50MHz.

For FR1, RAN4 to prioritize SBFD requirements for BS carrier bandwidth of 100MHz first and 50 MHz secondly.

For FR1 SBFD requirements, from base station perspective, use the following configuration:

  - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <106, 51, 5>

  - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < ND, NU, NG > = <217, 51, 5>

For FR2-1 SBFD requirements, from base station perspective, the following assumptions for guardbands and UL and DL subbands can be used as starting point:

  - For FR2-1 DUD configuration in 100 MHz channel and 120 kHz SCS: < ND, NU, NG > = <25, 14, 1>

  - For FR2-1 DUD configuration in 200 MHz channel and 120 kHz SCS: < ND, NU, NG > = <52, 26, 1>

# References

- R4-2413514 “Way Forward for [112][307] NR_duplex_evo”, RAN4#112, Maastricht, Netherlands, 19th-23rd August, 2024