3GPP TSG-RAN WG4 Meeting #112bis 			         R4-2415679
Hefei, Anhui, China, 14th – 18th October, 2024
 
Agenda 		Item:6.9.4
Source:			SKY Perfect JSAT [TBD]
Title:			Ku-band VSAT UE RF requirement analysis

Document for:	Discussion and Information

Introduction

In this contribution, the technical information for Ku-band VSAT UE is described, as recommendations or regulations by the ITU and ETSI. We introduce the information and propose requirements for Ku-band VSAT UE. The information in this contribution is correct at the time of submission ([1] through [8]).

Analysis and Discussions

Antenna Sidelobe Pattern

The ITU recommends requirements for Ku-band VSAT transmit antenna sidelobe patterns as a non-mandatory nature and are described in ITU-R S.465-6 and ITU-R S.580-6. ITU-R S.580-6 is only for GSO (Geo-Stationary Satellite Orbit).

Table 2.1-1 ITU-R S.465-6

| θ value | Antenna gain relative to an isotropic antenna (dBi) |
| --- | --- |
| θmin ≤ θ ≤ 48° | 32 – 25log(θ) |
| 48° < θ ≤ 180° | -10 |
| θmin : 1 or 100λ/D, whichever is the greater [degrees], for D/ ≥ 50, θmin : 2 or 114 (D/)–1.09, whichever is the greater [degrees], for D/ < 50 λ : Wavelngth [m] D : Antenna Diameter [m] |  |

Table 2.1-2 ITU-R S.580-6

| θ value | Antenna gain relative to an isotropic antenna (dBi) |
| --- | --- |
| θmin ≤ θ ≤ 20° | 29 – 25log(θ) |
| 20° < θ ≤ 180° | Refer to ITU-R S.465 |
| θmin : 1 or 100λ/D, whichever is the greater [degrees] λ : Wavelength [m] D : Antenna Diameter [m] (Note) at least 90% of the side-lobe peaks does not exceed |  |

The antenna sidelobe performance issued by ITU recommendations are “recommendation” only and are not as a mandatory requirement. In addition, it is hard to meet this requirement, especially for smaller or flat panel antennas in the future.

Proposal 1 : The antenna sidelobe pattern should NOT be specified as a mandatory limit for Ku-band VSAT UE.

Adjacent Satellite Interference

ASI (Adjacent Satellite Interference) poses a significant challenge in the operation of VSAT systems since there is a risk that these signals may interfere with neighbouring satellite systems operating in same frequency bands.

Two types of ASI can occur. Downlink ASI, in which the ground receiving antenna beam width is wide or side-lobe pattern is high and receives interfering signal from adjacent satellites, and uplink ASI, in which adjacent satellites receive uplink interfering signals from ground antennas which have wider beam width or high side-lobe pattern, or improperly pointed, or both.

To mitigate uplink and downlink ASI in VSAT systems, coordination efforts between satellite operators and satellite regulators are essential to ensure compliance with spectrum allocation guidelines and to facilitate the resolution of interference issues that may arise. These efforts include careful terminal selection, link design, grooming and coordination of capacity, and setting and following appropriate operating constraints.

EMF Regulation

EMF (Electromagnetic Field) regulation is essential for ensuring public health and safety in the deployment of telecommunications infrastructure and electronic devices. These regulations typically set limits on the exposure of individuals to the EMF generated by various sources. The guidelines for EMF exposure are based on scientific research conducted by organizations such as the World Health Organization and the International Commission on Non-Ionizing Radiation Protection (ICNIRP). These organizations continuously evaluate the potential health effects of electromagnetic fields and establish safety thresholds to protect against any adverse impacts.

It is ultimately the responsibility of each nation or regional authority to enact and enforce regulations tailored to their circumstances which are appropriate and effective within their jurisdictions. As a result, EMF regulations may vary from one country to another and even within different regions of the same country, reflecting diverse policy priorities and interpretations of scientific evidence regarding EMF exposure.

Off-axis EIRP Density Limits for Ku-Band

The off-axis EIRP Density is the very important requirement, and the maximum permissible level is specified in ETSI standards.

Fixed VSAT

Maximum Permissible level of off-axis EIRP Density towards GSO from a Very Small Aperture Terminal (VSAT) is described in ETSI EN 301 428(Ver 2.1.2).

Table 2.4-1 Off-axis EIRP Density Limits for VSAT (ETSI EN 301 428)

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2° ≤ θ ≤ 7° | 33 – 25log(θ) | 40 |
| 7° < θ ≤ 9.2° | 12 | 40 |
| 9.2° < θ ≤ 48° | 36 – 25log(θ) | 40 |
| 48° < θ ≤ 70° | -6 | 40 |
| 70° < θ ≤ 180° | 4 | 40 |
| θ : Off-axis Angle [degrees] |  |  |

Mobile VSAT

Maximum Permissible level of off-axis EIRP Density for transmitting in Ku-bands is described in ETSI EN 302 340 (Ver 2.1.1) and ETSI EN 302 977 (Ver 2.1.1) for Earth Stations on board Vessels (ESVs) and Vehicle-Mounted Earth Stations (VMES) respectively.

Table 2.4-2 Off-axis EIRP Density Limits for Mobile VSAT

(ETSI EN 302 340, ETSI EN 302 977)

| θ value | Maximum Off-axis EIRP (dBW) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 2° ≤ θ ≤ 7° | 33 – 25log(θ) | 40 |
| 7° < θ ≤ 9.2° | 12 | 40 |
| 9.2° < θ ≤ 48° | 36 – 25log(θ) | 40 |
| 48° < θ ≤ 70° | -6 | 40 |
| 70° < θ ≤ 180° | 4 | 40 |
| θ : Off-axis Angle [degrees] |  |  |

Proposal 2 : The off-axis EIRP requirements standardized by the ETSI should be specified as mandatory for Ku-Band VSAT UE.

VSAT UE Maximum Output Power Limits for Ka-Band

In 3GPP TS 38.101-5 the following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

The maximum output power values for Fixed VSAT EIRP are found in Table below.

Table 2.5-1 UE maximum output power limits for Fixed VSAT

(3GPP TS 38.101-5, Table 9.2.1.1-2)

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 1 | 35 | 76.2 |
|  | 2, 3 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

The maximum output power values for Mobile VSAT EIRP are found in Table below.

Table 2.5-2 Maximum output power limits for Mobile VSAT

(3GPP TS 38.101-5, Table 9.2.1.2-2)

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.2 |
|  | 5 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

Maximum Output Power Limits for VSAT UE for Ku-band

The 3 main EIRP power limitations are as follows:

- The maximum permissible level of off-axis EIRP Density as defined by the ITU.

- Coordination for adjacent satellite system and other networks needs to be taken into consideration. ASI is a known issue between satellite operators and is mitigated by careful terminal selection, link design, grooming and coordination of capacity, and setting and following appropriate operating constraints.

- EMF limitations are established based on recommendations from public health organizations and government regulators such as the World Health Organization and the International Commission on Non-Ionizing Radiation Protection (ICNIRP). These limitations vary by country and region.

Assuming the VSAT complies with these 3 aforementioned considerations, TRPMAX and EIRPmax can also vary due to factors such as carrier bandwidth (or a number of sub-carriers), power amplifier size, antenna gain (antenna size), satellite in use, frequency in use and area in use.

Limiting maximum TRP and EIRP for VSAT UE on-axis direction towards SAN impacts satellite link characteristics (throughput, SNR, link margin etc.).

Meanwhile, off-axis EIRP density specification is very important for minimizing the interference to co-existing networks. Therefore, the TRP and EIRP for VSAT UE should NOT be limited and off-axis EIRP density should be specified.

Proposal 3 : The maximum TRP and EIRP specified for the Ka-band in 3GPP TS 38.101-5, Section 9.2.1, Table 9.2.1.1-2 and Table 9.2.1.2-2 should NOT be applied to Ku-Band VSAT UE and no such requirements should be specified for the Ku-band VSAT UE.

Conclusion

For the Ku-Band VSAT UE, we propose to introduce the related recommendations and regulations published by the ITU for Antenna sidelobe pattern and maximum off-axis EIRP density, and not to specify maximum EIRP and Power limit.

Proposal 1.  The antenna sidelobe pattern should NOT be specified as a mandatory limit for Ku-band VSAT UE.

Proposal 2.  The off-axis EIRP density requirements standardized by ETSI should be specified for Ku-Band VSAT UE.

Proposal 3.  The maximum TRP and EIRP specified for the Ka-band in 3GPP TS 38.101-5, Section 9.2.1, Table 9.2.1.1-2 and Table 9.2.1.2-2 should NOT be applied to Ku-band VSAT UE and no such requirements should be specified for the Ku-band VSAT UE.

References

[1] ITU Recommendation S.580-6, Radiation diagrams for use as design objectives for antennas of earth stations operating with geostationary satellites

[2] ITU Recommendation S.465-6, Reference radiation pattern for earth station antennas in the fixed-satellite service for use in coordination and interference assessment in the frequency range from 2 to 31 GHz

[3] ETSI EN 301 428 (Ver 2.1.2), Satellite Earth Stations and Systems (SES); Harmonised Standard for Very Small Aperture Terminal (VSAT)

[4] ETSI EN 302 340 (Ver 2.1.1), Satellite Earth Stations and Systems (SES); Harmonised Standard for satellite Earth Stations on board Vessels (ESVs)

[5] ETSI EN 302 977 (Ver 2.1.1), Satellite Earth Stations and Systems (SES); Harmonised Standard for Vehicle-Mounted Earth Stations (VMES)

[6] Adjacent Satellite Interference in Mobile/VSAT Environments Intelsat White paper

[7] https://www.who.int/

[8] 3GPP TS 38.101-5 Satellite access Radio Frequency (RF) and performance requirements