3GPP TSG-RAN WG4 Meeting #112bis	R4-2415681

Hefei, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Discussion and simulation results on MIMO layer evaluation for 6Rx UE

Agenda item:	6.1.1.3.2

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements, especially for handheld UEs. In the RAN#104 meeting, the WID has been updated [2]. The latest objectives of 6Rx are as below:

Objective of Core part WI 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. Objective of Performance part WI 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - Support 4 MIMO layers at least, and follow the conclusion in core part for whether to support 6 MIMO layers - Specify the SDR requirements to support 6Rx

From the objective descriptions of 6Rx, 4 MIMO layers should be supported at least, but 6 MIMO layers can be supported once the gain and feasibility are verified. In last RAN4#112 meeting, initial link level simulation assumptions alignment for the 6 MIMO layers performance gain validation of 6Rx receiver has been achieved [3], although many options for some parameters proposed. In last RAN4#112 meeting, our initial simulation results [5] which based on XP low/medium (even the correlation factor downgrades from 0.6 to 0.1) show that the performance of 6 MIMO layers case is hard to outperform 4 MIMO layers. In this contribution, we provide our further analysis and simulation results for 6Rx receiver based on the agreed initial simulation assumptions and real product correlation matrix.

- Discussion and simulation results

Therotially, rank is used to describe the characteristics of the channel quantitatively for spatial mulitiplexing, which is the number of uncorrelation/independent virtual channels. For the scenarios with the same number of Rx antennas, it’s no doubt that the performance of 6 MIMO layers could outperform 4 MIMO layers if the channel uncorrelation promised. However, once correlation between antennas are introduced, the performance of 6 MIMO layers can decrease compared with 4 MIMO layers. The correlation not only come from UE receive antennas but also may come from BS transmit antennas and wireless channels. Therefore, both UE antenna correlation and BS antenna correlation matrix should be considered for the 6 MIMO layers performance evaluation.

  - UE antenna correlation matrix

UE antenna correlation matrix should be highly related with the product design, different product design may correspond to different antenna correlation matrix. In our view, this is the reason why RAN4 has recieved some contributions focusing on foldable devices which may understate the antenna correlation with a larger form factor than normal handheld UE. However, on top of the larger space of the foldable UEs, it also should be noted that there would be a lot of other restrictions to support the advanced form factor in our understanding. Since 6-layer MIMO performance evaluation is expected to be based on realistic antenna correlation as below Way Forward [3], and it could be a common understanding of RAN4 that 6-layer support would be not easy for normal handheld UEs based on the latest discussion, the simulation results in this contribution are based on ECC antenna correlation matrix A and B for foldable devices. Even for the the foldable devices on the market, according to our experiences, it’s not easy to make RF path between two sides due to the hinge and both side full screen for example. Hence we take two antenna deployments as reference in Figure 2-1, which are surely practical layouts of 6Rx for foldable UEs.

Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios

Observation 1: It could be a common understanding of RAN4 that 6-layer support would be not easy for normal handheld UEs based on the latest discussion.

Observation 2: For realistic antenna correlation factors of the foldable devices, it also should be noted that there would be a lot of other restrictions to support the advanced form factor on top of the larger space of the foldable UEs.

UE antenna correlation matrix A

UE measured ECC (Envelop Correlation Coefficient) matrix A for N41 based on antenna deployment in Figure 2-1 (a).

1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00

UE antenna correlation matrix B

UE measured ECC matrix B for N77 based on antenna deployment in Figure 2-1 (b).

1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00

(a)                                           (b)

Figure 2-1. Antenna deployment examples for foldable UE

  - BS antenna correlation matrix

Considering BS antenna correlation coefficient may also impact the 6-layer MIMO performance, the simualtion assumption of BS antenna correlation matrix should also be aligned. Ideal BS antenna correlation matrix (no correlation between different BS transmit antennas) could be used for simualtion results alignment. In addition, BS antenna correlation matrix with some correlations between different BS transmit antennas could be defined since it’s more realistic.

BS antenna correlation matrix A

Identity matrix assuming no correlation between different BS transmit antennas.

1001000000001001000000000000000000000000000000001001000000001001

BS antenna correlation matrix B

Refer to the channel spatial correlation matrix defined in 38.101-4 B.2.3.1, the BS antenna correlation matrix could be described as below.

1α149α449α949α1649α2549α3649αα149*1α149α449α949α1649α2549α3649α449*α149*1α149α449α949α1649α2549α949*α449*α149*1α149α449α949α1649α1649*α949*α449*α149*1α149α449α949α2549*α1649*α949*α449*α149*1α149α449α3649*α2549*α1649*α949*α449*α149*1α149α*α3649*α2549*α1649*α949*α449*α149*1

RAN4 defines α as 0.3 for ULA Medium correlation and 0.9 for ULA high correlation. Here we set α as 0.05 in order to introduce a really low correlation coefficient between BS transmit antennas.

  - Simulation assumptions

As there are many options for different parameters in the feasibility study link level simulation assumption [3], it’s necessary to narrow down some options. Our simulations are based on Table 2-1.

Table 2-1 test parameters for 6Rx

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30kHz |
| Antenna configuration |  | Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 2: TDLA30-10 |
| Rank |  | Fixed, rank 4 and 6 |
| MCS |  | Option 2: MCS13, MCS17 from MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | Specific to each Reference channel |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Position of the first DMRS for PDSCH mapping type A | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | Antenna ports indexes | {1000-1003} for Rank 4 tests {1000-1005} for Rank 6 tests |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for rank <= 4 2 for rank > 4 1 for case “2plus0” of rank > 4 |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| UE correlation matrix |  | Matrix A: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Matrix B: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 |
| BS correlation matrix |  | Matrix A: 1001000000001001000000000000000000000000000000001001000000001001 Matrix B: α as 0.05 1α149α449α949α1649α2549α3649αα149*1α149α449α949α1649α2549α3649α449*α149*1α149α449α949α1649α2549α949*α449*α149*1α149α449α949α1649α1649*α949*α449*α149*1α149α449α949α2549*α1649*α949*α449*α149*1α149α449α3649*α2549*α1649*α949*α449*α149*1α149α*α3649*α2549*α1649*α949*α449*α149*1 |

  - Simulation results

Based on the test parameters in Table 2-1, our simulation results are summarized in Figure 2-1 - Figure 2-2 for MCS13 and MCS17 separately, in which the case 8T6R-6L-2plus0 use 2+0 DMRS symbols.

Figure 2-1. Performance comparison for 6Rx with MCS13

Figure 2-2. Performance comparison for 6Rx with MCS17

From our simulation results for both MCS13 and MCS17, for 6Rx with 6 MIMO layers cases, performance gain could be observed only at higher SNR ranges compared with 6Rx with 4 MIMO layers cases, for ideal channel conditions with no BS antenna correlation cases (BS correlation matrix A). There are rather performance loss for cases with BS antenna correlation introduced (BS correlation matrix B). Even the performance for 6 MIMO layers case with 2+0 DMRS symbols configured is improved compared with the case of 2+2 DMRS symbols configured, it’s still worse than 4 MIMO layers case.

Observations from our simulation results could be summarized as:

Observation 3: For 6Rx antenna cases, 6 MIMO layers cases could outperform 4 MIMO layers cases only at higher SNR value range using ideal channel conditions with no BS antenna correlation cases.

Observation 4: For 6Rx antenna cases with some BS antenna correlation introduced with both MCS13 and MCS17, the performance of 6 MIMO layers cases is even worse than 4 MIMO layers cases.

Observation 5: For 6Rx antenna cases with some BS antenna correlation introduced with both MCS13 and MCS17, the performance of 6 MIMO layers with 2+0 DMRS symbols cases is still worse than 4 MIMO layers cases.

From above observations, it seems it’s hard to get the benefit using 6Rx with 6 MIMO layers since the antenna correlation introduced in BS side (BS correlation matrix B) and UE side (UE correlation matrix A and B) correlation deployment.

Simulation results analysis

According to our simulation results and observations above, 6 MIMO layers could only benefit at higher SNR value range using ideal channel conditions with no BS antenna correlation cases. Even if we assume 0.05 BS antenna correlation could be achieved instead of 0.3 for BS medium correlation, the performance of 6 MIMO layers case is still worse than 4 MIMO layers case. In addition, for the practical deployment, we can easily assume that it is hard to promise 0.05 BS antenna correlation factor. As a consequence of this performance evaluation, for handheld UE with 6Rx receiver antennas, we propose not to introduce requirements for 6 MIMO layers.

Proposal 1: For handheld UEs with 6Rx antennas, considering correlation will be introduced in realistic deployment, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic antenna correlation matrixes.

- Conclusion

In this contribution, we provide simulation assumptions, simulation results analysis, and views on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

Observation 1: It could be a common understanding of RAN4 that 6-layer support would be not easy for normal handheld UEs based on the latest discussion.

Observation 2: For realistic antenna correlation factors of the foldable devices, it also should be noted that there would be a lot of other restrictions to support the advanced form factor on top of the larger space of the foldable UEs.

Observation 3: For 6Rx antenna cases, 6 MIMO layers cases could outperform 4 MIMO layers cases only at higher SNR value range using ideal channel conditions with no BS antenna correlation cases.

Observation 4: For 6Rx antenna cases with some BS antenna correlation introduced with both MCS13 and MCS17, the performance of 6 MIMO layers cases is even worse than 4 MIMO layers cases.

Observation 5: For 6Rx antenna cases with some BS antenna correlation introduced with both MCS13 and MCS17, the performance of 6 MIMO layers with 2+0 DMRS symbols cases is still worse than 4 MIMO layers cases.

Proposal 1: For handheld UEs with 6Rx antennas, considering correlation will be introduced in realistic deployment, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic antenna correlation matrixes..

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104 meeting, Huawei

[3] R4-2414285, WF on 6Rx UE requirements, RAN4#112 meeting, AT&T

[4] R4-2409459, Views on MIMO layer for 6Rx UE, RAN4#111 meeting, Samsung

[5] R4-2412877, Discussion and initial simulation results on MIMO layer evaluation for 6Rx UE, RAN4#112 meeting, Samsung