3GPP TSG-RAN WG4 Meeting #112bis	R4-2415686

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.1.1.3.4

Source:	Intel Corporation

Title:	Views on SRS insertion loss compensation and reporting enhancements

Document for:	Discussion

# Introduction

The Rel-19 WID on UE RF enhancements for NR FR1/FR2 and EN-DC Phase 4 was approved in RAN#103 with the following objectives related to 6Rx for handheld and FWA UEs [1]:

### 4.1 Objective of Core part WI The objectives of core part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: … 6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. …

In this contribution we provide further views on the topic insertion loss imbalance across SRS ports.

# Discussion

## Background

The topics of SRS insertion loss imbalance was discussed in RAN4 #110bis and #111. The RAN4 #110bis agreements are captured in WF [2]:

## Sub-topic 4-1: General considerations for SRS IL imbalance issue Way forward: Further discuss the topic of SRS IL imbalance issues with the following proposals used as a starting point. Whether a solution is agreeable, depends on further discussion of the following candidate solutions and/or identified issues. - Proposal 1: Study the issues of insertion loss imbalance across SRS ports. - Proposal 2: Further discuss SRS IL for 6RX type of devices taking into consideration: - SRS insertion loss requirements for 6RX UEs - SRS IL impact on performance - SRS insertion loss compensation - UE assistance on SRS insertion loss (power imbalance) - Others - Proposal 3: Companies should, independent of their view, provide constructive technical inputs on SRS TX power imbalance topic. ## Sub-topic 4-2: Candidate solutions for the SRS IL imbalance issue Way forward: Further discuss the following options considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation. - Option 1: Specify UE behaviour and requirements for scenarios, when UE has sufficient power to compensate the power imbalance (Case 2) and require UE to perform SRS IL compensation up to the maximum power capabilities. - Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX_L,f,c. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed. Per discussion during RAN4 meetings there seem to be no clear understanding whether all existing UEs do perform the SRS IL compensation under these conditions. So, it is recommended to consider specific requirements / conformance requirements in future to guarantee proper UE implementations. - Case 2 (near max Tx power): In this scenario the actual required SRS transmission power is higher than PCMAX_L,f,c, but is still below the max transmission power PCMAX_H,f,c. Based on the interpretation of TS 38.101-1 above, in this case the UE behaviour is undefined and UE may or may not perform SRS IL compensation. - Case 3 (max Tx power): In this scenario the actual required SRS transmission power is equal to PCMAX_H,f,c. (i.e. hypothetical scenario). In this case UE is not capable to perform any SRS IL compensation. - Option 2: At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively. - Option 3: Not specifying reporting of SRS IL offsets due to IL imbalance. - Option 4: IL imbalance reporting mechanism for SRS AS including both static reporting and dynamic reporting. - Static reporting is up to UE implementation, and UE needs to indicate the power compensation behaviour to NW if UE reports statically. - Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered. - Option 5: IL imbalance reporting mechanism for SRS AS including both the configured maximum output power per SRS resource and the power headroom per SRS resource. - The PH used for the SRS resource can be a Type 3 but used for a new MAC-CE “SRS resource power report” and can be used also for a carrier configured for PUSCH transmission. - Other options are not precluded. Consider some clarifications according to the RAN1 and RAN4 specs. - RAN1 specification on power control doesn’t require or imply that UE shall apply the same PCMAX for all AS-SRS transmission occasions - Under the minimum requirement ∆TRxSRS defined in RAN4 specification, it is up to UE implementation whether to apply exact value for corresponding AS-SRS transmission occasions.

The RAN4 #111 agreements are captured in WF [3]:

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

The RAN4 #112 agreements are captured in WF [4]:

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

## Legacy UE behaviour

Based on Rel-18 discussions in RAN1/4 there was no clear understanding among companies on the anticipated UE behavior in terms of SRS IL compensation during SRS transmissions with the following options mentioned:

- Option 1: UE does not make Tx power imbalance compensation.

- Option 2: UE performs Tx power imbalance compensation up to its maximum capabilities in non-power limited conditions (i.e., applies power compensation in case there is sufficient power to do so). For example, in case of 20dBm configured Tx power and 8dB SRS IL, the UE should be capable to compensate 3dB loss in case of 23dBm maximum output power.

- Option 3: Keep up to UE implementation and UE can choose whether and when to apply compensation.

Observation #1:	In Rel-18 discussion there was no consensus among the companies on the legacy UE behavior for SRS IL handling.

The legacy UE behavior can be understood based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

The SRS transmission power in TS 38.213 is defined in a way that unless the SRS transmission power exceeds the configured maximum output power PCMAX,f,c limit (which is defined in TS 38.101-1), the actual SRS transmission power depends on L1 power control parameters only such as p0 for active UL BW, SRS bandwidth, pathloss and others. It can be observed that the SRS insertion loss is not taken into consideration, and UE is supposed to compensate any SRS insertion loss as long as the total power is below PCMAX,f,c.

The UE configured maximum output power parameter PCMAX,f,c is defined in TS 38.101-1. The effect of SRS insertion loss is accounted in the PCMAX via the parameter ∆TRxSRS, which reflects the worst-case implementation in terms of SRS insertion loss (i.e., upper bound). It can be observed that PCMAX is not a pre-defined fixed value and represents a certain range of values, which gives flexibility to possible UE implementations.

PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

Observation #2:	The legacy UE behavior is based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

Let’s consider a simplified example of SRS power control equations under an assumption that all factors reducing PCMAX except for SRS IL are equal to 0 dB. The equations can be represented in the following form:

PSRS = min(PCMAX, PSRS,PC) PSRS,PC = PCMAX_L ≤ PCMAX ≤ PCMAX_H PCMAX_L = PPowerClass – ∆TRxSRS PCMAX_H = PPowerClass

In the Figure 1 below we provide an illustration of the SRS transmission power levels and effective SRS power imbalance between ports as a function of PSRS,PC under different assumptions of PCMAX selection (equal to either PCMAX,H or PCMAX,L) based on the equations above for the following cases

- Case A: ∆TRxSRS = 7dB, ∆TRxSRS,UE  =  7dB (UE-specific value is same as the spec value)

- Case B: ∆TRxSRS = 7dB, ∆TRxSRS,UE  =  3dB (UE-specific value is better than the spec value)

The following curves are illustrated in the figures:

- “P0” – SRS transmission power for the main SRS AP without SRS IL

- “P1” – SRS transmission power for the diversity SRS AP with SRS IL

- “P1,EFF” – SRS transmission power for the diversity SRS AP before applying SRS IL

- “P0-P1” – SRS transmission power imbalance between SRS AP with and without SRS IL (P0-P1)

Figure 1. SRS transmission power levels and effective SRS power imbalance

Based on the analysis above we make the following observations:

- Full SRS IL compensation is performed at least for power levels PSRS,PC ≤ PCMAX,L

- The existing definition of PCMAX implies that UE performs at least partial SRS IL compensation in the power range of PCMAX,L < PSRS,PC < PCMAX,H

- For Case A when the effective SRS IL is same as the min requirement (∆TRxSRS = ∆TRxSRS,UE) the effective SRS AP power imbalance does not depend on the selection of PCMAX level

- For Case B when the effective SRS IL is better than the min requirement (∆TRxSRS,UE <∆ TRxSRS,UE)

  - The effective SRS AP power imbalance may depend on the selection of PCMAX level

  - UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities in case PCMAX=PCMAX,L

To summarize, the UE behavior may depend on the specific SRS transmission power:

- Case 1 (non near max Tx power): In this scenario the SRS transmission power (PSRS) is below PCMAX,L. Based on TS 38.213 the UE is required to compensate any insertion loss and no further changes in the specification are needed.

- Case 2 (near max Tx power): In this scenario the actual required SRS transmission power is higher than PCMAX_L,f,c, but is still below the max transmission power PCMAX_H,f,c (PCMAX,L < PSRS < PCMAX,H). In this case the UE behavior is not fully defined and UE may or may not necessarily perform SRS IL compensation depending on specific SRS IL for this UE and chosen PCMAX,f,c value.

- Case 3 (max Tx power): In this scenario the actual required SRS transmission power is equal to PCMAX_H,f,c. In this case UE is not capable to perform any SRS IL compensation.

Observation #3:	For Case 1 (non near max Tx power) scenario the UE is supposed to always compensate any SRS insertion in SRS transmissions based on TS 38.213.

Observation #4:	For Case 2 (near max Tx power) scenario the UE behavior in terms of SRS IL is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities based on TS 38.101-1

## SRS insertion loss impact on performance

The SRS insertion loss results in power imbalance of SRS signals transmitted from different TX chains. The SRS antenna switching is used at the gNB side to estimate the propagation channel, which is further used for DL CSI calculation in TDD systems. The gNB uses the respective SRS estimates to derive the DL channel estimates, and there may be a mismatch between DL channel estimates using SRS for APs with insertion loss (even in case of perfect channel estimation) and the actual DL channels corresponding to different UE antennas (as the power imbalance at the TX side is typically >> than the imbalance at the RX side), which leads to an incorrect DL CSI estimation and as result in the degradation of overall DL system performance.

During Rel-18 timeframe RAN1 performed an in-depth analysis of SRS IL impact on the DL performance. The latest RAN1 discussion took place in RAN1 #114 (August 2023) and the summary document [4] mentions that “SRS antenna switching is the key feature to acquire DL CSI for reciprocity based TDD. However, due to the Tx switching at UE side, the insertion loss (IL) of diversity branch(es) can differ from that of main branch as identified by RAN4. Aforementioned IL imbalance is called SRS IL imbalance, which can be led by RF switch and trace loss difference.” During RAN1 discussion many companies have shown a noticeable performance loss in case SRS IL is not compensated at the UE and/or gNB sides - “It can be observed from most companies’ result that the existence of SRS IL imbalance will cause non-negligible performance degradation, especially under MU-MIMO and/or high-rank scenario.”.

For Rel-19 work we do not see a strong motivation to repeat the analysis of the impact on the performance, since the extensive studies were already conducted in Rel-18. Instead, we recommend focusing on the identification of solutions to resolve the technical problem and introduce mechanisms to avoid/minimize the impact of SRS IL on the performance.

Observation #5:	Rel-18 RAN1 results have indicated that existence of SRS IL imbalance will cause non-negligible performance degradation.

## SRS insertion loss compensation solutions with minimum impact

Based on the analysis of legacy UE behavior in clause 2.2, we see at least two problems:

- For scenarios when UE operates in the non near max Tx power scenario the UE is supposed to always compensate any SRS insertion in SRS transmissions, however given diverse comments during prior 3GPP discussions on UE implementations, it may be reasonable to introduce specific RAN4 requirements to ensure that UE behavior is properly tested.

- For the case when the effective SRS IL is better than the min requirement (∆TRxSRS,UE < ∆TRxSRS,UE) UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities conditions. UE may still have sufficient power headroom to partially compensate SRS IL, which can be helpful to improve the SRS quality. Furthermore, gNB does not have a clear understanding on the UE behavior and cannot perform proper compensation of UL/DL mismatch at it is side. Therefore, it is recommended to clearly specify UE behavior of partial SRS IL compensation.

Proposal #1:	Specify requirements for Case 1 scenarios with PSRS,PC ≤ PCMAX,L   to ensure that legacy UE performs SRS IL compensation.

Proposal #2:	Specify solutions, UE behavior and requirements for scenarios, when UE has sufficient power to compensate the power imbalance and require UE to perform SRS IL compensation up to the maximum power capabilities.

The requirements in TS 38.101-1 on SRS IL represent the worst-case conditions in terms of UE implementation. Commercial UE implementations may have better SRS IL than the ∆TRxSRS in the specification. The information on the actual/instantaneous UE SRS IL (∆TRxSRS,UE) can be helpful for gNB to: 1) correctly derive DL channel estimates based on SRS measurements (e.g., for the case when UE does not perform compensation), and 2) more precisely identify specific power conditions, where UE operates. It is recommended to further discuss the mechanisms for UE assistance mechanisms to inform network on the actual SRS transmission power imbalance among TX chains. At least semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE) is suggested.

As explained in 2.2 the when the effective SRS IL is better than the min requirement (∆TRxSRS,UE < ∆TRxSRS,UE) UE may underutilize PA capabilities and not perform SRS IL compensation up to its maximum capabilities in case PCMAX < PCMAX,H. In order to make sure that UE performs more optimal SRS IL compensation, it is possible to adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL rather that the common worst-case value as follows

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

Proposal #3:	Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE).

Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

# Conclusion

In this paper we provide views on the insertion loss imbalance across SRS ports. In summary, the following observations and proposals are made:

Observation #1:	In Rel-18 discussion there was no consensus among the companies on the legacy UE behavior for SRS IL handling.

Observation #2:	The legacy UE behavior is based on current specifications of the SRS power control mechanism in TS 38.213 and based on configured maximum output Tx power in TS 38.101-1.

Observation #3:	For Case 1 (non near max Tx power) scenario the UE is supposed to always compensate any SRS insertion in SRS transmissions based on TS 38.213.

Observation #4:	For Case 2 (near max Tx power) scenario the UE behavior in terms of SRS IL is not fully specified and UE may or may not necessarily be required to perform SRS IL compensation up to its maximum power capabilities based on TS 38.101-1

Observation #5:	Rel-18 RAN1 results have indicated that existence of SRS IL imbalance will cause non-negligible performance degradation.

Proposal #1:	Specify requirements for Case 1 scenarios with PSRS,PC ≤ PCMAX,L   to ensure that legacy UE performs SRS IL compensation.

Proposal #2:	Specify solutions, UE behavior and requirements for scenarios, when UE has sufficient power to compensate the power imbalance and require UE to perform SRS IL compensation up to the maximum power capabilities.

Proposal #3:	Introduce semi-static UE assistance or capability signalling on the information on the UE SRS IL (∆TRxSRS,UE).

Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

# References

- RP-240828 “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN#103, March 2024

- R4-2406585 “WF on requirements for 6Rx”, AT&T, RAN4#110bis, April 2024

- R4-2410751 “WF on requirements for 6Rx”, AT&T, RAN4#111, May 2024

- R4-2414285 “WF on requirements for 6Rx”, AT&T, RAN4#112, August 2024

- R4-2317621 WF on UE RF requirements for 8Rx, NTT DOCOMO, RAN4 #108bis, October 2023

- R1-2308458 FL summary on UE SRS IL imbalance issue Moderator (Huawei)