3GPP TSG-RAN WG4 Meeting #112bis	R4-2415689

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.17.2.2

Source:	Intel Corporation

Title:	Views on logistics of AI/ML model and dataset sharing for CSI compression use case

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1], and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting with the latest version available in [3].

In Rel-19 RAN4 conducted the work on the studies of testability and interoperability of different AI-ML use cases. The studies for the CSI compression use case test feasibility require exchange of dataset and AI-ML models among the companies. To facilitate further RAN4 analysis several logistical questions on AI-ML model and dataset sharing framework need to be decided. The respective discussions took place in the recent RAN4 and RANP meetings. In accordance with RAN#105 decisions RAN4 was tasked to further look into the details of the technical aspects of data sharing and provide respective information to MCC to facilitate establishment of the common 3GPP infrastructure [4]:

- RAN Plenary agrees to formally start implementing the infrastructure/platform needed by RAN4 for data sharing - MCC to work out the details of the data sharing platform with input from RAN4 experts - Discussion on the platform setup with MCC to be triggered as soon as possible - Inputs to MCC based on previous RAN4 discussions (data size, etc) - RAN4 to discuss the technical aspects (data formats, model formats etc. for purposes of studying the two-sided CSI compression) in the next RAN4 meeting

In this contribution we provide further views on the details of AI-ML models and dataset sharing.

# Discussion

The details of the AI-ML models and dataset sharing were discussed as a part of post RAN4 #112 email discussion and the following conclusions were made [2]:

2.1 Data size 2.1.1 Training data: - Companies agree that the current assumptions and estimates for the CSI compression use case are reasonable (Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance) - These assumptions are specific to the CSI compression use case in RAN4 and for the current simulation assumptions. - Larger datasets could be needed/used in future studies depending on the use case. It would be good to maintain some flexibility - Importance of ensuring proper data upload/download speeds and supporting breakpoint resume due to potential network interruptions – to be discussed with MCC 2.1.2 AI/ML Model Storage/Sharing: - Companies agree that the current assumptions and estimates for the CSI compression use case are reasonable (Model size: ~7MB – 100MB, Upper bound should be 600MB per company per year -> ~10GB per year) - These assumptions are specific to the CSI compression use case study in RAN4 - The upper bound is assuming that a company contributes in each meeting within a year (6 WG meetings) 2.1.3 Testing and inference data: - Most companies commented that testing and inference data should be a subset of the training dataset so there would be no need to consider the data size separately - Testing data set should be 10-25% of the training dataset (most companies showed 10-12.5%) - Other details on partitioning or how to use the datasets would have to be discussed separately 2.2 Data Formats: 2.2.1 Training/Testing/Inference data: - Possible formats to be used for the dataset which brought up are: NumPy, Matlab and Parquet - Formats will have to be discussed with MCC to see which ones can be used in 3GPP. - Most companies prefer NumPy, some where ok also with Matlab - The details of the data formatting (how data is represented, what is contained in each field, etc) will have to be discussed and agreed after the file format will be decided 2.2.2 AI/ML models: - Possible formats to be used for the AI/ML models are ONNX, PyTorch and TensorFlow - Formats will have to be discussed with MCC to see which ones can be used in 3GPP - There are pros/cons for each format, most companies seem to favor ONNX as it is open source, however, more discussion is needed to agree on a format - Further details on the format (contents of the files and what information needs to be provided) will have to discussed/agreed after deciding on the format 2.3 Other Issues - How to document the file formats will have to be further discussed - Some level of documentation will be needed such that there is a common reference and anyone can process the data - Different use cases will likely have different needs and all formats discussed here apply for the CSI compression use case - If there will be a need to specify datasets or models, the level of documentation required might be different(more details might be needed, etc) - The need for an API for data sharing needs to be discussed - The “lifetime” of the data needs to be further discussed, some data used in intermediate steps in the RAN4 study might not have to be stored for a long time

Below we provide views on the remaining open issues including Training/Inference dataset formats and AI-ML model formats.

## Training/Inference dataset formats

Our views on the dataset formats are provided below:

- File format

  - Either NumPy or Matlab (v7.3+) file formats are recommended to be used

  - Both NumPy and MATLAB have efficient binary file formats for storing large arrays of data. NumPy formats are optimized for Python and NumPy. MATLAB HDF5 .mat can offer greater portability across different systems and programming languages including Python as well.

  - Both NumPy and Matlab HDF5 are open formats.

  - There is no substantial difference in file sizes and either format can work well. The expected file sizes for 600K dataset with (2,13,32) arrays are:

    - .npy file size: 1904 MB

    - .mat file size (compressed): 1720 MB

- Dataset format

  - A single dataset file for training and validation dataset. Separate dataset file for inference.

  - Separate dataset files to represent different system-level scenarios and for different number of target MIMO layers.

  - Dataset file includes a single float32 array of size [NS, NCH, NSB, NANT]

    - NS 		Number of dataset samples

    - NCH = 2 		Number of channels (real/imaginary parts)

    - NSB = 13 	Number of sub-bands

    - NANT = 32 	Number of BS Tx antennas / ports

  - The dataset includes eigenvectors of size [NANT x 1]

  - The following pre-processing is assumed for the dataset eigenvectors derivation based on agreements in [5]:

    - Eigenvectors of size [NANT x 1] are calculated based on average covariance channel matrix for the given MIMO layer for each sub-band.

    - Average of covariance channel matrix for the sub-band is calculated as an average of per-PRB covariance channel matrices

    - Per-PRB covariance channel matrix is derived based on 1 sample of channel realization per PRB

    - The phase of all entries of the eigenvector is normalized based on the phase of the 1st entry of the eigenvector

    - L2 norm of the eigenvector is equal to 1.

- How to handle datasets from different companies

  - Further discussion is needed on whether and how to mix the datasets from different companies. The respective step is not deemed necessary for the purposes of feasibility studies but may be required for the purpose of performance requirements definition.

  - For feasibility studies, it is recommended to select datasets provided by several companies and perform training separately on the respective datasets.

Proposal #1: 	Adopt the following Training/Inference dataset formats.

- Either NumPy or Matlab (v7.3+) file formats

- Use a single file for training and validation datasets, and a separate file for inference dataset

- Use separate dataset files to represent different system-level scenarios and for different number of target MIMO layers.

- Dataset file includes a single float32 array of size [Ns = 600000, NCH = 2, NSB = 13, NANT = 32]

- The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447

Select datasets provided by several companies and perform training separately on the respective datasets

## AI/ML models formats

For the feasibility analysis to progress, it is essential for companies to be able to exchange and utilize pre-trained decoder models. To facilitate this, there must be an agreement on the procedure and format for sharing AI-ML models. This agreement should prioritize the ease of model sharing and, critically, the usability of the models by other companies. These approaches should be compatible with PyTorch and/or TensorFlow, which are the commonly used development tools among companies. The suggested approaches for further discussion are as follows.

- Option 1: Share model parameters (weights/biases) in PyTorch or TensorFlow formats and source codes

  - Feasible with both PyTorch and TensorFlow. PyTorch allows the saving of a state dictionary containing the model parameters (PyTorch-specific checkpoint format), while TensorFlow permits the saving of model parameters as checkpoint files (HDF5 format).

  - Users will require the model's code to reconstruct the architecture before loading the weights, necessitating the sharing of the original code.

  - It is recommended to agree on a single development tool (PyTorch or TensorFlow) and align on the version of libraries to minimize compatibility issues.

  - This approach may reduce cross-platform portability issues.

- Option 2: Share entire models in PyTorch or TensorFlow formats (no source codes)

  - Entire models can be shared and used directly in their respective PyTorch or TensorFlow (SavedModel) formats, which include both the model architecture and the weights.

  - This method is the simplest for sharing models and does not require source code sharing. However, there may be portability issues between different platforms or versions, which may be difficult to overcome.

  - It is recommended to agree on a single development tool (PyTorch or TensorFlow) and align on the version of libraries to minimize compatibility issues.

- Option 3: Share models in ONNX (Open Neural Network Exchange) format (no source codes)

  - ONNX allows for the sharing of models across different frameworks that support ONNX, without the need for the original source codes.

  - This approach is applicable to both PyTorch and TensorFlow, and the models can be used in other deep learning frameworks that support ONNX (e.g., PyTorch, TensorFlow, MATLAB, OpenVINO).

  - Potential compatibility issues with ONNX may still arise and require further investigation.

  - ONNX models are typically used for inference and are a good candidate for inference testing, if such tasks arise in RAN4.

  - ONNX models by default do not support backpropagation and not applicable for training framework, which is a key pre-requisite for feasibility analysis task. Either manual conversion or additional tools/libraries (e.g., onnx2pytorch, onnx2torch) may be required to convert ONNX models to PyTorch / TensorFlow to allow use of models in the training loop. However, even in case of using libraries whether the converted models can be used in training effectively or not depends on multiple factors. For instance, if the conversion introduces operations that do not support gradient computation, training will not be possible.

Taking these factors into account, we think that for feasibility studies it may be preferable to consider more than a single approach. For instance, models can be shared in 1) ONNX format, and 2) either PyTorch or TensorFlow formats (without source codes). Furthermore, it is recommended to agree at least on the commonly used version of Python/ PyTorch/TensorFlow.

In addition, to the agreement on file formats for model sharing, further details of AI-ML models I/O interfaces needs to be clarified. For instance, models with trainable quantization blocks may require extra inputs depending on the implementation (e.g., trained encoder quantization weights are provided to the decoder). Therefore, to simplify alignment it is recommended focus on fixed grid quantization approaches, while more advanced methods can be considered at a later stage. In this case, the I/O interfaces for the encoder/decoder become straightforward.

Proposal #2: 	AI-ML models are shared in both 1) ONNX format, and 2) PyTorch (or TensorFlow) formats

# Conclusion

In this paper we provide our views on logistical aspects of AI/ML models and dataset sharing for the CSI compression use case. In summary, make the following proposals:

Proposal #1: 	Adopt the following Training/Inference dataset formats.

- Either NumPy or Matlab (v7.3+) file formats

- Use a single file for training and validation datasets, and a separate file for inference dataset

- Use separate dataset files to represent different system-level scenarios and for different number of target MIMO layers.

- Dataset file includes a single float32 array of size [Ns = 600000, NCH = 2, NSB = 13, NANT = 32]

- The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447

Select datasets provided by several companies and perform training separately on the respective datasets

Proposal #2: 	AI-ML models are shared in both 1) ONNX format, and 2) PyTorch (or TensorFlow) formats

# References

- RP-221348, “SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #96, June 2022

- 3GPP TR 38.843 “Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface”

- RP-242399, “WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #105, September 2024

- RP-242350,	“AI/ML Logistical Issues”, CAICT, Qualcomm, RAN #105, September 2024

- R4-2414447, “Alignment of model training and data set parameters for AI/ML-based CSI compression” , Nokia, RAN4 #112, August 2024