# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415693

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.3.2

Source: 	Samsung

Title: 	UE RF requirements for Less than 5MHz supporting in NTN FR1 bands

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, supporting 3MHz CHBW for NTN operation is one of major objective with detailed WID as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signalling to support the above objectives

In this contribution, we analyse the expected specification impact on supporting 3MHz CHBW from UE RF requirements perspective.

2		Discussion

2.1 Section 6 Tx RF requirements

In below table we summarize the expected changes/update on TS 38.101-5 section 6 Tx RF requirements due to 3MHz CHBW introduction.

| Section | Expected changes/update |
| --- | --- |
| 6.2 Transmitted power 6.2.1 UE maximum output power 6.2.2 UE maximum output power reduction 6.2.3 UE additional maximum output power 6.2.4 Configured transmitted power | According to WID, target PC is PC3. No update for MPR, refer to TN specifciation For regulation requirements and A-MPR (table), further work required for below NS values: - Band n254: NS_03N, NS_04N and NS_05N - Band n256: NS_24 - Band n255: NS_02N (no A-MPR required) |
| 6.3 Output power dynamics 6.3.1 Minimum output power 6.3.2 Transmit OFF power 6.3.3 Transmit ON/OFF time mask 6.3.4 Power control | 3MHz CHBW need to be added into section 6.3.1 and 6.3.2 without change on the requirements. |
| 6.4 Transmit signal quality | No update expected |
| 6.5 Output RF spectrum emission 6.5.1 Occupied bandwidth 6.5.2 Out of band emission 6.5.3 Spurious emission 6.5.4 Transmit intermodulation | For 6.5.1 Occupied bandwidth: - 3MHz CHBW need to added into table without change on requirement. For 6.5.2 out of band emission: - 3MHz CHBW need to be added into ACLR table without change on requirements - General emission mask needs to be update with 3MHz CHBW as similar changes for breaking point from TN TS 38.101-1 For 6.5.3 Spurious emission - FOOB need to be updated to add 3MHz CHBW with 6MHz boundary No update on 6.5.4 transmit intermodulation |

6.2 Transmitted power

For MPR, in TN specification 38.101-1, separate value is specified for Pi/2 BPSK under 3MHz CHBW (note 3 in table 6.2.2-1).  In NTN specification, MPR directly refer to TN specification. We believe same change applied for NTN on MPR table update for PC3.

Proposal 1: For 6.2.2 MPR requirements, reusing existing requirements from TN specification under 3MHz.

  - No update on NTN specification required (MPR directly refer to TN specification)

Proposal 2: For A-MPR and corresponding regulation requirements, further update and evaluation work required if 3MHz need to be applied:

  - Band n254: NS_03N, NS_04N and NS_05N

  - Band n256: NS_24

  - Band n255: NS_02N (no A-MPR required)

6.3 Output power dynamics

Minimum output power and transmit OFF power were specified as -40dBm and -50dBm irrespective CHBW. For 3MHz CHBW, same requirements are applied with measurement bandwidth as 2.715MHz.

Proposal 3: For 6.3.1 Minimum output power and 6.3.2 transmit OFF power, add 3MHz CHBW into table with existing requirements:

- Minimum output power: -40dBm with the measurement bandwidth 2.715 MHz

- Transmit OFF power: -50dBm with the measurement bandwidth 2.715 MHz

Table 6.3.1-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 3 | -40 | 2.715 |
| 5 | -40 | 4.515 |
| 10 | -40 | 9.375 |
| 15 | -40 | 14.235 |
| 20 | -40 | 19.095 |

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth | (MHz) | 3, 5, 10, 15, 20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| Transmit OFF power | (dBm) | -50 |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

6.5 Output RF spectrum emission

For occupied channel bandwidth, 3MHz CHBW need to be added into table without change on the requirements.

Proposal 4: For 6.5.1 “Occupied channel bandwidth”, add 3MHz CHBW into table 6.5.1-1 for “Occupied channel bandwidth”.

Table 6.5.1-1: Occupied channel bandwidth

|  | NR NTN satellite channel bandwidth (MHz) |
| --- | --- |
|  | 3, 5, 10, 15, 20 |
| Occupied channel bandwidth (MHz) | Same as NR NTN satellite channel bandwidth |

For spectrum emission mask, general emission mask needs to be update with 3MHz CHBW as similar changes for breaking point from TN TS 38.101-1

Proposal 5: For 6.5.2 general spectrum emission mask: Reuse 3MHz emission mask requirements from TN specification TS 38.101.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20 |  |
| ± 0-1 | -13 | -13 | -13 | 1 % of channel BW |
| ± 1-5 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |
| ± 6-10 |  | -25 |  |  |
| ± 5-BWChannel |  |  | -13 |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |

Proposal 6: For 6.5.2.4.1 NR ACLR, add 3MHz CHBW with same requirement as 30dBc for PC3.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3, 5,10,15,20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

Proposal 7: For 6.5.3 Spurious emission: FOOB need to be updated to add 3MHz CHBW with 6MHz boundary

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20 | BWChannel + 5 |

Proposal 8: No specification impact foreseen for 6.4 Transmit signal quality, 6.5.4 Transmit intermodulation,  6.3.3 Transmit ON/OFF time mask, 6.3.4 Power control and 6.5.2.4.2	UTRA ACLR on supporting 3MHz CHBW.

2.2 Section 7 Rx RF requirements

In below table we summarize the expected changes/update on TS 38.101-5 section 7 Rx RF requirements due to 3MHz CHBW introduction.

| Section | Expected changes/update |
| --- | --- |
| 7.3 Reference sensitivity | New requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254. - 2.2 dB scaling factor can be applied based on existing 5MHz requirements FRC can be refer to TN specification without update on TS 38.101-5. |
| 7.4 Maximum input level | 3MHz CHBW need to be added into table without change on the requirements. |
| 7.5 Adjacent channel selectivity | 3MHz CHBW need to be added into table without change on the requirements. - The interference signal configuration also need to be updated with CHBW 3MHz and offset. |
| 7.6 Blocking characteristics 7.6.2 In-band blocking 7.6.3 Out of band blocking 7.6.4 Narrow band blocking | For in-band blocking, similar change as TN specification, add 3MHz CHBW into table and interference frequency offset need to be updated. For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements. For narrow band blocking, similar update as TN specification, 3MHz CHBW added into table with Pw as 18dBw. |
| 7.7 Spurious response | 3MHz CHBW need to be added into table without change on the requirements. |
| 7.8 Intermodulation characteristics | NTN specification directly refer to TN specification and update in TN specification for 3MHz CHBW also applicable. No update on NTN specification expected. |
| 7.9 Spurious emissions | No update expected |
| FRC | NTN UE RF specification, FRC refer to TN specification and new 3MHz FRC already specified in TN specification which can be applied for NTN as well. No additional update on FRC expected. |

7.3 Reference sensitivity

Proposal 9: For 7.3 Reference sensitivity, new requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254:

  - 2.2 dB offset can be applied based on existing 5MHz requirements

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -102.2 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.4 Maximum input level

Proposal 10: For 7.4 Maximum input level 3MHz CHBW need to be added into table without change on the requirements as -40dBm.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 3, 5, 10, 15, 20 |
| Power in Transmission Bandwidth Configuration3 | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-2 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in 3GPP TS 38.101-1 [5] Annex A.3.2.3 for 64 QAM. NOTE 3: Power in transmission bandwidth configuration value is rounded to the nearest 0.5dB value. |  |  |

7.5 ACS

Proposal 8: For 7.5 ACS, 3MHz CHBW need to be added into table without change on the requirements.

  - The interference signal configuration also needs to be updated with CHBW 3MHz and offset.

Table 7.5-1: ACS for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20 |
| ACS | dB | 33 | 30 | 27 |

Table 7.5-2: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm | REFSENS + 45.5 dB | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset)2 | MHz | 3/-3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | -71.5 | -71.5 | -68.5 | -65.5 |
| Pinterferer | dBm | -40 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3/-3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

7.6 Blocking requirements

Proposal 9: For 7.6 blocking requirements, following changes required:

  - For in-band blocking, add 3MHz CHBW into table and interference frequency offset need to be updated.

  - For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements.

  - For narrow band blocking, 3MHz CHBW added into table with Pw as 18dBm.

Table 7.6.2-1: In-band blocking parameters for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + 9 dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 and 15 kHz SCS. NOTE 3: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.6.3-1: Out-of-band blocking parameters for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

Table 7.6.4-1: Narrow Band Blocking

| Operating Band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 |
| n254, n255, n256 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |
|  |  |  | 18 | 16 | 13 | 14 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 3 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  |
|  | Fuw (offset SCS= 30 kHz)3 | MHz | NA |  |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: The PREFSENS power level is specified in clause 7.3.2. NOTE 3: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |  |

7.7 Spurious response

Proposal 10: For 7.7 Spurious response, 3MHz CHBW need to be added into table without change on the requirements as -40dBm.

Table 7.7-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration value is rounded to the next higher 0.5dB value. |  |  |  |  |

Proposal 11: FRC: reusing TN specification FRC of 3MHz, no additional update in NTN specification.

3		Conclusion

In this contribution, brief summary on UE RF specification impact of introducing 3MHz CHBW was provided.

Conductive requirements:

6.2 Transmitted power

Proposal 1: For 6.2.2 MPR requirements, reusing existing requirements from TN specification under 3MHz.

  - No update on NTN specification required (MPR directly refer to TN specification)

Proposal 2: For A-MPR and corresponding regulation requirements, further update and evaluation work required if 3MHz need to be applied:

  - Band n254: NS_03N, NS_04N and NS_05N

  - Band n256: NS_24

  - Band n255: NS_02N (no A-MPR required)

6.3 Output power dynamics

Proposal 3: For 6.3.1 Minimum output power and 6.3.2 transmit OFF power, add 3MHz CHBW into table with existing requirements:

- Minimum output power: -40dBm with the measurement bandwidth 2.715 MHz

- Transmit OFF power: -50dBm with the measurement bandwidth 2.715 MHz

6.5 Output RF spectrum emission

Proposal 4: For 6.5.1 “Occupied channel bandwidth”, add 3MHz CHBW into table 6.5.1-1 for “Occupied channel bandwidth”.

Proposal 5: For 6.5.2 general spectrum emission mask: Reuse 3MHz emission mask requirements from TN specification TS 38.101.

Proposal 6: For 6.5.2.4.1 NR ACLR, add 3MHz CHBW with same requirement as 30dBc for PC3.

Proposal 7: For 6.5.3 Spurious emission: FOOB need to be updated to add 3MHz CHBW with 6MHz boundary

Proposal 8: No specification impact foreseen for 6.4 Transmit signal quality, 6.5.4 Transmit intermodulation, 6.3.3 Transmit ON/OFF time mask, 6.3.4 Power control and 6.5.2.4.2	UTRA ACLR on supporting 3MHz CHBW.

Conductive Rx RF requirements

7.3 Reference sensitivity

Proposal 9: For 7.3 Reference sensitivity, new requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254:

  - 2.2 dB offset can be applied based on existing 5MHz requirements

7.4 Maximum input level

Proposal 10: For 7.4 Maximum input level 3MHz CHBW need to be added into table without change on the requirements as -40dBm.

7.5 ACS

Proposal 8: For 7.5 ACS, 3MHz CHBW need to be added into table without change on the requirements.

  - The interference signal configuration also needs to be updated with CHBW 3MHz and offset.

7.6 Blocking requirements

Proposal 9: For 7.6 blocking requirements, following changes required:

  - For in-band blocking, add 3MHz CHBW into table and interference frequency offset need to be updated.

  - For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements.

  - For narrow band blocking, 3MHz CHBW added into table with Pw as 18dBm.

7.7 Spurious response

Proposal 10: For 7.7 Spurious response, 3MHz CHBW need to be added into table without change on the requirements as -40dBm.

FRC

Proposal 11: FRC: reusing TN specification FRC of 3MHz, no additional update in NTN specification.

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] TS 38.101-1 v18.6.0

[3] TS 38.101-5 v18.6.0

[4] R4-2413528, Way Forward for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi