# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415694

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.3.1

Source: 	Samsung

Title: 	System parameters for Less than 5MHz supporting in NTN

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, supporting 3MHz CHBW for NTN operation is one of major objective with detailed WID as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signalling to support the above objectives

In this contribution, views on remaining open issues as list in the WF [4] were provided:

System parameters Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - FFS Issue 1-2-6: Signalling of SS raster - FFS Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN - FFS

2		Discussion

Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

3MHz CHBW was introduced with punctured PBCH (12 RB). The sync raster location for 3MHz CHBW is separate compared to normal sync raster with other CHBWs.

Supporting 3MHz CHBW is required additional UE processing complexity in terms of initial cell search on new sync raster, baseband processing on punctured PBCH/control channel, RF capability of supporting 3MHz CHBW.

For TN specification, UE feature list (51-1/-2) were introduced for supporting 3MHz CHBW and 12 RB coreset #0 under NR_FR1_lessthan_5MHz_BW WI. And in TN specification, supporting 3MHz CHBW was agreed as optional feature as per band basis.

| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

Regarding UE capabilityt, either extend existing UE feature list/capability signalling (51-1, 51-2) to cover NTN bands as per band basis or new separate UE feature list can be introduced for NTN 3MHz CHBW similar as TN. We believe extending existing UE feature list shall be more proper way given this feature was introduced by RAN1 with basic physical layer update which for NTN operation we reuse the design from TN without any changes on RAN1.

Proposal 1: Support 3MHz CHBW for NR-NTN in FR1-NTN bands is optional as per FR1 NTN band basis. Proposal 2: Extend existing UE capability signaling “support3MHz-ChannelBW-Symmetric-r18” to cover NTN FR1 bands.

Issue 1-2-6: Signalling of SS raster

We didn’t see any needs of introducing new signalling of SS raster as well as UE supports 3MHz CHBW than UE need to support corresponding SS rasters.

Proposal 3: No need to introduce new signalling for SS raster.

Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN

Supporting Redcap/eRedcap NTN operation is still on discussion in another parallel Rel-19 WI NTN Phase 3.

For Rel-18 TN supporting 3MHz CHBW, it’s clearly stated that 3MHz CHBW feature is not applicable for UEs supporting Redcap and eRedcap.

Observation: Supporting 3MHz channel bandwidth to Redcap/eRedcap NTN is out Rel-19 scope.

Proposal 4: Not consider Redcap/eRedcap NTN operation on supporting 3MHz CHBW in Rel-19 timeframe.

3		Conclusion

In this contribution, we provide views on remaining open issues for supporting 3MHz.

Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

Proposal 1: Support 3MHz CHBW for NR-NTN in FR1-NTN bands is optional as per FR1 NTN band basis. Proposal 2: Extend existing UE capability signaling “support3MHz-ChannelBW-Symmetric-r18” to cover NTN FR1 bands.

Issue 1-2-6: Signalling of SS raster

Proposal 3: No need to introduce new signalling for SS raster.

Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN

Observation: Supporting 3MHz channel bandwidth to Redcap/eRedcap NTN is out Rel-19 scope.

Proposal 4: Not consider Redcap/eRedcap NTN operation on supporting 3MHz CHBW in Rel-19 timeframe.

We also provide a template for work split of CR drafting on supporting 3MHz CHBW to trigger discussion on work split in RAN4#112bis meeting at BDaT session.

| TS 38.101-5 NTN UE RF |  |  |
| --- | --- | --- |
| Sections | Scope | Volunteer on CR drafting |
| Big CR | Big draft CR/CR on TS 38.101-5 UE RF |  |
| Section 5 | System parameters |  |
| Section 6.2 | UE Tx transmitted power requirements |  |
| Section 6 except 6.2 | Other Tx RF requirements except 6.2 |  |
| Section 7 | UE Rx RF requirements |  |
| TS 38.108 SAN RF |  |  |
| Big CR | Big draft CR/CR on TS 38.108 SAN RF |  |
| Section 5 | System parameters |  |
| Section 6 | Conductive Tx RF requirements |  |
| Section 7 | Conductive Rx RF requirements |  |
| Section 9 | Radiated Tx RF requirements |  |
| Section 10 | Radiated Rx RF requirements |  |
| Annex | FRC, others if needed |  |

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] TS 38.101-1 v18.6.0

[3] TS 38.101-5 v18.6.0

[4] R4-2413528, Way Forward for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi