3GPP TSG-RAN WG4 Meeting #112bis	R4-2415696

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on CSSF optimization of R19 RRM enhancements

Agenda Item:	6.15.2.2

Document for:	Discussion

- Introduction

In recent RAN plenary 105 meeting, a revised WID of R19 RRM enhancements has been approved in [1]. We summarize the objectives of the WID as below:

Objective of Core part WI - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. Specify RRM performance requirements and test cases for the Core part enhancements.

This revision makes the general condition on CSSF enh more clear. As long as the UE is not operating with multi-Rx based simultaneous L3 measurement, then the CSSF enh can be applicable. Based on this, we believe no need to pay effort to identify the general condition in which CSSF enh can be applicable. Companies could pay more attention to the exact solution itself.

During last RAN4 meeting, the following progress were achieved and summrized in [2] as below:

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption” Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - Final applicability of the solutions can be FFS - EN-DC - EN-DC with FR2 only intra band CA - EN-DC with FR2 only inter band CA - EN-DC with FR1 +FR2 CA (FR1 PSCell) - EN-DC with FR1 +FR2 CA (FR2 PSCell) - NE-DC - NE-DC with FR2 only intra band CA - NE-DC with FR2 only inter band CA - NE-DC with FR1 +FR2 CA (FR1 PCell) - SA - FR1+FR2 CA (FR1 PCell) - FR1+FR2 CA (FR2 PCell) - FR2 only intra-band CA - FR2 only inter-band CA - NR-DC - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

In this paper, we provide our views on the 2nd adaptation of the 1st topic.

- Discussion

Regarding this sub topic, we share our views from the following aspects:

- Whether to prioritize any type of L3 measurement

- Exact solution

- Whether to prioritize any type of L3 measurement

During last meeting, some company proposed to prioritize intra-frequency measurement without gap among all the intra-frequency/inter-frequency/inter-RAT measurement without gap. We admit the intra-frequency measurement dominates the searcher consumption, focus the CSSF enhancement on the intra-frequency measurement could be more concentrated. But it is better not to restrict the applicable case, i.e. even the inter-frequency or inter-RAT measurement without gap configured, still the CSSF enhancement could be applicable. Based on such principle, when we re-design the CSSF factor, all type of measurements including intra-frequency/inter-frequency/inter-RAT measurement without gap should be considered.

Proposal 1: To to more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered.

- Exact solution

- During previous meetings, multiple solutions proposed by companies. We classify them into multiple types:

- Type 1: Only measure one serving carrier per band

- Type 2: Dynamic searcher sharing between PCC and SCC

- Type 3: Adaptation on 2*NSCC_CSIRS

- Type 4: Adaptation on the assumption of searcher number

- From our point of view, we are open to all types of optimization. It seems that Type 1 and Type 2 can be deemed as the starting point.

- Observation 1: Multiple solutions proposed by companies. We classify them into multiple types:

- Type 1: Only measure one serving carrier per band

- Type 2: Dynamic searcher sharing between PCC and SCC

- Type 3: Adaptation on 2*NSCC_CSIRS

- Type 4: Adaptation on the assumption of searcher number

- Proposal 2: Type 1 and Type 2 can be deemed as the starting point.

- When the exact solutions on how to optimize CSSF in FR2-1 are concluded, the solution could be extended to FR1 if possible.

- Proposal 3: Extend the solution to FR1 if possible.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: To to more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered.

- Observation 1: Multiple solutions proposed by companies. We classify them into multiple types:

- Type 1: Only measure one serving carrier per band

- Type 2: Dynamic searcher sharing between PCC and SCC

- Type 3: Adaptation on 2*NSCC_CSIRS

- Type 4: Adaptation on the assumption of searcher number

- Proposal 2: Type 1 and Type 2 can be deemed as the starting point.

- Proposal 3: Extend the solution to FR1 if possible.

- Reference

- RP-242380 “Revised WID: NR Radio Resource Management (RRM) Phase 5”, Apple, CATT

- R4-2414026 “WF for NR_RRM_Ph5_Part1”, Apple