3GPP TSG-RAN WG4 Meeting #112bis	R4-2415703

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of R19 SBFD

Agenda Item:	6.20.3

Document for:	Discussion

- Introduction

In RAN plenary 102 meeting, a R19 WI on Sub-band full duplex (SBFD) was approved in [1]. We summarize the objectives of the WI as below:

- Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for co-channel CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

We highlight the objectives relevant to RAN4 RRM session. For the 1st RAN4 RRM aspects, it clearly shows that RAN4 should define the requirements for CLI measurement. Based on the investigation during SI phase and the RAN1 progress until RAN1#117 meeting in R19 phase, the CLI measurement contains UE-to-UE CLI and gNB-to-gNB CLI. The 2nd RAN4 RRM task is general, all RRM aspects regarding the SBFD operation should be identified and concluded.

In last meeting, good discussion were held and the following progress were achieved in [2] as below:

### Sub-topic 1-1: Work plan Agreements (from online session) - The work plan in R4-2412534 is approved. ### Issue 2-1-1: Scope of requirements for UE-to-UE CLI handling Agreements (from online session) - RAN4 to define RRM requirements for L1 based UE-to-UE CLI measurement and reporting. ### Issue 2-1-7: Side condition Agreements (from online session) - RAN4 to define side conditions for L1 based UE-to-UE CLI measurement requirements. - RAN4 to discuss at least time offset between DL timing and SRS arrival timing, SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP. - The RAN1 progress will be taken into account when discussing the values for the side conditions. ### Issue 2-1-8: Measurement period Agreements (from online session) - RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements, - RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design. - Number of shots for simulation: 1, 2, 3. - Companies to bring proposals on other simulation parameters in the next meeting. ### Issue 2-2-1: Scope of requirements for gNB-to-gNB CLI handling Agreements (from online session) - RAN4 not to define RRM requirements for gNB to gNB CLI handing. ### Issue 2-3-1: Requirements for legacy UE Agreements (from online session) - The legacy UEs shall perform and operate according to the legacy RRM requirements and no new RAN4 RRM requirement for legacy UEs, despite being served by a gNB operating with SBFD. - Clarification on the applicability of the existing RRM requirements in RAN4 spec is not precluded. - The legacy UEs refer to non-SBFD aware UE. ### Issue 2-3-2: Requirements for SSB based measurement Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell.

It has been determined to not define RRM requirements for gNB-to-gNB CLI, no need further discussion. In this paper, we provide our views on the UE-to-UE CLI and SBFD operation based on RAN1 progress.

- Discussion

- 2.1 UE-to-UE CLI

Background of R16 CLI

Before we diving into the discussion of R19 CLI, we can review the legacy R16 CLI measurement first. In legacy R16 CLI, only UE-to-UE CLI measurement was introduced to support dynamic TDD. Through performing the CLI-RSSI and/or SRS-RSRP measurement on the configured resource within active DL BWP, the potential victim UE can detect the interference to its DL Rx cased by the UL Tx of the aggressor UE. Both the SRS-RSRP and CLI-RSSI measurements are L3 measurement, L3 filtering allowed to the measurement results. RAN4 decided the measurement requirements and accuracy requirements for the two types of CLI resource.

When identifying the accuracy of SRS-RSRP measurement, the side condition of timing offset between UE’s DL reference timing in the serving cell and SRS arrival time was decided as Terror_SRS_RSRP, defined in clause 10.1.22.1 in [6].

Since no QCL-type D info was configured for the CLI resource in FR2, so the beam assumption of CLI resource is defined in [6] as the UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Based on RAN1 progress until RAN1#117 meeting[3][4][5], the framework of UE-to-UE CLI is depicted from the following aspects.

- Measurement type

- The UE-to-UE CLI is L1 measurement, no longer L3 measurement, which is distinguished from legacy R16 CLI;

- Measurement resource

- The UE-to-UE CLI also contains SRS-RSRP measurement and CLI-RSSI measurement. The measurement resource could be periodic, semi-persistent or aperiodic resource (set). The SRS-RSRP measurement resource is based on the legacy RS pattern, no new pattern introduced.

- How to determine the QCL typeD assumption of the CLI resource, still FFS.

- Measurement reporting

- At least aperiodic reporting supported.

- The report quantities includes: L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices.

Observation 1: The R19 UE-to-UE CLI measurement is determined as L1 measurement, both SRS-RSRP measurement and CLI-RSSI measurement are allowed, the resource can be configured as periodic, semi-persistent, or aperiodic resource. At least aperiodic reporting supported.

Observation 2: The QCL typeD assumption of the CLI resource is configured or determined by the NW.

- From RAN4 perspective, we analysis the UE-to-UE CLI from the following the following aspects:

- Inter-cell CLI and/or intra-cell CLI

- Measurement period requirements

- Side condition

- Accuracy requirements

- Inter-cell CLI and/or intra-cell CLI

- It should be noted that it is clarified in RAN1 note as below, no specification impact for SRS configuration information exchange between gNBs. In last meeting, when discussing the side condition especially the condition of time offset, it is proposed that different time offset be assumed for intra-cell and inter-cell CLI measurement. If we look at the RAN1 agreement highlighted blue, it seems that the inter-cell CLI resource configuration is up to NW implementation.

- Agreement: (RAN1#117) - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource

Observation 3: For the CLI resource configuration, due to no info exchange between gNBs, both intra-cell and inter-cell CLI resource configuration is up to NW implementation.

- Furthermore, based on the following progress, RAN3 would discuss the SBFD configuration exchange between cells. To our understanding, until now, neither of the two possibilities including same SBFD T/F configuration and different SBFD T/F configuration between SBFD cells is excluded.

Agreement: (RAN1#116bis) If coordinated scheduling in time and/or frequency is supported for gNB-to-gNB CLI handling and UE-to-UE CLI handling, the following is recommended to be specified - Information exchange of semi-static cell-specific SBFD time and frequency location configuration Agreement: (RAN1#117) For enhancements for CLI handling, the following are recommended from RAN1's perspective: - Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3] - Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3]

Observation 4: Until now, neither of the two possibilities including same SBFD T/F configuration and different SBFD T/F configuration between SBFD cells is excluded.

From this point of view, we believe both intra-cell and inter-cell CLI measurements are beneficial to help the NW be aware of the intra-cell and inter-cell DL/UL interference avoidance. For the case of aligned SBFD configuration between cells, the intra-cell CLI benefits the DL/UL interference avoidance mainly.  Whereas for the case of disaligned SBFD configuration between cells, both intra-cell and inter-cell CLI contribute to the DL/UL interference avoidance.

However from the UE perspective, the victim UE performs CLI measurement on the RSSI/RSRP CLI resource, regardless the aggressor UEs are intra-cell or inter-cell, which is transparent to the victim UE.

Proposal 1: Both intra-cell and inter-cell CLI measurements are considered for UE-to-UE CLI.

- Measurement period requirements

- For UE-to-UE CLI measurement, currently there are 3 methods are on the table based on recent RAN1 progress:

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

- Within the 3 methods, only Method#1 is direct measurement, by which the victim UE can directly detect the RSSI in the DL subband, the interference to the UE Rx in DL subband can be inferred. The other two are both indirect measurements, victim UE senses the RSRP/RSSI of the UL subband, after receiving such CLI report, NW could estimate the interference leakage to the Rx operation on the DL subband of the victim UE. We believe the measurement period requirements under different methods should be discussed one by one.

Proposal 2: The measurement period requirements for the three candidate UE-to-UE CLI measurements should be discussed one by one.

Based on last meeting agreements, RAN4 could determine the number of shots for measurement based on evaluation. Before entering into the simulation details, the parameter assumption should be aligned between companies. After reviewing the R16 CLI evaluation discussion and progress, so as to reuse the R16 simulation assumption as much as possible, the following assumptions for the SRS-RSRP simultation are preferred by us:

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | UE0 (cell 0) [victim UE] | UE1 (cell 1) [aggressor UE] | UE2 (cell 1) [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1, 30GHz for FR2 |  |  |
| Subcarrier spacing |  | 15kHz, 30kHz,60kHz for FR1 60kHz, 120kHz for FR2 | same as UE0 | same as UE0 |
| SRS configuration | Number of port | N/A | 1 | 1 |
|  | TransmissionComb |  | 2 | 2 |
|  | combOffset |  | 0 | 0 |
|  | Cyclic shift |  | 0 | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 | 0 |
|  | Number of symbol |  | 1 | 1 |
|  | Number of RB Note1 |  | 24 or 48 | same as UE1 |
|  | Frequency domain position and shift |  | 0 | 0 |
|  | Periodicity [slot] |  | 5 | same as UE1 |
| Timing errorNote2 for UE 1 and UE2 |  | [4.67 usec for FR1, 3.67 usec for FR2] | - | - |
| CP Length |  | Normal |  |  |
| Number of measurement samples |  | [1 or 2 or 3] | N/A | N/A |
| Measurement bandwidth [RB] |  | Same as SRS transmission RB |  |  |
| Measurement period [slot] |  | 5 | N/A | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A | N/A |
| Propagation conditions |  | AWGN, TDL-A10, TDL-C100 for FR1, AWGN, TDL-A75, TDL-C300 for FR2 |  |  |
| SINR |  | -3dB or -1dB | N/A | N/A |
| Note 1: Allocated RBs are within active BWP for measurement UE. Note 2: Timing error is Tc*NTA_offset + Tother. Tother = Tpropagation_delay + Tcell_phase_synchronization Where Tother = 4.67usec for FR1 and 3.67usec for FR2 under the assumption that ISD=500m for FR1 and ISD=200m for FR2 Note 3: No frequency hopping is assumed. |  |  |  |  |

Proposal 3: Reuse the simulation assumption for SRS-RSRP in R16 CLI as much as possible, the following parameter setting are suggested:

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | UE0 (cell 0) [victim UE] | UE1 (cell 1) [aggressor UE] | UE2 (cell 1) [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1, 30GHz for FR2 |  |  |
| Subcarrier spacing |  | 15kHz, 30kHz,60kHz for FR1 60kHz, 120kHz for FR2 | same as UE0 | same as UE0 |
| SRS configuration | Number of port | N/A | 1 | 1 |
|  | TransmissionComb |  | 2 | 2 |
|  | combOffset |  | 0 | 0 |
|  | Cyclic shift |  | 0 | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 | 0 |
|  | Number of symbol |  | 1 | 1 |
|  | Number of RB Note1 |  | 24 or 48 | same as UE1 |
|  | Frequency domain position and shift |  | 0 | 0 |
|  | Periodicity [slot] |  | 5 | same as UE1 |
| Timing errorNote2 for UE 1 and UE2 |  | [4.67 usec for FR1, 3.67 usec for FR2] | - | - |
| CP Length |  | Normal |  |  |
| Number of measurement samples |  | [1 or 2 or 3] | N/A | N/A |
| Measurement bandwidth [RB] |  | Same as SRS transmission RB |  |  |
| Measurement period [slot] |  | 5 | N/A | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A | N/A |
| Propagation conditions |  | AWGN, TDL-A10, TDL-C100 for FR1, AWGN, TDL-A75, TDL-C300 for FR2 |  |  |
| SINR |  | -3dB or -1dB | N/A | N/A |
| Note 1: Allocated RBs are within active BWP for measurement UE. Note 2: Timing error is Tc*NTA_offset + Tother. Tother = Tpropagation_delay + Tcell_phase_synchronization Where Tother = 4.67usec for FR1 and 3.67usec for FR2 under the assumption that ISD=500m for FR1 and ISD=200m for FR2 Note 3: No frequency hopping is assumed. |  |  |  |  |

- Side condition

- For Method#2, we believe the following side conditions are need to consider:

- Side condition of time offset between UE’s DL reference timing in the serving cell and SRS arrival time.

- Side condition of SRS Es/Iot.

- Side condition of maximum/minimum SRS-RSRP

For the time offset, the following options were discussed in last meeting:

- Time offset between DL timing and SRS arrival timing - Option 1: Rel-16 CLI SRS RSRP assumption - Option 2: Remove cell phase error from option 1 - Other options are not precluded, pending on RAN1 progress.

Since until right now, no relevant progress achieved in RAN1, so it seems that RAN4 needs to identify the time offset as one of the side condition. We prefer to down-select between Option 1 and Option 2.

Option 2 applies to the intra-cell CLI without cell phase error considered. Option 1 includes the cell phase error so it is available to the inter-cell CLI. Based on our analysis in the 1st issue, neither intra-cell nor inter-cell CLI measurement could be excluded until now, so we prefer Option 1 which accomodates both intra-cell and inter-cell CLI measurements.

Proposal 4: For the side condition of time offset, reusing R16 CLI SRS RSRP assumption to accomodate both intra-cell and inter-cell CLI measurements for Method#2. No need to identify time offset for Method#1 and #3.

For the side condition of SRS Es/Iot, similar as above, only need to define this side condition for Method#2. When we identify the side condition of SRS Es/Iot, we should jointly consider several points: 1) The probable distance between the aggressor UE and victim UE; 2) The measurement accuracy with selected measurement samples and possible time offset. So the exact value of SRS Es/Iot could be determined according to the evaluation results.

Proposal 5: Determine the side condition of SRS Es/Iot for Method#2 according to the evaluation results. Not need this side condition in Method#1 and #3.

Regarding the side condition of minimum RSRP, which is relevant to the receiver REFSENS. So for all Method#1, 2, 3, directly reuse the value identified in R16 CLI measurements.
For the side condition of maximum RSRP, which is only needed for Method#2. The exact value is determined by the SRS Tx power at aggressor UE, the antenna gain and the pathloss etc. So whether reuse the value in R16 SRS-RSRP CLI, depending on whether these factors changed.

Proposal 6: For the side condition of minimum RSRP, reuse the value in R16 CLI measurements.

Proposal 7: For the side condition of maximum RSRP, whether reusing the value in R16 SRS-RSRP CLI measurement, depend on whether the assumptions on SRS Tx power, antenna gain and pathloss changed or not.

- Accuracy requirements

- After the requirements of measurement period identified, then RAN4 can determine the accuracy requirements. Since the R19 UE-to-UE CLI is no longer L3 measurement, so the legacy UE-to-UE CLI accuracy requirements can not be reused directly. The exact accuracy can be determined based on the evaluation results.

Proposal 8: The R19 UE-to-UE CLI is L1 measurement, instead of L3 measurement, so the legacy UE-to-UE CLI accuracy requirements can not be reused directly. Determining the accuracy requirements based on the evaluation results.

- 2.3 SBFD operation

- RAN1 developed the discussion on SBFD resource configuration, collision handling and Tx/Rx/Measurement procedure and some progress achieved. Based on these details on SBFD operation, the impact on RRM should be identified by RAN4.

- Before diving into the RAN4 aspects, the basic SBFD resource configuration should be noticed.

- The SBFD resource are semi-statically configured including frequency and time domain. In time domain, some consecutive DL and flexible symbols in one or multiple consecutive slots can be configured as SBFD symbols, with the periodicity same as TDD-UL-DL pattern periodicity. In frequency domain, 1 or 2 DL subbands and 1 UL subbands are allowed. All SBFD symbols share the same DL/UL subband configuration. Based on such SBFD resource configuration, full duplex operation is assumed from NW perspective, while half duplex operation is assumed from UE perspective.

- Besides, some types of collision between the semi-static configuration and dynamic scheduling were recognized in RAN1. The already approved aproaches are generally reusing the existing collision handling mechanism in NR for F symbols, i.e. the dynamic scheduling prioritize semi-static configuration. But no specific consideration for RRM measurement. So we need identify the impact on RRM measurements.

- Requirements for SSB based measurement

- In last meeting, it was approved that the existing requirements are reused for SSB based serving cell measurement. The reason behind lies in that the SSB is prioritized over semi-statically configured UL transmission and dynamically scheduled UL transmission. But similar as in legacy, how to handle the potential overlapping between neighbour cell SSB and the UL transmission, which is not touched by RAN1.

- Actually the scheduling restriction and MG could handle such collision. For the neighbour cell measurement without MG, the UL transmission was deprioritized by the RRM measurements in TDD. It is straightforward to reuse the measurement restriction to handle the collision between the neighbour cell measurements and UL transmission, including both semi-statically configured UL transmission and dynamically scheduled UL transmission. For the neighbour cell measurement with MG, since the measurements were performed within MG, due to the interruption caused by MG, so it is impossible to collide between neighbour cell measurements and UL transmission. While regarding the collision between MG and UL transmission, considering both MG and SBFD symbols are configured semi-statically, so from NW perspective, it is meaningless to configure MG and SBFD in the same symbol. However the MG pattern and periodicity are pre-defined in the unit of ms, not relevant to TDD-UL-DL pattern periodicity, the periodicity of SBFD is same as the TDD-UL-DL pattern periodicity, therefore the collision between MG and SBFD symbols may happen. Since interruption caused by MG, so no matter SBFD symbol or non-SBFD symbol overlapped with the interruption caused by MG, the RRM measurements with MG is prioritized than the data transmission.

Proposal 9: For the neighbour cell SSB based measurement, including both measurement with MG and measurement without MG, reusing the existing measurement restriction and interruption requirements to handle the collision between measurements and UL transmission in SBFD symbols.

- Requirements for CSI-RS based measurement

- The CSI-RS based RRM measurement is performed based on the periodic CSI-RS configured via csi-rs-ResourceConfigMobility. According to RAN1 progress as below, for the collision between semi-statically configured DL reception and dynamically scheduled UL transmission, existing collision handling mechanism in NR for Flexible symbol is reused, i.e. the dynamically scheduled UL transmission could cancle the semi-statically configured DL reception. Following this solution, the CSI-RS based RRM measurement would be impacted by the overlapping UL transmission in SBFD symbols.

Agreement:(RAN1#116) For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included. Agreement:(RAN1#116bis) If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

- Observation 5: According to RAN1 progress, the CSI-RS based RRM measurement would be impacted by the overlapping dynamically scheduled UL transmission in SBFD symbols.

- When some measurement occasions are cancled by UL transmission in SBFD symbols, to guarantee the measurement performance, applying additional scaling factor to extend the CSI-RS based measurements could be considered.

- Proposal 10: Applying additional scaling factor to guarantee the performance of CSI-RS based measurements provided some measurement occasions are cancled by UL transmission in SBFD symbols.

- Furthermore, it seems that the existing scheduling restriction for TDD is not applicable to the SBFD symbols between the CSI-RS based RRM measurements and UL transmission any more. So some new solution of scheudling restriction for TDD could be considered.

- Proposal 11: The existing scheduling restriction for CSI-RS based measurement in TDD is not applicable to SBFD symbols any more, need to consider new solution.

- Requirements for scheduling restriction

- Actually the scheduling restriction has been touched in the analysis on the above two issues. To summarize, RAN4 needs to identify the scheduling restriction on the SBFD symbols. For SSB based RRM measurement, it seems directly reusing the existing scheduling restriction is fine. While for CSI-RS based RRM measurement, some updated scheduling restriction is needed compared with legacy schduling restriction.

- Proposal 12: RAN4 needs to identify the scheduling restriction on the SBFD symbols for SSB based RRM measurements and CSI-RS based RRM measurements respectively. For SSB based, reusing the existing scheduling restriction is preferred. For CSI-RS based, some updated solution is needed.

- Requirements for RACH

- Based on RAN1 progress, RACH is allowed in SBFD symbols, new RO configurations and resource mapping were touched by the SBFD based RACH. Since the RACH relevant requirements are generic, so we can not see any impact on RACH requirements to support the RACH in SBFD symbols.

- Proposal 13: No impact on the RACH requirements.

- The Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Observation 1: The R19 UE-to-UE CLI measurement is determined as L1 measurement, both SRS-RSRP measurement and CLI-RSSI measurement are allowed, the resource can be configured as periodic, semi-persistent, or aperiodic resource. At least aperiodic reporting supported.

Observation 2: The QCL typeD assumption of the CLI resource is configured or determined by the NW.

Observation 3: For the CLI resource configuration, due to no info exchange between gNBs, both intra-cell and inter-cell CLI resource configuration is up to NW implementation.

Observation 4: Until now, neither of the two possibilities including same SBFD T/F configuration and different SBFD T/F configuration between SBFD cells is excluded.

Proposal 1: Both intra-cell and inter-cell CLI measurements are considered for UE-to-UE CLI.

Proposal 2: The measurement period requirements for the three candidate UE-to-UE CLI measurements should be discussed one by one.

Proposal 3: Reuse the simulation assumption for SRS-RSRP in R16 CLI as much as possible, the following parameter setting are suggested:

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | UE0 (cell 0) [victim UE] | UE1 (cell 1) [aggressor UE] | UE2 (cell 1) [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1, 30GHz for FR2 |  |  |
| Subcarrier spacing |  | 15kHz, 30kHz,60kHz for FR1 60kHz, 120kHz for FR2 | same as UE0 | same as UE0 |
| SRS configuration | Number of port | N/A | 1 | 1 |
|  | TransmissionComb |  | 2 | 2 |
|  | combOffset |  | 0 | 0 |
|  | Cyclic shift |  | 0 | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 | 0 |
|  | Number of symbol |  | 1 | 1 |
|  | Number of RB Note1 |  | 24 or 48 | same as UE1 |
|  | Frequency domain position and shift |  | 0 | 0 |
|  | Periodicity [slot] |  | 5 | same as UE1 |
| Timing errorNote2 for UE 1 and UE2 |  | [4.67 usec for FR1, 3.67 usec for FR2] | - | - |
| CP Length |  | Normal |  |  |
| Number of measurement samples |  | [1 or 2 or 3] | N/A | N/A |
| Measurement bandwidth [RB] |  | Same as SRS transmission RB |  |  |
| Measurement period [slot] |  | 5 | N/A | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A | N/A |
| Propagation conditions |  | AWGN, TDL-A10, TDL-C100 for FR1, AWGN, TDL-A75, TDL-C300 for FR2 |  |  |
| SINR |  | -3dB or -1dB | N/A | N/A |
| Note 1: Allocated RBs are within active BWP for measurement UE. Note 2: Timing error is Tc*NTA_offset + Tother. Tother = Tpropagation_delay + Tcell_phase_synchronization Where Tother = 4.67usec for FR1 and 3.67usec for FR2 under the assumption that ISD=500m for FR1 and ISD=200m for FR2 Note 3: No frequency hopping is assumed. |  |  |  |  |

Proposal 4: For the side condition of time offset, reusing R16 CLI SRS RSRP assumption to accomodate both intra-cell and inter-cell CLI measurements for Method#2. No need to identify time offset for Method#1 and #3.

Proposal 5: Determine the side condition of SRS Es/Iot for Method#2 according to the evaluation results. Not need this side condition in Method#1 and #3.

Proposal 6: For the side condition of minimum RSRP, reuse the value in R16 CLI measurements.

Proposal 7: For the side condition of maximum RSRP, whether reusing the value in R16 SRS-RSRP CLI measurement, depend on whether the assumptions on SRS Tx power, antenna gain and pathloss changed or not.

Proposal 8: The R19 UE-to-UE CLI is L1 measurement, instead of L3 measurement, so the legacy UE-to-UE CLI accuracy requirements can not be reused directly. Determining the accuracy requirements based on the evaluation results.

Proposal 9: For the neighbour cell SSB based measurement, including both measurement with MG and measurement without MG, reusing the existing measurement restriction and interruption requirements to handle the collision between measurements and UL transmission in SBFD symbols.

- Observation 5: According to RAN1 progress, the CSI-RS based RRM measurement would be impacted by the overlapping dynamically scheduled UL transmission in SBFD symbols.

- Proposal 10: Applying additional scaling factor to guarantee the performance of CSI-RS based measurements provided some measurement occasions are cancled by UL transmission in SBFD symbols.

- Proposal 11: The existing scheduling restriction for CSI-RS based measurement in TDD is not applicable to SBFD symbols any more, need to consider new solution.

- Proposal 12: RAN4 needs to identify the scheduling restriction on the SBFD symbols for SSB based RRM measurements and CSI-RS based RRM measurements respectively. For SSB based, reusing the existing scheduling restriction is preferred. For CSI-RS based, some updated solution is needed.

- Proposal 13: No impact on the RACH requirements.

- Reference

- RP-234035, “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, 3GPP RAN#102, 2023.

- R4-2413900, “WF on SBFD RRM”, Huawei, HiSilicon, 3GPP RAN4#112, 2024.

- 3GPP TR 38.858_i10, Study on Evolution of NR Duplex Operation, (Release 18).

- Final_Minutes_report_RAN1#116_v100

- Final_Minutes_report_RAN1#116b_v100

- Final_Minutes_report_RAN1#117_v020

- TS 38.133_i50.