3GPP TSG-RAN WG4 Meeting #112bis	R4-2415708

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Other RX requirements for LP-WUR

Agenda item:	6.23.2.3

Document for:	Discussion

Introduction

In this document we provide our views on other receiver requirements for the LP-WUR. During the last meeting [1], no agreements were made regarding this agenda.

Discussion

Maximum input level

LP-WUR and main receiver (MR) will observe the same radio environment. Thus, for LP-WUR to reliably operate and co-exist with legacy NR signals, it should be able to handle large input signals. For the main receiver, as there is a limit for the maximum desired input signal level, thus a similar requirement should be specified for the LP-WUR.

Specify maximum input level requirements and side conditions for LP-WUR.

For the main receiver, the requirement power level varies based on channel bandwidth and modulation type. However, as possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz, a single value maybe sufficient.

Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

A single value for the maximum input level is sufficient for LP-WUR.

The value of maximum input level depends on the maximum gNB output power and the minimum coupling loss (MCL) between the gNB and the UE. The maximum gNB output power has not changed and is same for LP-WUS and NR signal. Further, there is no change on the assumptions regarding MCL. Thus, the value of maximum input level for MR can be used for LP-WUR.

There hasn’t been any change in assumptions regarding MCL and gNB output power.

Use maximum input level of -25 dBm for LP-WUR.

The reference measurement channel for the maximum input level requirement shall be defined for the LP-WUR as the current reference measurement channels use 64 QAM or 256 QAM. Further, the number of RBs is going to be different compared to the current ones.

Define reference channel for LP-WUR requirements.

Spurious emission

Spurious emissions are the narrow band CW emissions appearing at the UE antenna connector. Given that the current requirements are regulatory in nature, they should apply to the LP-WUR.

Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

Conclusion

In this paper following Observations and Proposals were made:

Proposal 1: Specify maximum input level requirements and side conditions for LP-WUR.

Observation 1: Possible bandwidths being considered for LP-WUS in FR1 are ≤5MHz.

Proposal 2: A single value for the maximum input level is sufficient for LP-WUR.

Observation 2: There hasn’t been any change in assumptions regarding MCL and gNB output power.

Proposal 3: Use maximum input level of -25 dBm for LP-WUR.

Proposal 4: Define reference channel for LP-WUR requirements.

Proposal 5: Reuse the spurious emission requirements from TS 38.101-1 for LP-WUR.

- References

- R4-2414309, WF on LP-WUS UE RF requirements, Vivo, RAN4#112, Maastricht, Netherlands, August 19th – August 23rd, 2024.