3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415711

Hefei, China, 14th -18th October, 2024

Source:	Qualcomm Incorporated

Title:	Power boosting and MPR reduction

Agenda item:	6.1.1.2.1

Document for:	Approval

# 1.	Introduction

In RAN4#112 there was a new WF [1] to study power domain enhancements for NR single carrier. In this paper we present further views on this topic.

# 2. 	Discussion

In RAN4#112 there was a WF created [1] to further study the open items on power domain enhancements for NR single carrier. Some of the unresolved issues are noted below:

#### Issue 1-2-1: Clarification of BS CBW

#### Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

#### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW

#### Issue 1-2-5: Boundary to apply ACLR and SEM

#### Issue 1-2-6: Boundary to apply SE

#### Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW

#### Issue 1-2-11: Signaling aspects

In the above mentioned WF it was proposed that companies clarify the meaning of BS CBW. In our opinion the BS CBW is the entire RF BS BW held by an operator. This CBW can be divided between several UEs or allocated to a single UE based on the operator configuration.

Proposal #1: The BS CBW is the entire RF BS BW held by an operator

In this same WF companies were asked to consider approaches for converting outer RB allocations to inner RB allocations. In the last meeting both UE CBW and BS CBW based schemes were considered. In our opinion outer allocations can be converted to inner allocations by extending the UE BW on one or both sides on the original UE BW. This can be done by the BS as it knows the excess BW available on both sides of the UE BW. When extending the UE BW the BS should indicate to the UE the extended NRB along with the new RB start so that the UE can correctly orient itself within the extended BW.

Proposal#2: BW extension should be BS CBW based whereby the BS signals the extended NRB and the new RB start to the UE

The WF also asks companies to consider whether and where to use IBE in the larger BS CBW. We think that IBE should be used within the UE BW as well as the extended UE BW.

Proposal#3: IBE should be used within the extended UE BW.

Once the UE BW is extended then the ACLR and SEM specifications should be applied from the edges of the extended UE BW.

Proposal#4: ACLR and SEM specifications should be applied from the edges of the extended UE BW

When the UE BW is extended, we think that it makes most sense for the 3GPP FOOB limits to apply from the edge of the extended UE BW. As mentioned previously in [2] there are ITU specifications [3] that must be satisfied for spurious emissions. The ITU specifications are referenced from the centre of the original UE BW while the 3GPP spurious emissions specifications are referenced from the edge of the UE BW. There is a possibility that for certain extended UE BWs the 3GPP specification may not cover the entire frequency range specified by the ITU spec. The selected size of the UE BW extension must ensure that the frequency range covered by the 3GPP FOOB specification encompasses the frequency range covered by the ITU specifications for the original UE BW. Otherwise, there would be frequency ranges where the ITU specifications apply that are not covered by the 3GPP specification.

Proposal#5: FOOB limits should apply from the edge of the extended UE BW

Proposal#6: The extended UE BW must be chosen such that the frequency range covered by the 3GPP FOOB specifications for the extended UE BW encompass the frequency range specified by the ITU specifications for the original UE BW.

# Conclusion

In this paper we further discuss our views on the study of power domain enhancement and make the following proposals:

Proposal #1: The BS CBW is the entire RF BS BW held by an operator

Proposal#2: BW extension should be BS CBW based whereby the BS signals the extended NRB and the new RB start to the UE

Proposal#3: IBE should be used within the extended UE BW.

Proposal#4: ACLR and SEM specifications should be applied from the edges of the extended UE BW

Proposal#5: FOOB limits should apply from the edge of the extended UE BW

Proposal#6: The extended UE BW must be chosen such that the frequency range covered by the 3GPP FOOB specifications for the extended UE BW encompass the frequency range specified by the ITU specifications for the original UE BW.

# References

[1] R4-2414441, WF on power domain enhancements, RAN4#112, August 19th – August 23rd, 2024

[2] R4-2407910, Discussion on power domain enhancement for NR single carrier, August 19th – August 23rd, 2024