3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415713

Hefei, China, 14th -18th October, 2024

Source:	Qualcomm Incorporated

Title:	MPR applicability for FR1 intra-band UL CA

Agenda item:	6.1.1.2.2

Document for:	Approval

# 1.	Introduction

In RAN4#112 there was much discussion on which MPR tables apply for FR1 intra-band ULCA when intra-band contiguous UL CA is configured but only 1 CC is activated. In this paper we further present our views on this topic.

# 2. 	Discussion

In RAN4#112 a WF was created to further discuss most issues related to intra-band contiguous and non-contiguous CA with a single cell activated [1]. It was discussed that for intra-band contiguous UL CA if only 1 CC is activated then for PC3 and PC2 scenarios the single CC MPR tables should apply. This issue could not be fully resolved at the last meeting and was left for further discussion. We see no reason why the single CC MPR tables cannot be used in the above scenarios. The single CC MPR tables have lower MPR values compared to the intra-band contiguous UL CA. The use of the single CC MPR values enables the UL to transmit more power there by increasing its throughput. The current single CC MPR tables 6.2.2-1, 6,2D.2-1 and 6.2.2-2 contain the following note:

- Applicable for a UE indicating support for UE capability [powerBoostRel18] or [powerBoostRel18TS] and if the IE [powerBoostPi2BPSKRel18] is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass]

This note indicates that the power boosting is possible for UE indicating [powerBoostRel18] or [powerBoostRel18TS] capability and the corresponding MPR tables shows that power boosting is possible for PC3 and PC2 QPSK and Pi/2 BPSK modulations. To ensure that power boosting is allowed the corresponding configured power requirements must be referenced along with each MPR table. Omission of the configured power requirements will leave the use of the power boosting feature ambiguous.

We see no reason why power boosting should not be allowed when only 1 CC is activated as it enhances UE performance.  Though the power boosting feature was developed for single CC we do not see why it should not be used for this scenario where only 1 CC is activated as the benefits of using power boosting are abundantly clear.

Observation#1: Boosting enhances UE performance and though it was developed for single CC there is no reason why it cannot be used when only 1 CC is activated.

It has been stated that the boost powers agreed in the standard are relatively small, however we think that any amount of power boost is worth exploiting as any extra UL power is beneficial to UE performance.

Observation#2: Any amount of power boost is beneficial to UE performance

Companies that have concerns for allowing power boosting when 1 CC is activated should bring reasons to justify their position.

Proposal#1: Companies that do not want power boosting when 1 CC is activated should justify their reasons

We propose that the following be agreed:

- For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

  - MPR defined in Table 6.2.2-1 along with the configured Tx power requirements stated in section 6.2.4 applies to UE power class 3 CA bandwidth classes B and C,

  - MPR defined in Table 6.2D.2-1 along with the configured Tx power requirements stated in section 6.2D.4   applies to power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - MPR defined in Table 6.2.2-2 along with the configured Tx power requirements stated in section 6.2.4 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal#2: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements.

  - MPR defined in Table 6.2.2-1 along with the configured Tx power requirements stated in section 6.2.4 applies to UE power class 3 CA bandwidth classes B and C,

  - MPR defined in Table 6.2D.2-1 along with the configured Tx power requirements stated in section 6.2D.4   applies to power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - MPR defined in Table 6.2.2-2 along with the configured Tx power requirements stated in section 6.2.4 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

We believe that though only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR as this is in keeping with what is done at present.

Proposal#3: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR.

Also, when 1 CC is activated the spurious emissions, ACLR and SEM specifications that are used for validation should correspond to that for the aggregated configured CBW.

Proposal#4: When 1 CC is activated the spurious emissions, ACLR and SEM specifications that apply are those for the aggregated configured CBW

For intra-band non-contiguous UL CA, the scheme to be used when only 1 CC is activated was left for further discussion [1]. From [2] we see that according to section 6.2A.2.2.0 the following is mentioned for UEs indicating and not indicating dualPA-Architecture.

#### 6.2A.2.2	UE maximum output power reduction for Intra-band non-contiguous CA

##### 6.2A.2.2.0	General

For intra-band non-contiguous CA, the allowed Maximum Power Reduction (MPR) for the maximum output power is specified into 2 types: MPR to meet -30dBm/MHz and -13dBm/MHz. The UE determins the MPR type as follows:

For UE indicating dualPA-Architecture supported

If OR (LCRB1 = 0, LCRB2 = 0)

MPR defined in Table 6.2.2-1 and Table 6.2.2-2 for PC3 and PC2 UE respectively

Else If AND( FIM3,low_block,low > SEM-13,low ,  FIM3,high_block,high < SEM-13,high )

MPR defined in Clause 6.2A.2.2.2.1 and Clause 6.2A.2.2.2.2 for PC3 and PC2 UE respectively.

Else

MPR defined in Clause 6.2A.2.2.1.1 and Clause 6.2A.2.2.1.2 for PC3 and PC2 UE respectively.

For UE without indicating dualPA-Architecture supported

If OR( LCRB1 = 0, LCRB2 = 0 )

For PC3 UE, MPR defined in Table 6.2.2-1, except for B < 9 MHz where 5.5 dB MPR is used;

For PC2 UE without indicating TxD, MPR defined in Table 6.2.2-2 is used, except for B < 11.52 MHz where 6.5 dB MPR is used;

For PC2 UE indicating TxD, MPR defined in Table 6.2D.2-1 is used, except for B < 11.52 MHz where the maximum value between 6.5 dB and MPR defined in Table 6.2D.2-1 is used.

According to [2] for a UE indicating or not indicating dualPA-Architecture for intra-band non-contiguous CA when either LCRB1=0 or LCRB2=0 the single CC MPR tables are used. As the RB determination occurs at scheduling it is seen that for the non-contiguous CA the standard [2] already accounts for the scenario where only 1 CC is scheduled. In the above text the exception stated for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for  PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement. These exceptions were added based on measurement work done in [3]

Observation#3: For PC3 and PC2 intra-band non-contiguous CA the standard already accounts for using the single CC MPR tables when only one CC is scheduled.

Proposal#5: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

Observation#4: The exceptions stated in section 6.2A.2.2.0 for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement.

# Conclusion

In this paper we present further views on MPR applicability for FR1 intra-band UL CA and make the following observations and proposals:

Observation#1: Boosting enhances UE performance and though it was developed for single CC there is no reason why it cannot be used when only 1 CC is activated.

Observation#2: Any amount of power boost is beneficial to UE performance

Proposal#1: Companies that do not want power boosting when 1 CC is activated should justify their reasons

Proposal#2: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements.

  - MPR defined in Table 6.2.2-1 along with the configured Tx power requirements stated in section 6.2.4 applies to UE power class 3 CA bandwidth classes B and C,

  - MPR defined in Table 6.2D.2-1 along with theconfigured Tx power requirements stated in section 6.2D.4   applies to power class 2 CA bandwidth classes B and C when TxD capability is indicated.

  - MPR defined in Table 6.2.2-2 along with the configured Tx power requirements stated in section 6.2.4 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal#3: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR.

Proposal#4: When 1 CC is activated the spurious emissions, ACLR and SEM specifications that apply are those for the aggregated configured CBW

Observation#3: For PC3 and PC2 intra-band non-contiguous CA the standard already accounts for using the single CC MPR tables when only one CC is scheduled.

Proposal#5: For PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required.

Observation#4: The exceptions stated in section 6.2A.2.2.0 for the cases when B< 9MHz or 11.52MHz and MPR is set to either 5.5 dB or 6.5 dB for PC3 and PC2 respectively is to meet the -30 dBm/MHz emissions requirement.

# References

[1] R4-2414441, WF on power domain enhancements, August 19th – August 23rd, 2024

[2] TS38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.6.0

[3] R4-2204225, 1CC fall-back MPR for NC UL CA with 1LO architecture, February 21st – March 3rd, 2022