3GPP TSG-RAN WG4 Meeting #112-Bis	R4-2415725

Hefei, China, Oct. 2024

Source	:	Qualcomm Incorporated

Title	:	On capturing wider applicability of higherPowerLimit-r17

Agenda item	:	6.1.1.1.4

Document for	:	Approval

# 1.	Introduction

RAN4 has agreed on wider applicability of higherPowerLimit-r17 and higherPowerLimitMRD-r17 [4]. There are however some details in the standard that need attention when the agreement is translated into a requirement.

In this contribution we share our view on the subject.

# 2. 	Discussion

In the previous 3 Tx WI, enabling higherPowerLimit-r17 was explicitly precluded by the WID [5].

- Specify requirements for 3Tx including - Introduce requirements for inter-band UL CA +UL MIMO/TxD - Introduce requirements for inter-band EN-DC +UL MIMO/TxD - Clarify applicable release for 3Tx with inter-band UL CA/EN-DC band combinations in 38.307 specification - Note 1: Increase UE power high limit feature is not included

The applicability of the capability is captured in some of the inter-band CA max. power tables, for example table 6.2A.1.3-1 in TS38101-3:

At the same time, the inter-band CA max. power tables for CA+TxD or CA+ULMIMO do not include these notes (due to WID preclusion). Consequently, in the Rel-18 standard there is a mix of inter-band CA max. output power tables, some of which enable higherPowerLimit-r17 and some that do not.

For Rel-19, wider applicability of higherPowerLimit-r17 was agreed [4]:

- For both 2Tx and 3Tx, higherPowerLimit-R17/higherPowerLimtMRDC-R17 is enabled for any specified inter-band band combination

To implement this wider applicability of higherPowerLimit-r17 RAN4 have two options:

- Add the same note to each max. power table now, and in the future that touches inter-band CA in some form (examples are 6.2H.3.1-1, 6.2L.3.1-1)

- Add a note in the general section (6.1A), for example: ‘A UE that supports a CA configuration with 2 uplink bands may signal a higherPowerLimit-r17 capability whereby the maximum output power of the UE may be exceeded in accordance with sub-clause 6.2A.4.1.3.’

The second option (statement in 6.1A) is advantageous because it is more compact, less repetitive and robust to future inter-band enhancements.

Observation 1: Given the general nature of the agreed wider applicability of higherPowerLimit-r17, implementation in the general section (6.1A, 6.1B) is optimal.

Proposal 1: For Rel-19, the wider applicability of higherPowerLimit-r17/ higherPowerLimitMRDC-r17 is implemented in the standard by removing legacy notes inside inter-band power class tables and adding in sections 6.1A and 6.1B respectively, the following:

/***** 38.101-1 ***/ ## 6.1A General The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification. A UE that supports a CA configuration with 2 uplink bands with different power classes may signal a higherPowerLimit-r17 capability whereby the maximum output power of the UE may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. /***** 38.101-3***/ ## 6.1B General A UE that supports a DC configuration with 2 uplink bands with different power classes may signal a higherPowerLimitMRDC-r17 capability whereby the maximum output power of the UE may be exceeded in accordance with sub-clause 6.2B.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

# 3.	References

[1] 	RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, March 18th – March 21st , 2024

[2] 	R4-2406583, ‘WF on HPUE UE for UL CA and EN-DC’, Samsung, RAN4#110-Bis, Apr. 2024

[3] 	R4-2410750, ‘WF on higher power UE’, Samsung, RAN4#111, May 2024

[4] 	R4-2414277, ‘WF on HPUE for CA in TN’, Samsung, RAN4#112, Aug. 2024

[5] 	RP-234046, ‘WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC’, Oppo, RAN#100, Dec  2023