3GPP TSG-RAN WG4 Meeting #112bis	R4-2415730

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.2.3.2

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE RX RSD requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2 high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario.

In this contribution, our evaluation about PC2 HPUE RX reference sensitivity degradation (RSD) requirements for IoT NTN is provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

Based on the WID’s objective, it indicates that it is necessary to evaluate whether RX RSD requirements would be needed when TX HPUE is enabled.  In RAN4#112 meeting, the agreed WF [2] provides the following works about RX RSD for IoT-NTN HPUE operation. The contents are shown as follows.

Issue 4-3: RSD for IoT-NTN - Agreement: - For NB-Iot based Iot-NTN, no RSD needed for HPUE - For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE - For eMTC based Iot-NTN with full duplex, further discuss RSD impact

According to our evaluation, because of high TX-RX duplex separation frequency and narrow CBW for IoT application, no RX RSD is needed for IoT-NTN B256 and B254 when UE enables PC2 TX.

Proposal 1: For eMTC based IoT-NTN with FDD full duplex, no RSD needed for B256 and B254 under PC2 HPUE operation.

3 Conclusion

In this paper, our views on RX RSD requirements for IoT-NTN PC2 UE for B256 and B254 are provided as follows:

Proposal 1: For eMTC based IoT-NTN with FDD full duplex, no RSD needed for B256 and B254 under PC2 HPUE operation.

Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung