# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415732

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.2.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor issues captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT) Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement) - Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 5 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions. - Option 6a (Nokia): Mobility Event triggering BSF reduction - Option 6b (Nokia): FBS is triggered by conditional Handover configuration - Option 6c (Nokia): BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC). - Option 7 (QC): RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements: - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. - FFS on the application delay of UE Rx beam sweeping factor switch. - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 9 (ZTE): Due to L3 measurement is long-term operation, power consumption issue could be considered, which may lead to some interaction signalling. But which would not be the applicability condition of applying fast beam sweeping. [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed. Issue 1-2-9: FFS: DRX is configured or not [Way forward]: FFS: - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized - Option 2 (Ericsson): FBS may cover DRX cases, no need to deprioritize DRX case.

When CA mode is configured, one searcher is for PCell in PCC while another searcher is for SCells or other cells in SCCs, thus one searcher is considered for single carrier. However, the target scenario for the BSF topic is that UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC, if UE has two searchers, it’s not precluded to consider the two searchers used for the single carrier. Therefore, searcher occupied by single carrier is depended on UE implementation, one searcher or two searchers can be occupied by the single carrier.

Proposal 1: Searcher occupied by single carrier is depended on UE implementation, one searcher or two searchers can be occupied by the single carrier.

For SSB processing delay/time for processing multiple beams received in a SMTC, if one searcher is occupied by the single carrier when UE supporting multi-Rx simultaneous reception for L3 delay enhancement, we support to allow additional SSB processing delay for processing multiple beams received in a SMTC, meanwhile, UE baseband processing capabilities need to be considered.

Proposal 2: If one searcher is occupied by the single carrier when UE supporting multi-Rx simultaneous reception for L3 delay enhancement, it’s supported to allow additional SSB processing delay for processing multiple beams received in a SMTC, meanwhile, UE baseband processing capabilities need to be considered.

For power consumption issue, it’s our understanding that UE supporting BSF reduction feature shall not always activate multi-Rx operation for all L3 measurement, and BSF reduction feature is not the only reason to activate multi-Rx operation. Therefore, UE supporting BSF reduction feature can activate multi-Rx operation for L3 measurement when some scenarios/ conditions are fulfilled, e.g., low mobility or cell edge. It’s open to discuss that activation/ deactivation of multi-Rx operation for UE supporting BSF reduction feature is depended on NW configuration or UE indication.

Proposal 3: UE supporting BSF reduction feature can activate multi-Rx operation for L3 measurement when some scenarios/ conditions are fulfilled, e.g., low mobility or cell edge.

Proposal 4: Activation/ deactivation of multi-Rx operation for UE supporting BSF reduction feature can be depended on NW configuration or UE indication.

For whether to configure DRX or not, different ranges of DRX cycle are considered in L3 measurement requirements, and M value related to beam sweeping factor is defined for each DRX cycle range, therefore, it’s no need to deprioritize DRX case.

Proposal 5: DRX can be configured like legacy requirements.

Status in the WF [1]:

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Proposal 1 (CATT): - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Proposal 2 (Apple, CTC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 3 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 4 (LGE): - If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4. - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 5 (Ericsson): - As a result of FBS, RX beam sweep factor can be defined as [4]. - Proposal 6 (Ericsson): - RAN4 to study the requirements relevant to switching between different operations, including: - Transition time period when enabling and disabling FBS in Rel-19. - Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18. - Proposal 7 (Samsung): - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline - Whether to/How to define the other values can be based on SLS results (if necessary)

In Rel-18 multi-Rx WI, fast beam sweeping method was introduced for FR2 L1 measurement delay reduction, which can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF. For UE supporting BSF reduction feature for L3 measurement, the Rx BSF can be reduced to 2/4/6 according to the UE capability.

Proposal 6: Fast beam sweeping method introduced for FR2 L1 measurement delay reduction in Rel-18 multi-Rx WI can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF.

Proposal 7: For UE supporting BSF reduction feature for L3 measurement, the Rx BSF can be reduced to 2/4/6 according to the UE capability.

Status in the WF [1]:

Issue 1-5: feature capability of L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Option 1(Apple, vivo, Nokia, Samsung): - RAN4 to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - Option 1a(Apple): The detailed definition can be postponed to the end of the core part discussion. - Option 2(Eircsson): - To support FBS, RAN4 to check if those capabilities for multi-Rx in Rel-18, e.g., faster RX beam sweeping, enhanced scheduling and measurement restrictions and multi-Rx preference indication, can be used directly. If the NW indicates FBS and multi-RX in Rel-18 is not enabled parallelly, we can observe the possibility of reusing at least part of such capabilities.

It’s preferred to decouple UE capabilities of R19 L3 BSF reduction and R18 multi-Rx, therefore, it’s proposed to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

Proposal 8: It’s proposed to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: Searcher occupied by single carrier is depended on UE implementation, one searcher or two searchers can be occupied by the single carrier.

Proposal 2: If one searcher is occupied by the single carrier when UE supporting multi-Rx simultaneous reception for L3 delay enhancement, it’s supported to allow additional SSB processing delay for processing multiple beams received in a SMTC, meanwhile, UE baseband processing capabilities need to be considered.

Proposal 3: UE supporting BSF reduction feature can activate multi-Rx operation for L3 measurement when some scenarios/ conditions are fulfilled, e.g., low mobility or cell edge.

Proposal 4: Activation/ deactivation of multi-Rx operation for UE supporting BSF reduction feature can be depended on NW configuration or UE indication.

Proposal 5: DRX can be configured like legacy requirements.

Proposal 6: Fast beam sweeping method introduced for FR2 L1 measurement delay reduction in Rel-18 multi-Rx WI can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF.

Proposal 7: For UE supporting BSF reduction feature for L3 measurement, the Rx BSF can be reduced to 2/4/6 according to the UE capability.

Proposal 8: It’s proposed to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

- References

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #112, 19 – 23 August, 2024.