# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415734

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.3

Source:	China Telecom

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, fast SCell activation for UE supporting Rel-18 EMR was discussed and the WF was approved [1].

In this paper, we present our views on fast SCell activation for UE supporting Rel-18 EMR issues captured in the WF [1].

- Discussion

Status in the WF [1]:

#### Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: - Direct SCell Activation at SCell addition - FFS: Single MAC CE based DL SCell Activation - FFS: Single MAC CE based PUCCH SCell Activation - RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

For the scope of fast SCell activation for UE supporting Rel-18 eEMR, it’s proposed to consider single MAC CE based normal SCell activation and RRC based direct SCell activation at SCell addition.

Proposal 1: For the scope of fast SCell activation for UE supporting Rel-18 eEMR, it’s proposed to consider single MAC CE based normal SCell activation and RRC based direct SCell activation at SCell addition.

Status in the WF [1]:

#### Issue 1-1-4: Target scenarios of fast SCell activation Proposals - Proposal 1: - RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay). - Fast SCell activation for UE supporting Rel-18 EMR should aim to achieve less than 100ms activation delay. RAN4 to discuss the options how this is achieved. - Proposal 2: - Suggest to focus on the following scenario for fast SCell activation for UE supporting Rel-18 EMR: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1. - Proposal 3: - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1.

For target scenarios of fast SCell activation, the unknown SCell activation delay is long especially for FR2-1 SCell activation compared with known case, therefore, it’s proposed to discuss unknown SCell activation delay reduction in FR1 and FR2-1.

Proposal 2: For target scenarios of fast SCell activation, it’s proposed to discuss unknown SCell activation delay reduction in FR1 and FR2-1.

Status in the WF [1]:

#### Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation #### Issue 1-2-1a: How to update the known condition with consideration of valid eEMR reporting FFS on proposals listed in summary for issue 1-2-1a.

In our understanding, if UE has reported valid results on the SCell to be activated before SCell activation command, the SCell to be activated can be considered as known, and the activation delay requirements for known SCell can be applied. It was agreed that RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command, and it’s FFS on how to update the known condition with consideration of valid eEMR reporting. In our views, if the valid EMR reporting of the SCell to be activated is sent within X time window before the reception of SCell activation command, the SCell to be activated can be regarded as known SCell.

Proposal 3: The SCell to be activated can be regarded as known SCell if the valid EMR reporting of the SCell to be activated is sent within X time window before the reception of SCell activation command.

Status in the WF [1]:

#### Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation Proposals - Option 1: - RAN4 to discuss whether it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB

SINR condition is defined in legacy SCell activation procedure, in our views, during the whole procedure for known SCell activation requirement, including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode, a consistent SINR condition can be defined to ensure channel stability.

Proposal 4: A consistent SINR condition can be defined to ensure channel stability during the whole procedure for known SCell activation requirement including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode.

- Conclusion

The contribution presents our views on fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: For the scope of fast SCell activation for UE supporting Rel-18 eEMR, it’s proposed to consider single MAC CE based normal SCell activation and RRC based direct SCell activation at SCell addition.

Proposal 2: For target scenarios of fast SCell activation, it’s proposed to discuss unknown SCell activation delay reduction in FR1 and FR2-1.

Proposal 3: The SCell to be activated can be regarded as known SCell if the valid EMR reporting of the SCell to be activated is sent within X time window before the reception of SCell activation command.

Proposal 4: A consistent SINR condition can be defined to ensure channel stability during the whole procedure for known SCell activation requirement including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode.

- References

- R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT, RAN WG4 Meeting #112, 19 – 23 August, 2024.