# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415738

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.23.4.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, RRM core requirements for LP-WUS/WUR was discussed and the WF was approved [1].

In this paper, we present our views on RRM core requirements for LP-WUS/WUR issues captured in the WF [1].

- Discussion

General aspects

Status in the WF [1]:

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

For serving cell measurement in case #3, in our views, considering the implementation complexity, it’s preferred not to consider combining the measurements across the two radios, i.e., MR and WUR

Proposal 1: RAN4 not to consider combining the measurements across the two radios, i.e., MR and WUR.

Status in the summary [2]:

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation - Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple oppo xiaomi Docomo CT LG Huawei, ZTE, CATT) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. (Apple) - If both LR and MR are ON, RAN4 to discuss whether UE uses LR measurement or UE uses both LR and MR measurement to check with the relaxation/offloading/LP-WUS-monitoring criteria (especially in case#3) (Apple). - P2: RAN4 needs to further evaluate serving cell measurement relaxation and neighbouring cell measurement relaxation by MR (Samsung) - P3: Further discuss MR RRM measurement relaxation after Case#2 and Case#3 are agreed by RAN4 or other work groups (CMCC) - P4: When UE stays in MR+LR state, RAN4 shall consider the MR serving cell measurement relaxation and also study the mobility performance to quantify the relaxation such as scaling factor for time period (ZTE) Recommendations: Suggest to agree P1 following the WI, details on requirements are encouraged to be purposed and are FFS

For core requirements to be specified for MR RRM relaxation, since it was approved in the WID that RAN4 and RAN2 specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions, therefore, we support RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Proposal 2: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Status in the WF [1] and summary [2]:

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself. Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself. Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring Recommendations: To moderator’s understanding, criteria will be designed by RAN2. Better to consider other related issues except for the criteria itself. Issue 1-1-8: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7 - Proposals - P1: It is suggested to define the same entry/exit conditions for LP-WUR serving cell measurement and LP-WUS monitoring (Xiaomi LG) - P2: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and case 1, or case 2 or 3 if case 2 or 3 are supported (vivo). - P3: In order to guarantee the power saving gain, the serving cell quality threshold in entry/exit condition for LP-WUS monitoring and the serving cell quality threshold entry/exit condition for LP-WUR measurement should be considered jointly. (CMCC) - P4: RAN4 needs to consider overall entry/exit condition for LP-WUR measurement, MR measurement relaxation, and LP-WUS monitoring jointly based on RAN2 agreements of entry/exit condition for LP-WUS monitoring. And if there are other conditions that should be considered, RAN4 needs to provide input to RAN2 (LG) - P5: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with RAN1 and RAN2. (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - P6: assumption of thresholds for RAN4 discussion can be (Apple): - the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering. - the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring. Recommendations: Suggest to check whether the following top level description: LP-WUR monitoring is triggered not early than MR RRM relaxation or MR serving cell offloading (case 1)

Regarding criteria (entry/exit conditions) for LP-WUR serving cell measurement/ MR RRM measurement relaxation/ LP-WUS monitoring, we support that RAN2 is the main group for criteria (entry/exit conditions) design.

Proposal 3: RAN2 is the main group for criteria (entry/exit conditions) design for LP-WUR serving cell measurement/ MR RRM measurement relaxation/ LP-WUS monitoring.

Status in the summary [2]:

Issue 1-1-11: Considerations on higher priority frequency layer - Proposals - P1: higher priority layer neighbor cell measurement could also be OFF for case #1. (Apple vivo) - P2: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor. (ZTE)

In our views, higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Proposal 4: Higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the summary [2]:

Issue 1-2-1: Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (Apple oppo xiaomi Ericsson, ZTE) - P2: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section (LG) - P3: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account (CATT) - P4: Postpone the discussion on this issue (CMCC) - P5: RAN4 need to define LR measurement accuracy requirements at least for the fully offloading scenario. (Nokia)

For LP-WUR measurement requirements at RRC_IDLE/INACTIVE state, the measurement delay, measurement accuracy, and measurement filtering can be discussed to specify. For accuracy requirements, since there is no measurement reporting to network for LP-WUR measurement at RRC_IDLE/INACTIVE state, it’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Proposal 5: It’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Status in the summary [2]:

Issue 1-2-2: Possible interruption related requirements for LP-WUR - Proposals - P1: Interruption requirements could be defined for the time between LP-WUS received paging indication to MR ready for Paging monitoring (Samsung xiaomi Docomo CMCC) - In addition, between LP-WUR measurement result fulfills exit criteria and MR to start measurement (CMCC) - In addition, between MR measurement result fulfills entry criteria and LP-WUR to start measurement. (CMCC ZTE) - P2: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO. (ZTE)

In our views, considering that UE needs certain preparation time for the transition between LP-WUR and MR, it’s proposed to consider interruption requirements for the time between LP-WUS received paging indication to MR ready for paging monitoring.

Proposal 6: Interruption requirements could be defined for the time between LP-WUS received paging indication to MR ready for paging monitoring.

MR RRM relaxation

Status in the summary [2]:

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell - Proposals - P1: Use relaxation/scaling factor for MR serving/neighbour cell relaxation(CATT Apple xiaomi CT CMCC vivo Huawei Samsung ZTE MTK) - P1-1: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor (CT vivo Samsung ZTE MTK) - P1-2: >=8 if case 2 or case 3 are introduced (CMCC) - P1-3 >=16 (Apple Huawei vivo MTK) - P2: RAN4 has further study on MR RRM relaxation factor for serving and/or neigbhor cell (LG Ericsson) - P3: RAN4 to discuss which, if any, of the legacy MR neighboring cell RRM measurement relaxation applicable to MR while configured with LP-WUS. (Nokia) - Existing idle mode mobility test cases applies for a UE configured with LP-WUR usage (Nokia) - P4: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor. (CATT ZTE) - P5: As a starting point for RRM relaxation case (MR is relaxed and LR is ON), RAN4 can consider RRM relaxation for intra-frequency measurements in IDLE/INACTIVE mode where both serving and neighbour cells can be measured in the same frequency layer. (MTK)

For MR RRM relaxation, MR RRM relaxation factor for serving cell/neighbor cell is an important issue to be discussed in RAN4. Referring to analysis of RRM for LP-WUR duty-cycled monitoring in TR 38.869, the average of power saving gain provided by LP-WUS is 4% when MR relaxed < 8 times, which is very limited. While the gain is 40% when 8 times<= MR relaxed <=16 times, and the gain is 60% when MR relaxed > 16 times. Therefore, it’s proposed to discuss relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for MR RRM relaxation.

Proposal 7: It’s proposed to discuss relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for MR RRM relaxation.

Status in the summary [2]:

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor - Proposals - P1: Neighbour measurements have the more relaxation than or equivalent relaxation as serving cell measurement. (Apple) - P2: Same relaxation factor applies to serving and neighbor cell measurements (Apple Huawei MTK) - P3: Different scaling factor for serving and neighbour cell relaxation (MTK) - P4: Further discuss whether to use same scaling factor or not (CATT)

On neighbor cell and serving cell relaxation factor, in our views, it’s preferred to define same relaxation factor for MR RRM relaxation for serving and neighbor cell measurements.

Proposal 8: Define same relaxation factor for MR RRM relaxation for serving and neighbor cell measurements.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: RAN4 not to consider combining the measurements across the two radios, i.e., MR and WUR.

Proposal 2: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Proposal 3: RAN2 is the main group for criteria (entry/exit conditions) design for LP-WUR serving cell measurement/ MR RRM measurement relaxation/ LP-WUS monitoring.

Proposal 4: Higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Proposal 5: It’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Proposal 6: Interruption requirements could be defined for the time between LP-WUS received paging indication to MR ready for paging monitoring.

Proposal 7: It’s proposed to discuss relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for MR RRM relaxation.

Proposal 8: Define same relaxation factor for MR RRM relaxation for serving and neighbor cell measurements.

- References

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo, RAN WG4 Meeting #112, 19 – 23 August, 2024.

- R4-2411818, Topic summary for [112][223] NR_LPWUS, vivo, RAN WG4 Meeting #112, 19 – 23 August, 2024.