3GPP TSG-RAN WG4 Meeting #112bis	R4-2415739

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.2.3.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE TX RF requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2 high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario.

In this contribution, our views about PC2 HPUE TX RF requirements for IoT-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

In RAN4#112 meeting, the agreed WF [3] provides the following works for specifying TX requirements (e.g., ACLR, MPR) for IoT-NTN under HPUE operation. The contents are shown as follows.

NTN HPUE UE RF – Tx requirements Issue 3-1-3: Consideration of power classes - Agreement - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed Issue 3-3-1: Maximum output power - Agreement - Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance can be considered as starting point for PC2/1.5/1 for NTN HPUE including NR-NTN and IoT-NTN. Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.

According to specification of HPUE UE ACLR for FR1 LTE bands, the ACLR values in TS 36.101 [2] are provided in Tables below for reference.

TS 36.101 Figure 6.6.2.3-1: Adjacent Channel Leakage requirements for one E-UTRA carrier

| TS 36.101 Table 6.6.2.3.1-1: General requirements for E-UTRAACLR Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth. 1.4 MHz. E-UTRAACLR1; 30 dB. E-UTRA channel Measurement bandwidth; 1.08 MHz. Adjacent channel centre frequency offset [MHz]; +1.4 / -1.4. | TS 36.101 Table 6.6.2.3.1-2: Additional E-UTRAACLR requirements for Power Class 1 Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth. 1.4 MHz. E-UTRAACLR1; 37 dB. E-UTRA channel Measurement bandwidth; 1.08 MHz. Adjacent channel centre frequency offset [MHz]; +1.4 / -1.4. |
| --- | --- |

TS 36.101 Table 6.6.2.3.2-1: Requirements for UTRAACLR1/2 Channel bandwidth / UTRAACLR1/2 / Measurement bandwidth. 1.4 MHz. UTRAACLR1; 33 dB. Adjacent channel centre frequency offset [MHz]; 0.7+BWUTRA/2 / -0.7-BWUTRA/2. UTRAACLR2; -. Adjacent channel centre frequency offset [MHz]; -. E-UTRA channel Measurement bandwidth; 1.08 MHz. UTRA 5MHz channel Measurement bandwidth (NOTE 1); 3.84 MHz. UTRA 1.6MHz channel measurement bandwidth (NOTE 2); 1.28 MHz.

TS 36.101 Table 6.6.2F.3-1: category NB1 and NB2 UE ACLR requirements GSMACLR; UTRAACLR. ACLR; 20 dB; 37 dB. Adjacent channel center frequency offset from category NB1 or NB2 Channel edge; ±200 kHz; ±2.5 MHz. Adjacent channel measurement bandwidth; 180 kHz; 3.84 MHz. Measurement filter; Rectangular; RRC-filter α=0.22. Category NB1 and NB2 channel measurement bandwidth; 180 kHz; 180 kHz. Category NB1 and NB2 channel Measurement filter; Rectangular; Rectangular.

According to TS 36.102 [4], the ACLR values are provided in Tables below for reference.

TS 36.102 Table 6.5A.3.4-1: Category M1 ACLR requirements Channel bandwidth / E-UTRAACLR / Measurement bandwidth. 1.4 MHz. E-UTRAACLR; 30 dB. E-UTRA channel Measurement bandwidth; 1.08 MHz. Adjacent channel centre frequency offset [MHz]; +1.4/-1.4.

TS 36.102 Table 6.5B.3.4-1: Category NB1 and NB2 UE ACLR requirements GSMACLR; UTRAACLR. ACLR; 20 dB; 37 dB. Adjacent channel center frequency offset from category NB1 or NB2 Channel edge; ±200 kHz; ±2.5 MHz. Adjacent channel measurement bandwidth; 180 kHz; 3.84 MHz. Measurement filter; Rectangular; RRC-filter α=0.22. Category NB1 and NB2 channel measurement bandwidth; 180 kHz; 180 kHz. Category NB1 and NB2 channel Measurement filter; Rectangular; Rectangular.

Additionally, based on R4-2415963 [5] NTN-TN coexistence evaluations, the evaluation shows that IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE due to sufficient ISD protection.

| R4-2415963[5] contents below for reference: |  |
| --- | --- |
| Figure 6: TN UL Throughput loss ratio with nAAS @ GEO IoT-NTN | Figure 7: TN UL Throughput loss ratio with nAAS @ LEO1200 IoT-NTN |

Observation 1: Based on R4-2415963[5] NTN-TN coexistence evaluation, the evaluation shows that IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE due to sufficient ISD protection.

Proposal 1: Discuss whether to reuse IoT-NTN PC3 ACLR values for PC2, PC1.5, and PC1 ACLR because the NTN-TN coexistence evaluation [5] shows IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE.

Regarding SEM for IoT-NTN HPUE, we think that some SEM requirements could be reused for PC2 for narrow CBW. But if the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements (e.g., ACLR of 30dB for PC2 to PC1), we suggest discussing whether SEM requirements would be relaxed.

Proposal 2: Regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing SEM requirements when the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements.

The TX MPR is relevant to ACLR, SEM and modulation EVM requirements. If IoT-NTN PC3 ALCR could be reused for HPUE power class and relaxing SEM would be possible under HPUE operation, we think that reusing NR-NTN PC3 MPR values for PC2 MPR would be one way for consideration.

Proposal 3: If IoT-NTN PC3 ACLR value could be reused for IoT-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing IoT-NTN PC3 MPR values for PC2 MPR would be one possible way.

3 Conclusion

In this paper, our views about HPUE TX RF requirements for NR-NTN UE are provided as follows:

Observation 1: Based on R4-2415963[5] NTN-TN coexistence evaluation, the evaluation shows that IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE due to sufficient ISD protection.

Proposal 1: Discuss whether to reuse IoT-NTN PC3 ACLR values for PC2, PC1.5, and PC1 ACLR because the NTN-TN coexistence evaluation [5] shows IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE.

Proposal 2: Regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing SEM requirements when the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements.

Proposal 3: If IoT-NTN PC3 ACLR value could be reused for IoT-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing IoT-NTN PC3 MPR values for PC2 MPR would be one possible way.

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- TS 36.101, “E-UTRA; User Equipment (UE) radio transmission and reception”

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung

- TS 36.102, “E-UTRA; User Equipment (UE) radio transmission and reception for satellite access”

- R4-2415963, “Simulation for NTN co-existence study”, MediaTek Inc.