# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415744

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Updated simulation results of MIMO layer evaluation for 6Rx UE

Agenda Item:	6.1.1.3.2

Document for:	Approval

Introduction

In the last three meetings, we discussed the feasibility of 6MIMO layer however there was no consensus [1][2][3]. For 6Rx, simulation methodology and parameters for 4&6 MIMO layer performance comparison are aligned, as shown in the Annex. In this contribution, we give updated simulation results for the comparisons between 4 and 6 MIMO layers, based on that, our views on 6 MIMO layer support are provided.

Discussion

For MIMO layer support for 6Rx, we had following WF [3] in the last meeting:

- Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE Issue 3-1-1: Tightening BS EVM requirement Agreement: - Do not tighten the minimum requirements of BS Tx EVM - Further discuss the assumption of Tx EVM for 6-layer performance evaluation. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-1-4: Performance requirements for 6Rx Agreement: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices. - Sub-topic 3-2: 6-layer Support Issue 3-2-1: 6-layer Support Way Forward: RAN4 to further discuss the following options. - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs. Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

For 6 MIMO layer evaluation, we agreed to take link level simulation as baseline, using TDL channel models with antenna correlation matrix. We conducted performance simulation for 4&6 MIMO layer and the detailed simulation assumptions were listed in the Annex.

Simulation assumptions

If we check the simulation parameters in the Annex, there are many items with more than one option. In this case, there are many combinations of simulation cases if there is no down selection. In our simulation, we choose two typical sets of simulation assumptions:

Priority 1：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer + UE Correlation matrix: Option 2

Priority 2：TDLA30-5+DMRS 1+1 for 4layer and 2+0 for 6layer+ UE Correlation matrix: Option 1

UE antenna correlation

Antenna correlation is an important aspect for designing MIMO antenna systems. With lower antenna correlation, it indicates better MIMO performance that can be achieved with the underlying antenna system. The antenna correlation coefficient is given as the range of 0.1~0.5 between any two antennas, which is reasonable considering the actual antenna performances. The UE antenna parameter is given in Table 1 in the annex. From our perspective, we think Option 2 is a more realistic UE antenna correlation matrix and we use it in our first priority simulation case.  We also considered Option 1 in our simulation, which is Priority 2 case.

BS EVM and correlation matrix

In the last meeting, we agreed not to tighten BS Tx EVM requirements. Therefore, we use minimum BS Tx EVM requirements in the simulation. For BS correlation matrix, we assumed 8Tx with ideal non-correlation matrix.

PDSCH DMRS configuration

Precoding: To demodulate PDSCH based on the channel estimation result of DMRS, the precoding of DMRS is aligned with PDSCH, i.e, Type 1 codebook.

Simulation results

### Simulation results for Priority 1

Priority 1：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer + UE Correlation matrix: Option 2

| a) MCS13 | b) MCS17 |
| --- | --- |
| c) MCS adaption |  |

From the simulation results, based on TDL channel model, it’s observed that the throughput gain can be obtained by 6 MIMO layer with fixed MCS (MCS13, MCS17) and Rank. The SNR needs to be larger than 13.5 dB under fixed rank with MCS 13 and 19.0dB with MCS 17 when 6 layer has performance gain than 4 layer.  However, fixed rank with fixed MCS can not be the case to judge if 6 layer is feasible or not since in the real deployment both rank and MCS are adapted according to actual channel environment. Take Figure a) as example, In real field, when SNR just exceeds 13.5dB, the throughput is already saturated for rank 4 and higher MCS can be scheduled to achieve higher throughput. It can not be decided if 6-layer still has better performance than 4- layer considering MCS adaption.

When MCS adaption is enabled, there is no gain of 6 MIMO layers compared with 4 MIMO layer, since higher MCS could be scheduled and correctly demodulated in the case of 4 MIMO layer, while may not be demodulated correctly in the case of 6 MIMO layers.

Observation 1: The SNR needs to be larger than 13.5 dB under fixed rank with MCS 13 and 19.0dB with MCS 17 when 6 layer has performance gain than 4 layer.

Observation 2: When enabling MCS adaption and fixed Rank, 6 MIMO layer has no gain over 4 MIMO layer.

### Simulation results for Priority 2

Priority 2：TDLA30-5+DMRS 1+1 for 4layer and 2+0 for 6layer+ UE Correlation matrix: Option 1

| a) MCS13 | b) MCS17 |
| --- | --- |
| c) MCS adaption |  |

From the simulation results, based on TDL channel model, it’s observed that the throughput gain can be obtained by 6 MIMO layer with fixed MCS (MCS13, MCS17) and Rank. The SNR needs to be larger than 12.5 dB under fixed rank with MCS 13 and 18.0dB with MCS 17 when 6 layer has performance gain than 4 layer.  However, fixed rank with fixed MCS can not be the case to judge if 6 layer is feasible or not since in the real deployment both rank and MCS are adapted according to actual channel environment. Take Figure a) as example, In real field, when SNR just exceeds 12.5dB, the throughput is already saturated for rank 4 and higher MCS can be scheduled to achieve higher throughput. It can not be decided if 6-layer still has better performance than 4- layer considering MCS adaption.

When MCS adaption is enabled, there is no gain of 6 MIMO layers compared with 4 MIMO layer when SNR is less than 35 dB, since higher MCS could be scheduled and correctly demodulated in the case of 4 MIMO layer, while may not be demodulated correctly in the case of 6 MIMO layers.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers. With poor Rx correlation, it requires better SNR conditions when 6 MIMO layer outperforms 4MIMO layer.

Observation 4: The SNR needs to be larger than 12.5 dB under fixed rank with MCS 13 and 18.0dB with MCS 17 when 6 layer has performance gain than 4 layer.

Observation 5: When enabling MCS adaption and fixed Rank, 6 MIMO layer has no gain over 4 MIMO layer under the condition SNR is less than 35 dB.

In the practical deployment, to adapt the dynamically changed traffic and channel state, it is more feasible to enable MCS and Rank adaption. With simulation Priority 1, typical UE antenna correlation matrix is used and there is no performance gain for 6 layer compared to 4 layer. With simulation priority 2, a much better UE Rx correlation is used and the SNR condition 35 dB is very difficult to achieve in the field when 6 layer outperforms 4 layer. From our simulation results, there is no benefit to support 6 MIMO layers for handheld UE.

Observation 6: In the practical deployment, it is more feasible to enable MCS and Rank adaption to adapt the dynamically changed traffic and channel state.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

Conclusion

This contribution discusses 6 MIMO-layer support for 6Rx. The following observations and proposal are made:

Observation 1: The SNR needs to be larger than 13.5 dB under fixed rank with MCS 13 and 19.0dB with MCS 17 when 6 layer has performance gain than 4 layer.

Observation 2: When enabling MCS adaption and fixed Rank, 6 MIMO layer has no gain over 4 MIMO layer.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers. With poor Rx correlation, it requires better SNR conditions when 6 MIMO layer outperforms 4MIMO layer.

Observation 4: The SNR needs to be larger than 12.5 dB under fixed rank with MCS 13 and 18.0dB with MCS 17 when 6 layer has performance gain than 4 layer.

Observation 5: When enabling MCS adaption and fixed Rank, 6 MIMO layer has no gain over 4 MIMO layer under the condition SNR is less than 35 dB.

Observation 6: In the practical deployment, it is more feasible to enable MCS and Rank adaption to adapt the dynamically changed traffic and channel state.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

- References

[1] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4#110-bis.

[2] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4#111.

[3] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112.

# Annex

Table 1 Feasibility Study Link-Level Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz or 100MHz |
| SCS |  | 30 |
| Antenna configuration |  | Option 1: 16Tx, 6Rx Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 1: TDLA30-5 Option 2: TDLA30-10 Option 3: TDLC300-100 |
| Rank |  | Fixed rank [4,5,6] |
| MCS |  | Option 2: MCS13, MCS17 from Table 1 Option 1: Adaptive MCS (target BLER 10%) |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | Option 1: [1+1(4L), 2+0(6L)] Option 2: [2+2(4L), 2+2(6L)] Option 3: [1+1 (4L), 2+2 (6L)] |
|  | Precoding | Random (further check) |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: Other options are not precluded. |