3GPP TSG-RAN4 Meeting #112bis	R4-2415753

# Hefei, China, Oct 11 – 18, 2024

Agenda item:			6.1.1.2.1

Source:	vivo

Title:	Discussion on UE RF power enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#112 meeting, the WF[1] has been agreed, and the remaining issue is as follows:

### Sub-topic 1-1: Approaches to enable MPR reduction for scenario 1 #### Issue 1-1-1: Approaches for scenario 1 - WF - Further evaluate the reduced MPR values with relaxation of ACLR/SEM to certain levels or waived - Other ways to reduce MPR are not precluded #### Issue 1-2-1: Clarification of BS CBW - Proposals - Proposal 1: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios. (Huawei) - WF - FFS in next meeting #### Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation - Proposals - Approach 1: UE CBW based - Option 1: The extended UE CBW based method as captured in the WF in last meeting. - Approach 2: BS CBW based - Option 2: BS CBW based inner/outer method. (Skyworks) - BS NRB instead of UE NRB. - Adding the amount of RB by which the UE RBs are shifted within the BS RBs to the UE RBstart. - The base station shall signal the extended NRB and RBstart shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR. - For margin or stricter emissions, the BS can signal a reduced NRB and RBshift. - Option 2a: the feature ‘narrower UE channel BW within wider BS bandwidth’ is specified for a UE configured with a UE-specific CHBW within a wider BS bandwidth, the said UE compliant with selected OOBE requirements applicable for the wider BS bandwidth (the cell bandwidth). (Ericsson, Nokia) - WF - Focused on extended UE CBW on common issues, which could also be applied for BS CBW based approach #### Issue 1-2-3: whether extended UE CBW edge could exceed the BS CBW edge - Proposals - Proposal 1: The extended UE CBW should not exceed the BS CBW. (Samsung, Sony, vivo, MTK, LGE) - Proposal 2: One is that there is no adjacent in-band/out-of-band co-existence issue, in which case, it is feasible that extended UE CBW edge exceeds the BS CBW edge. The other is that there exists adjacent in-band/out-of-band co-existence issue, and extended UE CBW edge cannot exceed the BS CBW edge. (ZTE) - WF - proposal 1 is agreeable for singe CC case, FFS multi-CC/BS transmitted BW case #### Issue 1-2-4: Whether and where to use IBE in the larger BS CBW - WF - Focus on how to extend IBE for usage between edges of UE CBW and extended UE CBW #### Issue 1-2-5: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE) - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson) - WF - FFS in next meeting #### Issue 1-2-6: Boundary to apply SE - WF - [SE applicability depends on the extended CBW] #### Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE - Proposals - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom) - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony) - WF - FFS in next meeting #### Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW - Proposals - Proposal 1: (Apple) - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel. - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third. - Proposal 2: (Qualcomm) - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW. - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW. - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo) - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO) - Proposal 5: (Huawei) - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication. - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency. - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK - WF - FFS with consideration of above proposals in next meeting #### Issue 1-2-9: Whether and how to consider the Asymmetrical extended CBW approach - WF - Asymmetrical extended CBW approach can be considered, FFS the details #### Issue 1-2-11: Signaling aspects - Proposals - Proposal 1: no changes of signalling are specified for the feature ‘narrower UE channel BW within wider BS bandwidth’ except possibly specification of a capability bit to indicate support of the feature. (Ericsson) - Proposal 2: Need to consider capability for extended UE CBW procedure. (LGE) - Proposal 3: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability. (Sony) - Proposal 4: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met. (vivo) - Proposal 5: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism. (CTC) - WF - To discuss the signalling aspects after sufficient evaluation of power boosting and/or MPR reduction in terms of relaxed requirements.

- Discussion

## 2.1. Scenario 1: Scenario with no adjacent in-band/out-of-band co-existence issue

Analysis on SEM relaxing

The current SEM of NR is evolved from LTE. The SEM of LTE is formulated in combination with WCDMA and regional regulatory requirements such as FCC, PHS of Japan, OFCOM and so on. For those cases where the regulatory requirements do not meet with the general mask, the “Additional spectrum emission requirement” should be followed. Therefore, the current SEM mask is not evaluated based on the strictest regulation requirement, so the level of SEM mask could also be relaxed to some extent. However, SEM is not like ACLR which is always derived based on system level coexistence studies, SEM should also consider the requirement of regional regulation, so whether the SEM could be waived completely still needs to be verified.

Proposal 1: The SEM amplitude value can be relaxed to a certain extent, but whether the SEM can be waived needs to be discussed.

The prerequisites for relaxing ACLR and SEM in Scenarios 1-1 and 1-2

For scenario 1-1, there is no adjacent in-band/out-of-band co-existence issue and only single operator in the spectrum, which mainly refers to two sub-scenarios: 1. One band is only allocated to one operator, e.g., n34 in China. 2. “co-construction and sharing by two operators”, e.g., n28 in China.

However, for the two sub-scenarios, the relaxation of ACLR and SEM of the target UE should be on the premise that there are no other bands in the frequency spectrum covered by the ACLR applying region of the target UE. In the other word, if the UE is allocated at the edge of the band and the BW of the UE is too wide to lead to the applying range of ACLR and SEM overlapping with the other adjacent bands, then the relaxation of the two requirements are not allowed. Otherwise the outside emission would not be controlled well.

For scenario 1-2, there is no adjacent in-band/out-of-band co-existence issue between the two adjacent operators. For example, there is reserved spectrum between two operators’ spectrum in the same area. In this scenario, there are two or more operators in one band but the co-existence issue could still be avoided, which should also be based on the condition that the reserved spectrum is wide enough to avoid the adjacent frequency range allocated to the other operators overlapping the applying range of ACLR and SEM of the target UE. If overlap occurs, then the relaxation should not apply, or the degree of relaxation should be limited.

Proposal 2: For scenario 1-1 and 1-2, relaxing ACLR and SEM should be on the premise that their applying range would not cover the band or spectrum range owned by another uncoordinated operator.

The effect of relaxing ACLR and(or) SEM

The figs below are the simulation results of waveform for DFT-S-OPFM and CP-OFDM. The modulation selected is QPSK and 16QAM as their outer configurations are mainly affected by ACLR and SEM. It can be seen from the simulation results that, the effect of MPR reduction by relaxing ACLR alone is limited (including a relaxation of 3/5dB on its value and the complete exclusion of the indicator). For CP-OFDM waveform, the difference between 3dB relaxation and 5dB relaxation in ACLR is more obvious than that between 5dB relaxation and ACLR waived. However, for DFT-S-OFDM waveform, there is little difference between the three levels of relaxation.

Observation 1: Relaxing ACLR only (including relaxing several dBs and waiving ACLR completely) has limited effect on MPR reduction for the outer region.

Relaxing both SEM and ACLR has more obvious effect on MPR reduction, the more the two requirements are relaxed, the greater the MPR reduction will be. For DFT-S-OFDM, most outer configurations could fall back to the level close to or equal to that of the inner region. For CP-OFDM, most outer configurations could achieve a reduction value of 0.6 to more than 1dB, but many RB allocations with big RB num still have a relatively large MPR which is far away from the level of inner configurations. However, even if the both requirements are fully waived, not all the outer configurations could fall back to the inner level, especially for the large RB num region.

Observation 2: Relaxing both SEM and ACLR has more obvious effect on MPR reduction.

Observation 3: Even if SEM and ACLR are both waived, not all the outer configurations could fall back to inner level, especially for large RB num regions.

Proposal 3: For scenario1, the reassessment of the outer MPR rather than the applicability of the inner MPR to outer region should be considered, especially for CP-OFDM.

Fig 1. DFT-S-OFDM-16QAM (spec: outer: ≤ 2; inner: ≤ 1)

Fig 2. DFT-S-OFDM-QPSK (spec: outer: ≤ 1; inner: ≤ 0)

Fig 3. CP-OFDM-16QAM (spec: outer: ≤ 3; inner: ≤ 2)

Fig 4. CP-OFDM-QPSK (spec: outer: ≤ 3; inner: ≤ 1.5)

## 2.2. Scenario 2: Narrower UE channel BW within wider BS bandwidth

Clarification of BS CBW

In the WF of #112 meeting, whether the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios is FFS. It was also agreed that for single CC case the extended UE CBW should not exceed the BS CBW, but multi-CC/BS transmitted BW case is FFS.

From the spec 38.104 it can be seen that the definition of BS channel bandwidth and BS RF bandwidth is different, BS CBW refers to the transmission bandwidth of the single NR RF carrier, while the BS RF CBW refers to the RF bandwidth of single or multiple carrier(s) within a supported operating band. In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth. However, in multiple carriers’ operation, the BS RF CBW represents the total transmission bandwidth which could be wider than the BS CBW and interferer the adjacent BS. For the BSs of the same operator, it seems that the interference between each other could be controlled by coordinating through dynamic adjustment of frequency spectrum allocation. It is not clear that whether the coordination could also be achieved between different operators in “co-construction and sharing” scenario, if so, then for multi-CC transmitted BW case, the extended UE CBW edge could exceed the BS CBW edge, but still within the corresponding operating band.

Proposal 4: If spectrum coordination between different BSs could be achieved, then multi-CC transmitted BW could exceed the BS CBW but still within the corresponding operating band.

BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink NOTE 1: The BS channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements. NOTE 2: It is possible for the BS to transmit to and/or receive from one or more UE bandwidth parts that are smaller than or equal to the BS transmission bandwidth configuration, in any part of the BS transmission bandwidth configuration. Base Station RF Bandwidth: RF bandwidth in which a base station transmits and/or receives single or multiple carrier(s) within a supported operating band NOTE: In single carrier operation, the Base Station RF Bandwidth is equal to the BS channel bandwidth.

The goal of MPR reduction in scenario2

Re-evaluating MPR of the outer region based on different extended CBW is similar to the concept that dividing the current outer MPR to several smaller categories, which should not be considered as a high priority.

First of all, RB configuration is a two-dimensional concept. Different RB configurations are related to not only RBstart, but also RBnum. Extending the edge of UE or RB allocation will change the relative relationship between the two parameters in the new bandwidth. However, extending the bandwidth or RB configuration of different UEs outward by different widths will result in different degrees of MPR reduction. If only reducing the MPR of part of the outer region but not converting all of the outer MPR to the inner level, the evaluation of the extended bandwidth will become very complicated and unnecessary.

Proposal 5: For scenario2, prioritize the applicability of the inner MPR to all the outer RB allocations, rather than further dividing the outer MPR to several smaller categories.

According to Proposal 5, asymmetric extension based on RB configuration also needs to be based on one premise: The distance of the RB allocation to the BS edge should be greater than or equal to x times RB num, x is the agreed proportion to extend on both sides of the RB allocation to ensure that the outer MPR could definitely reduce to the inner level after extension.

Proposal 6: The extension (including asymmetric and symmetric) should be based on one premise: The frequency separation of the RB allocation to the BS edge should be greater than or equal to x * RB num, x is the agreed extension proportion to guarantee the MPR reducing to inner level.

Applying range of IBE

After extending the UE CBW, the applied evaluation requirement between the edges of UE CBW and extended UE CBW should be discussed. Although the RBs in this “new guardband” could not be utilized in the actual UE, the frequency range still belongs to “in-band” concept as it is a kind of virtual bandwidth set for the UE, so IBE should be applied. Further, the general part of IBE consists of four equations and the max of them is filtered out as the final requirement:

IBE_general=max(PRB - 30 dB, )

Therefore, there are two elements to consider: NRB and ΔRB. They are both related to UE CBW and corresponding RB allocations: NRB represents the maximum transmission bandwidth configuration for each UE channel bandwidth, ΔRB is the starting frequency offset between the allocated RB and the measured non-allocated RB.

According to [2], the NRB of the IBE requirement refers to the total number of RB in the uplink, which should be based on the actual UE bandwidth indicated by the NW, and should not change with the virtual extension of UE CBW edge. Similarly, the calculation of ΔRB should also be limited within the actual UE CBW. The IBE requirement at the edge of actual UE CBW could be used as a reference to evaluate the radiation power level of the entire extended part. The illustration is as follows:

Fig 5. IBE applying range after bandwidth expansion

Based on the assumption above, there is another situation need to be discussed: Full RB allocation cases.

Since there are no remaining unallocated RBs, there is no IBE requirement in the full-RB allocation of the UE CBW. Therefore, how to evaluate the emission level between edges of UE CBW and extended UE CBW in this situation is a problem. Two methods can be applied: 1) Not evaluating the emission level in that area. 2) Using IBE requirement to evaluate the emission level, with ΔRB set to 0, indicating that the frequency difference between the allocated RB and the UE CBW edge is 0. The second method is preferable. The original requirements for this area used to be ACLR and SEM, so IBE is a kind of relaxation, and the nearest radiation leakage outside the UE CBW also needs to be evaluated to control the transmit modulation quality of the UE.

Proposal 7: IBE should be used between edges of UE CBW and extended UE CBW. And the IBE of this part should be equal to that of the original UE CBW edge.

The change of the spurious emission boundary

According to the WF of meeting #111, not relaxing the level of spurious emission requirements has become a consensus. However, the application range of SE still should be altered with the edge of the UE CBW shifting and the change of ACLR and SEM application range. Otherwise, SE would become the limiting factor for MPR reduction, and the application range will partially overlap with SEM, and it would not adhere to the definition and motivation of SE evaluation in 3GPP: Spurious emissions are emissions which are caused by unwanted transmitter effects but exclude out of band emissions unless otherwise stated. In ITU regulation, the definition of SE is similar: Spurious emissions include harmonic emissions, parasitic emissions, intermodulation products and frequency conversion products but exclude out-of-band emissions.

Proposal 8: The application range of SE should be altered with the shifting of the edge of the UE CBW and the change in the application range of ACLR and SEM.

The start point of ΔfOOB & integral region of OOBE

ACLR and SEM would be applicable from the edge of extended CBW, and the integral region and the boundary of OOBE should be based on UE CBW. From one side, SE need to be evaluated from the edge of SEM and according to ITU-R regulations, the SE should apply from 250% of the centre frequency of the “necessary bandwidth”. If the integral region is based on extended UE CBW, the boundary of OOBE would be extended to not only farther than current 3GPP requirements but also disobey the ITU requirement.

For example, in Figure 1(a), the integral CBW of OOBE is equal to the UE CBW. The SE applies from the edge of SEM and the frequency spacing with centre of UE CBW is 200% * UE CBW+5M, which meets with the requirement of ITU. However, in Figure 1(b), the integral CBW of OOBE is equal to the extended UE CBW. And the SE applies from 300% * UE CBW+5M, which is too much far away from the UE centre especially for the wider UEs and the protection effect of UE co-existence would be weaken a lot.

Proposal 9: For OOBE, the integral region should be based on UE CBW, and the starting point should be applied from edge of the extended UE CBW.

(a)

(b)

Fig 6. Comparison of two integral bandwidths

The BW basis of MPR evaluation

As has been analyzed above, the extension of the UE bandwidth would cause the change of the applying range and calculation method of the emission requirements. For IBE, as the NRB should be based on the actual UE bandwidth indicated by the NW, the level from the edge of UE CBW to BS CBW should be equal to that at the UE edge if MPR evaluation is based on BS CBW, which means that the IBE horizontal line should be drawn from the UE edge to the BS edge.

At the same time, the SE applying start point should also be shifted outside ACLR & SEM applying range. So, if the OOBE requirements are based on the BS CBW rather than the extended UE CBW, then the applying range of SE would be altered to a rather far region. For example, if the BS CBW is very wide (e.g.,100MHz), while the UE CBW is very narrow (e.g., 10 MHz) and is assigned to a location 10MHz away from the lower edge of the BS. Then the OOBE on the higher frequency side of the UE will move out 80 MHz, and the corresponding SE should also be applied from 950% * UE CBW + 5 MHz of the center frequency of the "necessary bandwidth", which is too far from the center of the terminal and much higher than the 200% requirement.

Observation 4: If the OOBE requirements are based on the BS CBW, the applying range of SE would be altered to a rather far region and incompatible with the ITU requirement.

On the other hand, powerboosting has been studied in R18 and some progress has been made, such as defining the concept of ‘enhanced power inner allocation region’ within the original inner region to delineate the range where PCMAX,L and PCMAX,H can be simultaneously boosted by ΔPPowerBoost without using any frequency shaping methods.

However, enhanced inner region is based on the relationship between RB configuration and UE CBW, and could not be utilized to the BS CBW without any verification. If a new inner, outer and edge allocation definition for MPR based on BS CBW is adopted then the two mechanisms are contradictory and the enhanced inner region would become a tricky range. The UE needs to determine the type of its RB allocation twice, i.e. relative position in BS CBW and relative position in UE.

Even if the concept could also be expanded to the BS CBW, then MPR of some outer cases of the UE would not only reduce to the inner level, but also to the powerboosting level. Especially for cases with UE full allocation, e.g., (DFT-S-OFDM)-PC3-QPSK, even the output power of the UE used to be limited by the 1dB outer MPR, it could now transmit at a higher power of 23+ΔPpowerboost (1dB) as it is allocated at the enhanced inner region of the BS CBW. It means that the output power could be boosted by as high as 2dB with the OOBE requirements shifting outside the BS, which could cause a much higher interference to the other UEs.

Observation 5: The new RB region definition based on BS CBW would contradict the powerboosting feature in the ‘enhanced power inner allocation region’ of R18.

Proposal 10: MPR should be evaluated based on the UE CBW(or extended UE CBW) rather than the BS CBW.

The judgement of shifting UE edge

The new mechanism of the UE to reduce MPR should be discussed, and there are two options could be considered as a starting point. The corresponding indication from NW to the UE is different and the mapping to MPR will be also decided by different sides:

1) The NW directly indicates that the UE's MPR reduces from outer to inner level:

The NW would judgement the deployment environment, when the RB configuration belongs to outer region and the UE (or RB allocation) is far enough away from the BS edge, the NW should send signaling to indicate MPR reduction and which side of the UE bandwidth could be extended virtually. Then UE would refer to the inner MPR requirement instead. At the same time, the corresponding emission requirements could be tested based on the agreed extended UE CBW, which is proportional to the UE CBW.

2) The NW indicates the extended bandwidth to the UE, and the mapping of the extended BW to MPR is decided by the UE:

The NW should send signaling to indicate a virtual extended UE CBW to UE, then UE would re-map to the MPR table according to the information. At the same time, the corresponding emission requirements could be tested based on the extended UE CBW. If after extending the UE CBW is wide enough to guarantee the RB configuration inside it could locate at the inner region then the MPR would reduce to the inner level, otherwise the MPR would remain at the outer level.

Comparing option1 to option2, option1 is preferred, as there is no need to redefine a new RB region according to the relationship of original UE CBW and the extended CBW. The frequency interval of the original UE CBW and the BS CBW could be easily judged by the NW as it decides the RB allocation of the UEs, the process is more concise. And when the interval is far enough then a simple indication including changing the MPR to an inner level instead could be sent to the UE, and the response could be quicker when the RB allocation changes. At the same time, there is no need to overextend the UE CBW anymore and the emission performance could be promised better.

Proposal 11: The frequency interval for UE(or RB allocation) and BS edge should be judged by the NW. When the interval is wide enough, the NW could indicate which side of the UE CBW should be extended.

- The UE would refer to the inner MPR requirement instead.

- At the same time, the corresponding emission requirements could be tested based on the agreed virtual extended UE CBW, which is proportional to the UE CBW.

- Conclusion

Proposal 1: The SEM amplitude value can be relaxed to a certain extent, but whether the SEM can be waived needs to be discussed.

Proposal 2: For scenario 1-1 and 1-2, relaxing ACLR and SEM should be on the premise that their applying range would not cover the band or spectrum range owned by another uncoordinated operator.

Observation 1: Relaxing ACLR only (including relaxing several dBs and waiving ACLR completely) has limited effect on MPR reduction for the outer region.

Observation 2: Relaxing both SEM and ACLR has more obvious effect on MPR reduction.

Observation 3: Even if SEM and ACLR are both waived, not all the outer configurations could fall back to inner level, especially for large RB num regions.

Proposal 3: For scenario1, the reassessment of the outer MPR rather than the applicability of the inner MPR to outer region should be considered, especially for CP-OFDM.

Proposal 4: If spectrum coordination between different BSs could be achieved, then multi-CC transmitted BW could exceed the BS CBW but still within the corresponding operating band.

Proposal 5: For scenario2, prioritize the applicability of the inner MPR to all the outer RB allocations, rather than further dividing the outer MPR to several smaller categories.

Proposal 6: The extension (including asymmetric and symmetric) should be based on one premise: The frequency separation of the RB allocation to the BS edge should be greater than or equal to x * RB num, x is the agreed extension proportion to guarantee the MPR reducing to inner level.

Proposal 7: IBE should be used between edges of UE CBW and extended UE CBW. And the IBE of this part should be equal to that of the original UE CBW edge.

Proposal 8: The application range of SE should be altered with the shifting of the edge of the UE CBW and the change in the application range of ACLR and SEM.

Proposal 9: For OOBE, the integral region should be based on UE CBW, and the starting point should be applied from edge of the extended UE CBW.

Observation 4: If the OOBE requirements are based on the BS CBW, the applying range of SE would be altered to a rather far region and incompatible with the ITU requirement.

Observation 5: The new RB region definition based on BS CBW would contradict the powerboosting feature in the ‘enhanced power inner allocation region’ of R18.

Proposal 10: MPR should be evaluated based on the UE CBW(or extended UE CBW) rather than the BS CBW.

Proposal 11: The frequency interval for UE(or RB allocation) and BS edge should be judged by the NW. When the interval is wide enough, the NW could indicate which side of the UE CBW should be extended.

- The UE would refer to the inner MPR requirement instead.

- At the same time, the corresponding emission requirements could be tested based on the agreed virtual extended UE CBW, which is proportional to the UE CBW.

- References

- R4-2414441, WF on power domain enhancements, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #112.

- R4_080678 UE In-band Emission Requirements, Qualcomm Europe, RAN4#46-bis.