3GPP TSG-RAN4 Meeting #112bis	R4-2415758

Hefei, China, Oct 11 – 18, 2024

Agenda item:			6.6.2

Source:	vivo

Title:	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

- Introduction

In RAN4#112 meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

Topic #2: Methods for reducing the number of UE Rx chains #### Issue 2-1-1: Applicability and clarification on the scope Agreement: - The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain - The two non-contiguous CCs is still under the current DL non-contiguous CA framework - Each fragment is treated as an individual CC - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz. - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks]

- Discussion

1. Reference architecture

OPEN ISSUES Recommended WF in #112 meeting: - The existing requirements for DL NCCA was specified based on partially-shared Rx architecture according to TR36.823. - FFS on following options for reducing number of Rx chains for DL non-contiguous CCs: - Option 1: Only the “fully shared Rx chain architecture” needs to be studied - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz - Other options are not precluded - Companies are encouraged to provide baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study - FFS on whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier.

In TR 36.823, it seems that the mainstream Rx architecture in the current spec is partially shared, which is the basis for specifying the corresponding RX requirements. And the fully separately architecture is considered as a complementary version. As has been discussed in the last meeting and recognized by most companies that fully-shared architecture is the most valuable research object in the fragmented carrier scenario, including the sharing of LO, LNA, analogy filter and so on.

Proposal 1: Reducing number of Rx chains for DL NC CCs should be based on fully-shared RX chain.

However, whether to share the baseband equipment such as BB filter is reserved as FFS in #112 meeting. the main problem is the in-gap interferer falls in the middle of the two target CCs and could not be filter out by the filter itself, especially when the analogy filter in the front-end RF chain also turns to being shared.

There is a similar scenario in TR 38.858: UE-UE co-channel inter sub-band model, which is to analyze the impact of interference that occurs between two UEs in close proximity, operating on adjacent sub-bands within the same channel. This interference occurs when UL transmission of an aggressor UE interferes with the DL reception of a victim UE in the same channel.

For receiver in-channel adjacent sub-band selectivity performance, no rejection/attenuation due to RF/BB filtering is assumed for legacy UE and only the selectivity performance of the FFT operation is studied. In the scenario, a new requirement specified as ‘in-channel adjacent subband selectivity’ is a measure of a receiver’s ability to receive an NR signal on its assigned downlink subband in the presence of an interference power on the adjacent uplink subband. 33 dB is agreed for FR1 in RAN4 for in-channel adjacent subband selectivity considering FFT operation.

In the related discussion paper [2] it can be seen that two aspects will impact the selectivity: receiver RF non-linearities leading to IM products from the RX RBs falling into the RX sub-bands and the FFT suppression of the interference level. According to the simulation results the FFT selectivity and frequency/time errors would not be the primary factor impacting the UE-UE interference, the dominating factor would be the RF front-end linearity. Therefore, even if there are time synchronization errors and frequency offset between the interferer and the target signal, FFT suppression rarely has a negative effect. If similar conclusions are extended to FC, it could be foreseen that FFT could provide a relatively ideal inhibition for the in-gap interferer. If the orthogonality between the jamming signal and the target CC is better, the suppression effect will also be better.

Observation 1: FFT could provide enough rejection ability toward the interference in the gap.

Proposal 2: BB filter sharing could also be considered in the shared RX chain.

However, if a single FFT is expected to cover the fragmented carriers, the SCS of the two target CCs should be aligned. Take SCS=30KHz and 15KHz as an example, if taking a data length corresponding to a symbol of 30 kHz for FFT, the frequency domain information at 15 kHz will be missing. Conversely, if taking a data length corresponding to a symbol of 15 kHz for FFT, the frequency domain information at 30 kHz will be aliased. Therefore, prioritizing the study of separate FFT for each fragmented carrier is better.

Observation 2: A single FFT could be expected to cover the fragmented carriers only on the premise of SCS alignment of the two CCs.

Proposal 3: Prioritize the study of separate FFT for each fragmented carrier.

2. PSD difference between carriers of co-located adjacent channel operators

OPEN ISSUES FFS on following options in next meeting - Option 1: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure) - For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level - equal PSD between two co-location operators could be assumed from gNB transmisstion point of view for evaluation purpose, as the starting point - The PSD difference from UE reception point of view could vary as per the different antenna type, antenna pointing direction, etc between operators - Option 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case are considered - Other options are not precluded

According to the analysis of [3] and [4], ‘co-location’ only involves the site (or the mast) of the two operators is shared, and not the infrastructure. According the premise even if the two operators’ gNBs are assumed to transmit at maximum power level, it also seems that only the path decay from gNB to UE is the same, but the PSD emitted from the two gNBs cannot be considered equal as the difference of antenna type, direction and so on.

However, the study of the PSD difference and DL performance degradation cause by the interferer should be based on the worst condition, e.g., the covering area of the two operators’ antenna is minimal. At the same time, the reference port for PSD difference evaluation should be based on UE receiver rather than BS side as it should be a receiving requirement.

Observation 3: “Co-location” deployment could only guarantee the same path attenuation, but not the same transmit power considering different infrastructure between operators.

Proposal 4: DL performance deterioration should be based on the worst-case scenarios with biggest PSD difference, i.e., different antenna and minimum sector coverage area.

Proposal 5: The reference port for evaluating PSD difference between carriers of co-located adjacent channel operators should be based on the UE receiver.

The reason of controlling the power imbalance between the two non-contiguous CCs is that: When the power of some carriers is significantly higher or lower than that of other carriers, this difference will directly affect the processing ability of the receiving signal. The relatively stronger power carrier may mask or interfere with the weaker power carrier, making it difficult for the latter to be correctly received and demodulated. But as for the in-gap signal which is kind of different, as it is the interference in the aggregated BW, so the power of the signal should be controlled as low as possible to confirm the correct receiving of the useful CCs. Therefore, discussion of “Determine a reasonable level of the power spectral density difference between carriers of co-located adjacent channel operators” could be simplified to “determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs”.

Proposal 6: Study on PSD difference should be simplified to “Determine a reasonable upper level of the PSD difference of the in-gap signal compared to the two non-contiguous CCs”.

Furthermore, as the final judgement information is the PSD difference, then the evaluation of in-gap interference is not only related to the power but also related to the bandwidth. If only the power difference could be controlled under a certain level in the inter-operator co-located BS scenario, then the sharing of the RX chains should be based on the premise that the BW of the in-gap signal should not be narrower than a certain range so that a relative low PSD level could be guaranteed. If the PSD difference limit is met, then the BW of the blocker signal should not be too wide, or else the output power will be too high and affect the reception performance.

Proposal 7: The bandwidth of the in-gap interference should also be limited within a certain range.

3. Signaling aspects

With the RX chains switching to shared architecture, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs. This would result in a wider variety of CA configurations supported by the UE. There are two kinds of methods for a UE to inform the network of new CA configurations it can support with adjusted RF requirements:

- The new CA configuration could be reported to the NW together with other default supported CA configuration, as they are related to the stable hardware capabilities. And it could be reported through a relative static mechanism, such as RRC signaling, as the current technology does. The RX chain state and CA configuration indication could be dynamically switched according to the environment.

- The new CA configuration could be separately reported to the NW when or after the RX chains switching to be shared, which is more dynamic but complex. When the RX chains could be switched to or after switching to the shared architecture, the UE reports to the NW that the new CA configuration it supports with the sharing architecture, and then the NW could send a new CA configuration to indicate to the UE that there are more CCs to be aggregated.

Proposal 8: The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration.

4. Fall back issue

When the prerequisite of sharing is not met, the UE should fall back to partially-shared or fully separated architecture. The trigger factor for falling back may be that the output power (or PSD) of the in-gap interferer is too high to maintain sharing with good receiving performance.

With the RX chains switching to being shared, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs. This would result in a wider variety of CA configurations supported by the UE. However, these new CA configurations could only be indicated with shared RX chains. Also, when the RX chains fall back to separate architecture, the CA configuration should also switch to default group with less CCs combined.

Observation 4: The CA configurations supported by shared and separate RX architecture are not always the same, some CA configurations are supported only in the shared state.

Proposal 9: If the separate architecture does not support the CA configuration in shared state, then the fallback of the Rx chain will also require corresponding changes to the CA configuration.

- Conclusion

Proposal 1: Reducing number of Rx chains for DL NC CCs should be based on fully-shared RX chain.

Observation 1: FFT could provide enough rejection ability toward the interference in the gap.

Proposal 2: BB filter sharing could also be considered in the shared RX chain.

Observation 2: A single FFT could be expected to cover the fragmented carriers only on the premise of SCS alignment of the two CCs.

Proposal 3: Prioritize the study of separate FFT for each fragmented carrier.

Observation 3: “Co-location” deployment could only guarantee the same path attenuation, but not the same transmit power considering different infrastructure between operators.

Proposal 4: DL performance deterioration should be based on the worst-case scenarios with biggest PSD difference, i.e., different antenna and minimum sector coverage area.

Proposal 5: The reference port for evaluating PSD difference between carriers of co-located adjacent channel operators should be based on the UE receiver.

Proposal 6: The study on PSD difference should be simplified to “Determine a reasonable upper level of the PSD difference of the in-gap signal compared to the two non-contiguous CCs”.

Proposal 7: The bandwidth of the in-gap interference should also be limited within a certain range.

Proposal 8: The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration.

Observation 4: The CA configurations supported by shared and separate RX architecture are not always the same, some CA configurations are supported only in the shared state.

Proposal 9: If the separate architecture does not support the CA configuration in shared state, then the fallback of the Rx chain will also require corresponding changes to the CA configuration.

- References

- R4-2414457, WF on fragmented DL carriers study, MediaTek Inc. RAN4#112.

- R4-2300544, Analysis of UE FFT selectivity performance for SBFD, Ericsson, RAN4#106.

- R4-2411310, Views on UE RF architecture and NW deployment assumption for fragmented carriers, Samsung, TELUS, Bell mobility, RAN4#112.

- R4-2412278, Operator’s initial views on FR1 fragmented carriers study, RAN4#112.