3GPP TSG-RAN4 Meeting #112bis	R4-2415759

Hefei, China, Oct 11 – 18, 2024

Agenda item:			6.6.3

Source:	vivo

Title:	Impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

In RAN4#112 meeting, the WF[1] has been agreed as follows, this contribution will discuss about the remaining open issues.

Topic #2: Methods for reducing the number of UE Rx chains #### Issue 2-1-1: Applicability and clarification on the scope Agreement: - The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain - The two non-contiguous CCs is still under the current DL non-contiguous CA framework - Each fragment is treated as an individual CC - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz. - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks]

- Discussion

According to the WF, the goal of the DL fragmented carrier study has been clarified, which is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain, e.g., from separate RF chains per CC to shared RF chain. Based on sharing of the related RF devices, the RX requirements and DL performance would be impacted.

ΔRIBNC

Sometimes an UL CC may cause strong interference especially considering a relative weak duplexer isolation and a very narrow frequency separation between the UL CC and the DL CC, as analyzed in [2]. In some cases when the UL and DL signals are both allocated at the adjacent edge of their spectrum, the frequency separation between the UL CC and the DL CC would be as narrow as the gap bandwidth between the transmit band and the receive band. Therefore, the impact of UL CCs on the UE receiver performance should be considered carefully. For example, the relaxation of REFSENSE should be further studied in this situation, e.g., ΔRIBNC. When the RX chain is switched to the shared mode, the filtering effect is relatively reduced compared to the separated architecture, so the DL CC closer to the UL side is more severely affected as more TX leakage would fall in the RX chain and the current REFSENSE may not be easily met.

However, ΔRIBNC does not need to be re-evaluated in TDD bands as it represents the interference caused by UL to DL, so the requirements to be re-evaluated are not exactly the same in TDD/FDD/SDL bands.

Observation 1: The requirements needed to be re-evaluated are not exactly the same in TDD/FDD/SDL bands.

Proposal 1: The ΔRIBNC should be re-evaluated in the shared RX chain architecture especially when the frequency separation between the UL CC and the DL CC is too narrow.

- No need to consider TDD bands.

ACS and in-band blocking

First of all, assuming the “shared RX chain” in the objective means in one RX chain there would not be dedicated filter for each CC, which not only presents a great challenge to the bandwidth and filtering effect of the shared filter, but also introduces some interference signal of the adjacent channel operators as the interference is allocated in the in-gap position of the two aggregated CCs and also within the filter passband. Also, a strong in-gap interfering signal may cause unnecessary power consumption and circuit saturation, it may have severe impacts into the dynamic range of the UE as well.

As mentioned in [3], currently the ACS, IBB and NBB requirements are developed based on separate RF chain assumption, the in-gap requirements are the same as out-of-gap requirements, and they are too stringent for the in-band cases. If the power of the in-gap interference could not be controlled to a relative low level contiguously, then the two RF requirements should be relaxed.

Proposal 2: According to the interference of the in-gap signal, ACS, IBB and NBB should be re-evaluated.

Image rejection of the in-gap interference

In the RX sharing scenario, when the LO is shared between two non-contiguous CCs to down-convert the two CCs simultaneously, the single LO could not be set in the middle of every CC but in the middle of the aggregated CCs instead. The strong interference from the in-gap signal would cause another important problem: the image issue. As shown in Fig 1, when a strong interfering signal is located inside the sub-block gap, and the CCs are allocated asymmetric, considering the LO location the image of the interference may fall into one of the CC. When the interference power is significant, the power of the image can also be high enough to degrade the effective demodulation of the CC especially when it is weak. Hence, the image interference should be evaluated based on the weaker CC.

Observation 2: The image of the in-gap interferer may overlap the target CC and degrade the demodulation performance.

Proposal 3: The image impact of the in-gap interferer should be evaluated based on the throughput of the weaker CC.

In the analysis of [4], there is a similar but identical problem in LTE intra-band C CA due to imbalanced LO. The image of one carrier would overlap another carrier after down-conversion and cannot be removed by any filtering scheme, since both carriers contain useful information. Therefore, the image rejection (IRR) is given as the ratio between the squared amplitudes of the original component and the image signal. IRR =25dBc with PSD difference =6dB is specified to guarantee the receiving performance. IRR could not be directly verified but can provide a reference for demodulation testing of the CC that is interfered by the image signal, so the source of the image could include not only another CC but also the in-gap blocker. Effective demodulation means the image rejection performance at a particular PSD difference is acceptable.

Observation 3: IRR could not be directly verified but can provide a reference for demodulation testing of the CC that is interfered by the image signal, so the source of the image of IRR could include not only another CC but also the in-gap blocker.

Fig 1. Contrast between separate architecture and shared architecture

Below is the calculation formula of IRR[5]: Clearly, if there was no amplitude imbalance present (ΔA = 0) between the I and Q branches of LO, the power ratio would be infinite. Due to the finite amplitude balance, a fundamental image rejection IRRfund is present. Furthermore, if the SCC (or any interference signal at the frequency area reserved for SCC) has ΔP dB larger received power compared to PCC, the total image rejection is decreased by the amount of ΔP. Likewise, reception of SCC is corrupted is PCC has larger reception power.

From the analysis above it can be seen that, IRR is related to the fundamental frequency dependent quadrature accuracy of the receiver analog parts, LO signal generation etc. If the LO is rather ideal, IRRfund would be higher. At the same time, if power imbalance of the two CC (i.e., ΔP) is very small, IRR would also be high enough to guarantee the rejection effect to image signal. Expanding to FC, if the PSD difference between the in-gap interference and the target CC could be controlled as low as possible, and the precision of the LO is very ideal the image rejection would no longer be an issue.

Observation 4: If the PSD difference between the in-gap interference and the target CC is sufficiently low, and the LO is ideal enough, then image rejection will no longer be an issue.

25dBc could be set as a starting point as it could guarantee the demodulation performance of the target CC currently, and further evaluation could be tested based on the assumption, a higher value is expected as the LO is better now.

Proposal 4: Expanding the concept of IRR to limit the image signal which is not only from another CC but also from the in-gap blocker, 25dB could be set as a starting point for further evaluation.

- Conclusion

Observation 1: The requirements needed to be re-evaluated are not exactly the same in TDD/FDD/SDL bands.

Proposal 1: The ΔRIBNC should be re-evaluated in the shared RX chain architecture especially when the frequency separation between the UL CC and the DL CC is too narrow.

- No need to consider TDD bands.

Proposal 2: According to the interference of the in-gap signal, ACS, IBB and NBB should be re-evaluated.

Observation 2: The image of the in-gap interferer may overlap the target CC and degrade the demodulation performance.

Proposal 3: The image impact of the in-gap interferer should be evaluated based on the throughput of the weaker CC.

Observation 3: IRR could not be directly verified but can provide a reference for demodulation testing of the CC that is interfered by the image signal, so the source of the image of IRR could include not only another CC but also the in-gap blocker.

Observation 4: If the PSD difference between the in-gap interference and the target CC is sufficiently low, and the LO is ideal enough, then image rejection will no longer be an issue.

Proposal 4: Expanding the concept of IRR to limit the image signal which is not only from another CC but also from the in-gap blocker, 25dB could be set as a starting point for further evaluation.

- References

- R4-2414457, WF on fragmented DL carriers study, MediaTek Inc. RAN4#112.

- R4-115584, Impact of non-contiguous intra-band CA on the UE receiver requirements, Ericsson, ST-Ericsson, RAN#61.

- RP-241257, Views on SI of Fragmented Carriers in Rel-19, Huawei, HiSilicon. RAN#104.

- R4-103677, Image Rejection in intraband carrier aggregation, Nokia, Nokia Siemens Networks, RAN4# AH#4.

- R4-102976, Considerations on retuning for deactivated SCell and related implications, Nokia Corporation, RAN4 #56