3GPP TSG-RAN WG4 Meeting # 112bis 	R4-2415762

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the UE parameter for 14800 to 15350 MHz

Agenda Item:	6.2.4.2

Document for:	Approval

- Introduction

In the last meeting, the parameters for 8GHz were finished but the parameters for 15GHz are still pending. In this contribution, we provide our views on the UE parameters.

- Discussion

- Channel bandwidth

For FR2, 200MHz is mandatory and 400MHz is optional, so it is suggested that use 200MHz as a typical value.

Proposal 1: 200MHz is used as typical channel bandwidth for 14800 to 15350 MHz in the LS.

- Signal bandwidth

For the 4GHz and 8GHz LS, the following agreement is used in the LS:

- Quote formula of RBs * SCS without number of RBs

It is better to follow the same rule in the 15GHz

Proposal 2: For the signal bandwidth in the 15GHz, it is suggested to quote formula of RBs * SCS without number of RBs

- UE form factor

o	FR1-like UE:

- Option 1: 1TX assumption with omnidirectional antenna

- Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion.

- Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA.

  - Companies to clearly state what value of N and G assumed.

o	FR2-like UE:

- FR2-likUE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels criterion.

In [1], two types of UE are agreed for co-existence evaluation:

It is quite controversial to decide the form factor of UE in this new frequency at this stage. In TR 38.820, similar discussion was happened but, in the end, both FR1-like and FR2-like are marked as possible in FR3. It is inefficient to take too much time on this topic.

Observation 1: It is inefficient to try to down-selection from different UE form factor since there no commercial UE is designed in this new frequency range at this stage.

To save effort, we prefer keep the door open and provide two set of parameters in the LS

Proposal 3: The UE parameters for both FR1-like and FR2-like are provided in the reply LS for 14800-15300MHz.

- Maximum output power

In current 24GHz band, the min peak EIRP is 22.4 dBm with 1x4 panel assumption. For 15GHz, 1x2 is assumed which means the radiated power should be 6 dB lower due to the loss in PA array and beamforming. However, it is possible to get better PA gain at 15GHz to compensate the loss in antenna array, so for FR2-like UE, we prefer keep same output power as 24GHz band. For FR1-like UE, 23dBm still can be the baseline, and other higher power class is not precluded.

Proposal 4: For FR1-like UE, the maximum output power is 23 dBm in the reply LS and other higher power can be captured in TR just like 8GHz; For FR2-like UE, the maximum output power is same as n258 PC3.

- Noise figure

For 8GHz, 13dB is agreed as noise figure which can also be used for FR1-like UE in 15GHz. For FR2-like UE, 10~11 dB was used in Rel-15 for EIS discussion, we prefer use same 11 dB in 15GHz.

Proposal 5: For FR1-like UE, the noise figure is 13 dB; For FR2-like UE, the noise figure is 11 dB.

- Conclusion

Proposal 1: 200MHz is used as typical channel bandwidth for 14800 to 15350 MHz in the LS.

Proposal 2: For the signal bandwidth in the 15GHz, it is suggested to quote formula of RBs * SCS without number of RBs

Observation 1: It is inefficient to try to down-selection from different UE form factor since there no commercial UE is designed in this new frequency range at this stage.

Proposal 3: The UE parameters for both FR1-like and FR2-like are provided in the reply LS for 14800-15300MHz.

Proposal 4: For FR1-like UE, the maximum output power is 23 dBm in the reply LS and other higher power can be captured in TR just like 8GHz; For FR2-like UE, the maximum output power is same as n258 PC3.

Proposal 5: For FR1-like UE, the noise figure is 13 dB; For FR2-like UE, the noise figure is 11 dB.

- References

- R4-2414284 WF for 15GHz UE parameters, RAN4#112, Qualcomm