3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415763

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the impact of CW node

Agenda Item:	6.21.2.1

Document for:	Approval

- Introduction

In the last meeting, the concern of the impact from CW to NR was arise, and in this contribution, we provide more analysis for this issue.

- Discussion

In current NR requirement, the narrowband blocking is defined as several dB relaxation of REFSENS when a CW interference is located at near the NR CBW.

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n70, n71, n72, n74, n75, n76, n85, n100, n101, n106 | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |
|  |  | dB | 18 | 16 | 13 | 14 | 16 | 16 | 16 |
|  | Puw (CW) | dBm |  | -55 |  |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1a, Table 7.3.2-1b and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. NOTE 5: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |  |  |

However, in [1], it is observed that the REFSESN relaxation is calculated based on the interference after the ACS suppression, which is actually not suitable for AIoT since the CW is inside the NR CBW.

Observation 1: The narrowband blocking takes the ACS into consideration which is not suitable for AIoT, whose CW is inside the NR CBW when in-band operation is assumed.

To further dig out the impact of CW when it is located inside the NR CBW, several simulation results are provided in [2]

Figure 1 Evaluation results in [2]

Based on the results, we can find that when the CW is orthogonal with NR subcarrier, i.e., α=0, the interference is much lower and also easier to be removed after FFT.

Observation 2: The impact of CW can be minimized by putting them in the locations where is orthogonal with NR subcarrier.

Proposal 1: To reduce the impact of CW, the CW in AIoT system should be orthogonal with NR subcarrier.

Even though the impact of CW can be mitigated as described above, it is still necessary to make sure the power of CW will not exceed dynamic range of NR receiver, otherwise the receiver cannot work at all due to the blocking of component. The worst case should be CW in DL spectrum since the CW will direct hit the NR UE receiver, and the Figure 2 shows the CW power distribution under different UE location assumptions.

Figure 2 CW power distribution at UE

Currently, the maximum input power of -25 dBm for most FDD band. When all UEs are inside the factory, there are about 15% UE cannot handle such interference. However, this should be the worst case which is quite unrealistic, and in most cases the interference will not exceed the dynamic range of NR UE.

Observation 3: In most cases, the CW interference will not exceed the receiver dynamic range of NR UE.

- Conclusion

In this contribution, we provide our analysis on the impact of CW.

Observation 1: The narrowband blocking takes the ACS into consideration which is not suitable for AIoT, whose CW is inside the NR CBW when in-band operation is assumed.

Observation 2: The impact of CW can be minimized by putting them in the locations where is orthogonal with NR subcarrier.

Proposal 1: To reduce the impact of CW, the CW in AIoT system should be orthogonal with NR subcarrier.

Observation 3: In most cases, the CW interference will not exceed the receiver dynamic range of NR UE.

- References

- R4-071090, Text proposal to TR 36.803 – Section 7.6 Blocking, Nokia, RAN4#43bis

- K. Fors, E. Axell, S. Linder and P. Stenumgaard, "On the Impact of CW interference on 5G NR," 2019 International Symposium on Electromagnetic Compatibility - EMC EUROPE, Barcelona, Spain, 2019, pp. 1049-1054.