3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415764

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the co-existence of AIoT

Agenda Item:	6.21.2.2

Document for:	Approval

- Introduction

In the last meeting, the simulation assumptions for AIoT co-existence evaluation were updated [1], and following cases for D2T2 need to be evaluated:

| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
| --- | --- | --- | --- | --- |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
|  |  |  | NR UL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-1 (O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |
| 6-2 (O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | Reader->NR UL | R2D |
|  |  |  | NR UL->device | R2D |

For time being, we only bring the simulation results and analysis for D2T2 in this meeting.

- Discussion

  - Simulation results and analysis

- Case 5 CW in UL spectrum Device->NR UL

| Device type | Device leakage level | UE distribution | Average throughput loss | 5% Cell edge throughput loss |
| --- | --- | --- | --- | --- |
| Device 1 | 25 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.0% | 0.0% |
|  |  | 100% indoor | 0.0% | 0.0% |
|  | 16 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.0% | 0.0% |
|  |  | 100% indoor | 0.0% | 0.0% |
| Device 2a | 25 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.0% | 0.0% |
|  |  | 100% indoor | 0.02% | 0.0% |
|  | 16 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.05% | 0.0% |
|  |  | 100% indoor | 0.11% | 0.0% |

For this case, the victim is NR BS which is outside of the factory and the aggressor power of device will suffer from the penetration loss, so the impact is ignorable.

- Case 5 CW in UL spectrum NR UL->Reader

| Device type | UE distribution | SNR loss / dB |  |  |
| --- | --- | --- | --- | --- |
|  |  | @5% | @50% | @95% |
| Device 1 | 100% outdoor | 0.0 | 0.0 | 0.0 |
|  | 10% indoor | 10.82 | 4.33 | 1.69 |
|  | 100% indoor | 23.6 | 27.13 | 20.64 |
| Device 2a | 100% outdoor | 0.0 | 0.0 | 0.0 |
|  | 10% indoor | 4.38 | 4.56 | 1.81 |
|  | 100% indoor | 14.01 | 20.31 | 19.67 |

In this case, the aggressor is NR UL of UE, so the location of UE has a significant impact on the results. When all UEs are outside of the factory, the SINR degradation of reader can be ignored but only if there is UE inside the factory, the SINR degradation will increase obviously.

- Case 5 CW in UL spectrum Reader->NR UL

| UE distribution | Average throughput loss | 5% Cell edge throughput loss |
| --- | --- | --- |
| 100% outdoor | 11.0% | 39.4% |
| 10% indoor | 13.7% | 0.0% |
| 100% indoor | 17.9% | 0.0% |

This case is the most problematic one. Even though the NR UEs are all outside of the factory, the throughput loss is still much larger than the 5%. There are mainly two reasons:

- The readers always transmit with maximum power while the NR UE needs to follow the power control, which make the interference is quite large compared to the intra-system interference.

- The legacy NR IBE is used for the emission of D2T2 reader, which is a quite pessimistic assumption.

It is noted that for 10% indoor and 100% indoor cases, the throughput loss at cell edge is 0%. This is because current simulation assumption will lead to a quite high pathloss when UE is inside the factory, and 5% UE throughput is already equal to 0 although without the interference from AIoT system.

- Case 5 CW in UL spectrum NR UL->Device

| Device type | UE distribution | SNR loss / dB |  |  |
| --- | --- | --- | --- | --- |
|  |  | @5% | @50% | @95% |
| Device 1 | 100% outdoor | 0.0 | 0.0 | 0.0 |
|  | 10% indoor | 4.68 | 5.74 | 3.24 |
|  | 100% indoor | 14.55 | 18.38 | 27.86 |
| Device 2a | 100% outdoor | 0.0 | 0.0 | 0.02 |
|  | 10% indoor | 4.69 | 5.71 | 3.54 |
|  | 100% indoor | 14.72 | 18.61 | 28.68 |

- Case 6 CW in DL spectrum Device->NR DL

| Device type | Device leakage level | UE distribution | Average throughput loss | 5% Cell edge throughput loss |
| --- | --- | --- | --- | --- |
| Device 1 | 25 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.13% | 0.27% |
|  |  | 100% indoor | 0.81% | 2.74% |
|  | 16 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.47% | 2.51% |
|  |  | 100% indoor | 3.28% | 13.13% |
| Device 2a | 25 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 0.53% | 3.30% |
|  |  | 100% indoor | 4.02% | 16.95% |
|  | 16 dBc | 100% outdoor | 0.0% | 0.0% |
|  |  | 10% indoor | 1.57% | 7.07% |
|  |  | 100% indoor | 12.14% | 61.45% |

Compared to the CW in UL spectrum, the interference from device to NR is a little bit higher when the CW is in the DL spectrum if there are UEs inside the factory. This is because when CW is in the DL spectrum, the NR UE that inside the factory will receive interference without help of penetration loss. However, under the 10% UE indoor assumption, the throughput loss is still smaller than the performance metric of 5% no matter for cell average or cell edge.

- Case 6 CW in DL spectrum NR DL->Reader

| Device type | UE distribution | SNR loss / dB |  |  |
| --- | --- | --- | --- | --- |
|  |  | @5% | @50% | @95% |
| Device 1 | 100% outdoor | 0.26 | 0.59 | 0.48 |
|  | 10% indoor | 0.20 | 0.60 | 0.41 |
|  | 100% indoor | 0.23 | 0.60 | 0.40 |
| Device 2a | 100% outdoor | 0.04 | 0.27 | 0.4 |
|  | 10% indoor | 0.04 | 0.27 | 0.4 |
|  | 100% indoor | 0.03 | 0.31 | 0.38 |

For this case, the aggressor is NR BS which is outside of factory, and the interference is also not significant due to the penetration loss.

- Case 6 CW in DL spectrum Reader->NR UL

| UE distribution | Average throughput loss | 5% Cell edge throughput loss |
| --- | --- | --- |
| 100% outdoor | 11.1% | 31.68 |
| 10% indoor | 12.23% | 0% |
| 100% indoor | 18.85% | 0% |

- Case 6 CW in DL spectrum NR UL->Device

| Device type | UE distribution | SNR loss / dB |  |  |
| --- | --- | --- | --- | --- |
|  |  | @5% | @50% | @95% |
| Device 1 | 100% outdoor | 0.0 | 0.0 | 0.0 |
|  | 10% indoor | 4.45 | 5.27 | 1.32 |
|  | 100% indoor | 14.71 | 18.56 | 23.96 |
| Device 2a | 100% outdoor | 0.0 | 0.0 | 0.0 |
|  | 10% indoor | 4.36 | 5.54 | 1.43 |
|  | 100% indoor | 14.65 | 18.75 | 24.04 |

For the two cases above, although the rules how to choose the reader may be different from the cases when CW in the UL spectrum, from statistic perspective, they are still similar.

Based on the analysis above, we have following observations:

Observation 1: The penetration loss paly an import role in the co-existence evaluation, and only if the NR UE exist inside the factory, significant performance degradation will appear in both NR system and AIoT system.

Observation 2: Even though with the help of penetration loss, the D2T2 reader still bring obvious interference to NR UL due to the poor assumption of emission and lack of power control.

Proposal 1: The following cases can be concluded as no co-existence issue:

| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
| --- | --- | --- | --- | --- |
|  |  |  | NR UL->reader | D2R |
|  |  |  | NR UL->device | R2D |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
| 6-1 (O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | NR UL->device | R2D |
| 6-2 (O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |

  - SINR threshold and outage probability

Based on the analysis in 2.1, the SINR degradation of several cases are larger than 1 dB. Following the rule in [1], SINR threshold that corresponding to the 10% BLER is needed to judge whether these cases have co-existence issue.

Recall that in LTE, the SNR of REFSENS is derived by using link level simulation with QPSK under AWGN. In the last meeting, it is also agreed that the AWGN is used in the REFSENS of LP-WUS which also use OOK waveform [2].

Observation 3: The AWGN was used to derive the SNR in both LTE/NR and LP-WUS.

Following the similar rule, the AWGN can also be used in AIoT LLS.

Proposal 2: AWGN is used in the link level simulation to derive the SINR threshold for R2D and D2R.

Based on the AWGN, several curves for R2D and D2R are provided below

Figure 1 SNR vs BLER for R2D and D2R

Obviously, the SNR at 10% BLER varies under different simulation assumptions, it is better to make sure the companies are on the same page before the discussion of the SINR threshold and outage probability.

Proposal 3: The baseline LLS assumption should be aligned in the group before the discussion of the outage probability.

  - Emission assumption for D2T2 reader

As mentioned in 2.1, the D2T2 reader will interfere NR UL significantly and one reason here is that the IBE of legacy NR UE is reused. Since the reader always transmit with maximum output power, a better allowed emission level compared to NR UE is more reasonable.

Proposal 4: Further study the feasible ACLR assumption for intermediate UE.

- Conclusion

In this contribution, we provide our analysis on the co-existence between NR and AIoT.

Observation 1: The penetration loss paly an import role in the co-existence evaluation, and only if the NR UE exist inside the factory, significant performance degradation will appear in both NR system and AIoT system.

Observation 2: Even though with the help of penetration loss, the D2T2 reader still bring obvious interference to NR UL due to the poor assumption of emission and lack of power control.

Observation 3: The AWGN was used to derive the SNR in both LTE/NR and LP-WUS.

Proposal 1: The following cases can be concluded as no co-existence issue:

| 5-1 | D2T2-A2-legacy UE only outdoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
| --- | --- | --- | --- | --- |
|  |  |  | NR UL->reader | D2R |
|  |  |  | NR UL->device | R2D |
| 5-2 | D2T2-A2-legacy UE indoor | R2D: UL CW2D and D2R: UL | Device->NR UL | D2R |
| 6-1 (O) | D2T2-B-legacy UE only outdoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |
|  |  |  | NR UL->device | R2D |
| 6-2 (O) | D2T2-B-legacy UE indoor | R2D: UL CW2D and D2R: DL | Device->NR DL | D2R |
|  |  |  | NR DL->reader | D2R |

Proposal 2: AWGN is used in the link level simulation to derive the SINR threshold for R2D and D2R.

Proposal 3: The baseline LLS assumption should be aligned in the group before the discussion of the outage probability.

Proposal 4: Further study the feasible ACLR assumption for intermediate UE.

- References

- R4-2414304 WF on Co-existence study for ambient IoT, CMCC, RAN4#112

- R4-2414309 WF on LP-WUS UE RF requirements, vivo, RAN4#112

- Simulation results