3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415765

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT BS

Agenda Item:	6.21.3.1

Document for:	Approval

- Introduction

In the last meeting, the following requirement is FFS:

- FFS whether following requirements are needed or not:

- TX

  - Output power dynamic (RE power control dynamic range, Total power dynamic range)

  - Some requirements like Transmit ON/OFF power

  - Transmitter transient period

  - IBE

  - Transmitter intermodulation

- RX

  - Dynamic range

  - In-channel selectivity

  - In-band blocking

  - Narrow-band blocking

  - Out-of-band blocking

  - Receiver intermodulation

  - Narrowband intermodulation

  - Receiver intermodulation

In this contribution we provide our views on these requirements.

- Discussion

- Output power dynamic (RE power control dynamic range, Total power dynamic range)

The power dynamic range is a requirement to reflect the performance of power amplifier and coverage, which should be a basic requirement for BS.For NB-IoT, this requirement is defined and is related to power boosting. So this requirement should be defined but may need to studied based on the OOK waveform.

- Transmit ON/OFF power/Transmitter transient period

These two requirements is only defined for TDD band in current BS spec, so for the AIoT which is work in the FDD band, these two requirements are not needed.

- IBE

Currently, the co-existence study assume the AIoT BS only support AIoT system, so the IBE is not needed unless both  NR and AIoT are supported by BS.

- Transmitter intermodulation

This requirement mainly defined for co-located BS scenario which doesn’t happen based on current AIoT assumption, so this requirement is also not needed.

Based on the analysis above:

Proposal 1: The following Tx requirement is not needed for AIoT BS

  - Transmit ON/OFF power/Transmitter transient period

  - IBE

  - Transmitter intermodulation

- RX dynamic range and blocking related requirement

Rx dynamic and blocking related requirements are defined to guarantee the receiver performance when different interferences exist, and it is better to keep them.

- In-channel selectivity

Since currently individual RF hardware is assumed for AIoT BS, this requirement may not be needed

Proposal 2: The following Rx requirement is not needed for AIoT BS

  - In-channel selectivity

- Conclusion

In this contribution, we provide our view on AIoT BS RF requirement.

Proposal 1: The following Tx requirement is not needed for AIoT BS

  - Transmit ON/OFF power/Transmitter transient period

  - IBE

  - Transmitter intermodulation

Proposal 2: The following Rx requirement is not needed for AIoT BS

  - In-channel selectivity

- References

- R4-2414305 WF on impacts of A-IoT on RF requirements, RAN4#112, Huawei