3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415766

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the RF requirement for AIoT device

Agenda Item:	6.21.3.2

Document for:	Approval

- Introduction

In the last meeting, the following requirement is FFS:

- FFS whether following requirements are needed or not:

- TX

  - Maximum output power

  - EVM

  - Transmit OFF power for device 1/2a

  - Transmit time mask

  - Minimum output power

  - Frequency error for device 2a

  - In band emissions (IBE) for device 2a

  - Carrier leakage for device 2a

  - Occupied bandwidth

  - Transmit intermodulation

- RX

  - Maximum input power

  - Receiver intermodulation

  - Rx spurious emission

  - ACS

  - ACSC

  - In-band blocking

  - Out-of-band blocking

In this contribution we provide our views on these requirements.

- Discussion

- Transmit OFF power for device 1/2a/Minimum output power

As we discussed in the last meeting, the device is incapable to control its power and the output power mainly depends on the incoming CW power, so all these requirements seem meaningless.

- Maximum output power

Honestly, the device cannot control its power but the maximum output power still meaningful since we should ensure the reflection loss is under control. Similar to RFID, this requirement can be defined as the power under a certain distance between device and the test probe.

- Transmit time mask

This requirement is to guarantee the device can change between on and off in a specific time to avoid the interference to other device or reader. Since the AIoT system also need control its inventory process, we prefer keep this requirement.

- EVM/Frequency error for device 2a

These two requirements are related to the signal quality, which is also needed for AIoT system. However, the EVM is not suitable for OOK waveform.

- In band emissions (IBE) for device 2a/ Occupied bandwidth

The IBE and occupied bandwidth can avoid the interference between device. Unless there is only one reader is activated, we prefer keep these requirements.

- Carrier leakage for device 2a

Currently we don’t assume the device is capable to large frequency shift, so the carrier leakage is not needed.

Based on the analysis above:

Proposal 1: The following Tx requirement is not needed for AIoT device

  - Transmit OFF power for device 1/2a

  - Minimum output power

  - Carrier leakage for device 2a

- Maximum input power

For NR UE, this requirement is to ensure the UE can work normally when it is close to the BS to avoid the incoming wanted signal is exceed the receiver dynamic range. For device, this requirement is also meaningful and can provide guidance for real deployment.

- ACS/ACSC

In co-existence study, we assume no ACS for the device, so the RF requirement should also following this assumption otherwise the co-existence results will be meaningless.

Since currently individual RF hardware is assumed for AIoT BS, this requirement may not be needed

- Rx spurious emission

This requirement is from regulatory and should be kept.

- In-band blocking\Out-of-band blocking

These two requirements is to guaranteed the receiver performance when a interference with large power exist. In our understanding, the device is cheep and has low capability, it is not needed force device can still work under such extremely bad situation.

Proposal 2: The following Rx requirement is not needed for AIoT device

  - ACS/ACSC

  - In-band blocking\Out-of-band blocking

- Conclusion

In this contribution, we provide our view on AIoT device RF requirement.

Proposal 1: The following Tx requirement is not needed for AIoT device

  - Transmit OFF power for device 1/2a

  - Minimum output power

  - Carrier leakage for device 2a

Proposal 2: The following Rx requirement is not needed for AIoT device

  - ACS/ACSC

  - In-band blocking\Out-of-band blocking

- References

- R4-2414305 WF on impacts of A-IoT on RF requirements, RAN4#112, Huawei