3GPP TSG-RAN WG4 Meeting #112bis	R4-2415779

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.23.2.1

Source:	 vivo

Title:	Discussions on LP-WUS system parameters

Document for:	 Approval

- Introduction

Issue 1-1-2: Performance metric for Rx RF requirements

Agreement:

- 1% MDR for evaluation [and core requirements]

- FFS on the testability issue.

Issue 1-1-3: Performance metric for Demodulation requirements

Agreement:

- [1% or 0.1%] FAR as assumption for MDR result calibration.

Issue 1-1-6: Specify FR2 LP-WUS RF requirements

Agreements:

- Wait to RAN1 progress on FR2 LP-WUS work.

Issue 1-2-1: Channel raster for LP-WUR

Agreements:

  - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS.

  - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS

Agreements:

  - Leave RAN1 to make decision.

  - RAN4 can discuss assumption of RBs under 15kHz SCS for simulation

Issue 1-2-5: Whether need to define CBW for LP-WUR?

Agreements:

  - No need to define BW for LP-WUR

In RAN4#112 meeting, the following agreements were reached [1]:

This contribution further discusses LP-WUS system parameters.

- LP-WUS UE RF

2.1 Performance metric

Last meeting, 1% MDR has been concluded as performance metric for LP-WUS evaluation, however few companies also discussed a potentially relaxed MDR as RF requirements. In our understanding, to align with RAN1 analysis and RAN4 simulation outcome, the performance metric as 1% MDR for RF requirements is a reasonable way to go. The testing time and testability issue can be further discussed.

Proposal 1: Confirm 1% MDR for RF core requirements of LP-WUS.

2.1 Number of RBs for 15kHz SCS LP-WUS

The number of RBs for 15kHz SCS LP-WUS is still FFS in RAN1. Last RAN4 meeting, RAN4 confirmed that final number RBs for 15kHz SCS LP-WUS will be RAN1 decision, but RAN4 could make decision as assumption for simulation evaluation.

Based on the analysis of 30kHz LP-WUS ACS [3], 4 RBs is potentially needed to support LP-WUR tolerate the same level of adjacent channel interference as MR. For 15kHz SCS, to ensure 5MHz operation of LP-WUS, 11 RBs of LP-WUS at 15kHz SCS is a reasonable assumption.

Therefore, we suggest RAN4 select 11 RBs for 15kHz LP-WUS RF performance evaluation. The analysis can be shared to RAN1 if it is helpful to RAN1 for making decision on number of PRBs.

Proposal 2: RAN4 select 11 RBs LP-WUS with 15kHz SCS as simulation assumption for RF performance evaluation, the outcome can be shared to RAN1 for consideration if needed.

2.1 FR2 work for LP-WUS

Way Forward

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support

- Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2.

- Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis.

  - Reuse the FR1 design when possible

- No additional guidance in RAN#105 for RAN4 work and TU allocation

In last RAN plenary meeting, FR2 is confirmed as scope of Rel-19 LP-WUS/WUR WI [2].

The signal design would reuse FR1 as much as possible, however, RAN1 still needs some analysis to confirm the waveform design for FR2.

Regarding RAN4 FR2 RF requirements, there is no further guidance and additional TU allocated for RF work. Therefore, previous RAN4 agreement still valid that FR1 work is prioritized. To make progress on FR2 work, similar simulation analysis work should also be done.

Observation 1: FR2 LP-WUS is within the scope of WI. There is no further guidance from RAN on RAN4 work.

Proposal 3: To make progress on FR2 LP-WUS, RAN4 should perform the similar simulation analysis as FR1 to specify RF requirements.

- Conclusion

In this contribution, we share our views on LP-WUR system parameters:

Proposal 1: Confirm 1% MDR for RF core requirements of LP-WUS.

Proposal 2: RAN4 select 11 RBs LP-WUS with 15kHz SCS as simulation assumption for RF performance evaluation, the outcome can be shared to RAN1 for consideration if needed.

Observation 1: FR2 LP-WUS is within the scope of WI. There is no further guidance from RAN on RAN4 work.

Proposal 3: To make progress on FR2 LP-WUS, RAN4 should perform the similar simulation analysis as FR1 to specify RF requirements.

# References

- R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

- RP-242361, Way Forward on LP-WUS for FR2, vivo, RAN#105

- R4-2415780, Further discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112bis