3GPP TSG-RAN WG4 Meeting #112bis	R4-2415780

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.23.2.2

Source:	 vivo

Title:	Further discussions on LP-WUS REFSENS, ASCS and ACS

Document for:	 Approval

- Introduction

Based on agreements for REFSENS, ASCS and ACS achieved in previous meetings [1] [2], this contribution shares our further simulation results for LP-WUS REFSENS, ASCS and ACS requirements.

- LP-WUS UE RF

  - Link-level simulation for LP-WUS target SNR

In LTE/NR, SNR = -1 dB is widely used to calculate REFSENS and this value is derived from 95% throughput at link-level simulation under AWGN, 1/3QPSK, no HARQ, 12,5 symbol per subframe as shown in [3][4].

Figure 1 Target LTE SNR simulation with 5, 10 and 20 MHz bandwidths (throughput in Mbps); one receiving antenna [3].

Figure 2 Target NR SNR simulation with 5, and 10 MHz bandwidths [4].

According to the agreed simulation assumption in [1], and agreements in [2]:

Issue 1-1-2: Performance metric for Rx RF requirements

Agreement:

- 1% MDR for evaluation [and core requirements]

- FFS on the testability issue.

Issue 2-1-1: Channel model to specify LP-WUS RF requirements

Agreement:

  - Follow typical RF requirements approach, AWGN should be selected.

Issue 2-1-2: Target SNR simulation condition

Agreement:

  - Follow similar approach of MR NR, no repetition should be used when simulate target SNR.

Issue 2-1-3: Whether RAN4 should conclude a target SNR first

Agreement:

- Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first.

Issue 2-1-4: Target SNR value

Agreements:

  - RAN4 further discuss target SNR value for LP-WUS

  - FFS single SNR for OOK-based and OFDM-based receiver

Our simulation results for LP-WUS SNR with AWGN model at 1% MDR is presented in Table 1.

Table 1 SNR@1% MDR and @5%MDR with 4bit ADC under AWGN (no repetition)

| Modulation | Chip rate | Channel structure | Receiver type | SNR@1% MDR |
| --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 8bit | Envelope | -3.0 dB |
|  |  |  | OFDM | -9.3 dB |
|  |  | 16bit | Envelope | -2.9 dB |
|  |  |  | OFDM | -9.2 dB |
| OOK-4 | M = 1 | 8bit | Envelope | -3.1 dB |
|  |  | 16bit |  | -3.0 dB |
|  | M = 2 | 8bit |  | -4.2 dB |
|  |  | 16bit |  | -4.0 dB |
|  | M = 4 | 8bit |  | -2.5 dB |
|  |  | 16bit |  | -2.3 dB |
| OOK-4 | M = 1 | 8bit | OFDM | -9.3 dB |
|  |  | 16bit |  | -9.1 dB |
|  | M = 2 | 8bit |  | -9.5 dB |
|  |  | 16bit |  | -9.2 dB |
|  | M = 4 | 8bit |  | -6.5 dB |
|  |  | 16bit |  | -6.2 dB |

Observation 1: The worst case is OOK-4, M=4, payload = 16bit with envelope-based receiver, and the SNR at 1% MDR is about -2.3 dB.

Observation 2: With the same modulation, the SNR performance of OFDM-based receiver is always 4~6dB better than Envelope-based receiver.

In last RAN4 meeting, there are two different views on potential target SNR value for LP-WUS/WUR, i.e., single SNR value (similar to MR approach) or two different values for Envelop-based and OFDM-based receivers. For NR, the baseband SNR performance at different modulation is different, but single target SNR as -1dB is used to evaluate the REFSENS requirements. For LP-WUR, it is suggested that single target SNR value is used to specify REFSENS for different receiver types.

Proposal 1: Define a single target SNR based on worst case of LLS outcome, i.e., Envelope receiver, [-2]dB.

  - ASCS requirements

Using the agreed simulation parameters in [1], the ASCS link level simulation results are summarized in Table 2.

Table 2 SNR performance degradation for ASCS (without guard RBs inside)

| modulation | Chip rate | payload | receiver | Freq | Freq offset | Filter order | ASCS SNR @ 1% MDR (with in-band NR) | target SNR@1%MDR (no in-band NR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOK-1 | M = 1 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -2.9 dB | -3.0 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.7 dB | -7.7 dB |
|  |  |  |  |  |  | 5th | -8.0 dB | -8.0 dB |
|  |  |  |  |  | 20 ppm | 3rd | -6.0 dB | -6.0 dB |
|  |  |  |  |  |  | 5th | -6.2 dB | -6.2 dB |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -7.7 dB | -7.7 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.1 dB | -5.1 dB |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -9.2 dB | -9.2 dB |
|  |  |  |  |  |  | 5th | -9.0 dB | -9.0 dB |
|  |  |  |  |  | 10 ppm | 3rd | -7.5 dB | -7.5 dB |
|  |  |  |  |  |  | 5th | -7.5 dB | -7.5 dB |
|  |  |  |  |  | 20 ppm | 3rd | -5.0 dB | -5.0 dB |
|  |  |  |  |  |  | 5th | -5.0 dB | -5.0 dB |
| OOK-4 | M = 4 | 16bit | Envelope | 900 MHz | The results for different offset are similar | The results for different filter are similar | -2.3 dB | -2.3 dB |
|  |  |  |  | 2.6 GHz |  |  |  |  |
|  |  |  |  | 3.5 GHz |  |  |  |  |
|  |  |  | OFDM | 900 MHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -6.1 dB | -6.1 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -5.9 dB | -5.9 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  | 2.6 GHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -5.5 dB | -5.5 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -3.8 dB | -3.8 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  | 3.5 GHz | 0 ppm | 3rd | -6.2 dB | -6.2 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 10 ppm | 3rd | -5.1 dB | -5.1 dB |
|  |  |  |  |  |  | 5th |  |  |
|  |  |  |  |  | 20 ppm | 3rd | -2.2 dB | -2.2 dB |
|  |  |  |  |  |  | 5th |  |  |

In the Table above, we don’t differentiate the performance under different guard RB (0/1/2) and the reason here is we observe that the benefit of guard RB is not significant for ASCS.

Observation 3: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

It is also observed that the frequency offset has no impact on Envelope receiver but significant impact on OFDM receiver, the reason is that the frequency offset impacts a lot the coherence detection of receiver rather than the interference power level.

Based on above simulation, to ensure the implementation, max 1 RB is needed as guard RB for ASCS requirements.

Proposal 2: Max 1 RB is required for ASCS requirements.

Proposal 3: RAN4 can define the ASCS test parameters as following in Table 3

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1] at each side |
| MDR |  | 1% |

  - ACS requirements

For ACS, the simulation is performed under the similar condition of MR, i.e., a high LP-WUS SNR condition of 14dB+LP-WUS REFSENS. Based on the discussions in RAN4#112 meeting, most companies believe that the interference condition of ACS for LR and MR is nearly the same, so in our simulation we use the same interference level as MR for LP-WUS ACS evaluation.

Our simulation results were provided last meeting in [5], and we identified that OOK4 M=4 is the worst case of ACS. In the last meeting, we provide the simulation results for the required RB numbers under different increased interference power level on top of LR REFSENS which vary from 20dB to 33dB. Since the MR and LR are implemented in the same UE, the common understanding is that they should suffer from same interference condition, and from this perspective, it is unnecessary for LR to endure same increased interference power ratio, as shown in the figure below:

Figure 3: Illustration for the gap between MR and LR under same interference

Due the worse REFSENS of LR, the required interference power only about 17.7~22.7 dB on top of the LR REFSENS, which also depends on the LR SNR and noise figure.

Observation 4: under same interference power level of MR, the required ACS of LR will not exceed 23 dB.

We also provide updated simulation results focused on OOK4 M=4 case. The following figure shows the relationship between guard RB, SNR and NF for OOK-4 M=4 waveform.

Figure 4： Relationship between NF, SNR and guard RB number

Based on the simulation analysis, for envelop-based receiver NF = 16 dB, SNR = -2 dB, 4 Guard RB is enough for LR ACS.

In TR 38.869, the reciprocal mixing is mentioned which may have impact on the LR when a high-power level adjacent interferer signal that is present at antenna mixes with the oscillator phase noise sideband, as shown below.

Figure 5: Reciprocal mixing for ACI case in TR 38.869

In the agreed simulation assumption [1], phase noise model is not considered. However, we figure out some performance degradation with consideration of poor phase noise performance of LP-WUR, so in this contribution we provide updated results with phase noise being added into link-level simulation.

In [6], the phase noise model for WUR in Wi-Fi is provided. For simplicity, we use the phase noise power at 5MHz offset and integrate it over the entire bandwidth of the WUS, and the influence of guard RB is also considered.

Figure 6: phase noise model of LP-WUS for WiFi in IEEE 802.11

Based on the above phase noise model, we use the following value for simulation:

Table 4 Phase noise used in the simulation under different offset

| Offset | 5MHz | 5MHz +1RB | 5MHz +2RB | 5MHz +3RB | 5MHz +4RB |
| --- | --- | --- | --- | --- | --- |
| Phase noise | -90.5dBm/Hz | -91.1dBm/Hz | -91.7dBm/Hz | -92.2dBm/Hz | -92.7dBm/Hz |

After adding the phase noise, the simulation results of ACS are shown in Figure 6.

Figure 6: updated ACS results with consideration of phase noise model (LP-WUS level is fixed REFSENS+14dB)

The results show the under 23 dB higher interference power level (at this condition, the interference level is same as MR condition, as illustrated in Figure 3), 4 guard RBs is still enough when the phase noise impact of reciprocal mixing is considered.

Observation 5: After taking the impact of reciprocal mixing of phase noise, 4 guard RB is still enough for WUR under 23 dB higher interference power (same as MR interference level).

With above analysis and observations, we believe the ACS requirements should consider worst case of the receiver. The dynamic range of LP-WUR is not that large as MR, to ensure the coverage of sensitivity then the capability of LP-WUR to tolerate high interference may be limited. RAN4 should consider a different ACS requirement for this new type of receiver.

Observation 6: The ACS requirements for LP-WUS should be defined within 15~23dB, with number of guard RB as 1~4.

Proposal 4: LP-WUS ACS requirements can be specified as 23dB with ACS Guard RB as 4.

  - REFSENS requirements

The following agreements for REFSENS have been agreed last meeting [2]:

Issue 2-2-2: Baseline architecture for OOK-based LP-WUS

Agreement:

  - use zero-IF receiver as a baseline architecture for envelop based LP_WUR

Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS

Agreements:

  - Wait for RAN1’s conclusion on the OFDM-based LP-WUR.

Issue 2-2-4: whether different NF for OOK-based and OFDM-based

Agreement:

  - Differentiate NF for OOK-based and OFDM-based receiver.

Noise figure is a key value to specify LP-WUS REFSENS, based on the study phase outcome in TR 38.869 and agreed baseline architecture last meeting, NF as 15dB for OOK-based receiver and 11dB for OFMD-based receiver can be considered.

Proposal 5: The following NF can be considered:

For OOK based WUR

- [15] dB

For OFDM based WUR

- [11] dB

- Conclusion

In this contribution, we share our views on LP-WUR REFSENS, ASCS and ACS:

Observation 1: The worst case is OOK-4, M=4, payload = 16bit with envelope-based receiver, and the SNR at 1% MDR is about -2.3 dB.

Observation 2: With the same modulation, the SNR performance of OFDM-based receiver is always 4~6dB better than Envelope-based receiver.

Proposal 1: Define a single target SNR based on worst case of LLS outcome, i.e., Envelope receiver, [-2]dB.

Observation 3: The benefit of guard RB in ASCS case is minor, and the impact of adjacent NR signal on the WUS sensitivity is negligible.

Proposal 2: Max 1 RB is required for ASCS requirements.

Proposal 3: RAN4 can define the ASCS test parameters as following in Table 3.

Table 3: Test parameters for LP-WUS ASCS

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 5-100MHz |
| LP-WUS Power | dBm | REFSENS |
| In-band Pinterferer | dBm | NR signal, same as LP-WUS |
| Guard RB between LP-WUS and NR |  | [1] at each side |
| MDR |  | 1% |

Observation 4: under same interference power level of MR, the required ACS of LR will not exceed 23 dB.

Observation 5: After taking the impact of reciprocal mixing of phase noise, 4 guard RB is still enough for WUR under 23 dB higher interference power (same as MR interference level).

Observation 6: The ACS requirements for LP-WUS should be defined within 15~23dB, with number of guard RB as 1~4.

Proposal 4: LP-WUS ACS requirements can be specified as 23dB with ACS Guard RB as 4.

Proposal 5: The following NF can be considered:

For OOK based WUR

- [15] dB

For OFDM based WUR

- [11] dB

# References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo, RAN4#111

- R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

- R4-080125, TP Receiver Requirements: REFSENS, ACS and blocking, Ericsson, RAN4#46

- R4-1709564, Preliminary SNR values for NR UE REFSENS, Huawei, Hisilicon, NR ad-hoc #3, 2017

- R4-2412059, Discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112

- IEEE802.11-17/0326r0, WUR Phase Noise Model Follow-Up