3GPP TSG-RAN WG4 Meeting #112bis		R4-2415782

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.23.2.4

Source:	 vivo

Title:	Discussions on LP-WUS Testability

Document for:	 Approval

- Introduction

This contribution shares further views on LP-WUS RF requirements testability issues.

- LP-WUS UE RF testability

In last RAN4 meeting, it was agreed that RAN4 should further discuss high-level approach on how to achieve LP-WUS performance verification, in WF [1].

There are two different views, one way is to verify the RF performance based on full process of LP->MR wakeup procedure, this is close to real working scenario of LP-WUR, but drawback is testing time consuming and realizability of test system. Another way around is a simple approach to verify the LP-WUS performance based on test mode without wake-up MR each time, but just record the LP-WUS outcome and calculate MDR.

In our understanding, if the MR is wake-up each time based on normal procedure of LR to MR, then to meet the statistic of test results, the overall testing time will be unacceptable. More important, the LR to MR wakeup can be a functionality test, rather than RF performance testing. For REFSENS and other RX requirements, we just need to verify LR output results via test mode, no wake-up action is needed at each OOK-ON signal/sequence, during testing period.

Observation 1: LR to MR wakeup can be a functionality test.

Observation 2: For REFSENS and other RX requirements, we just need to verify LR output results via test mode, no wakeup action is needed at each OOK-ON signal, during testing period.

Therefore, before discussing detailed test procedures, RAN4 should align first the general guidance that the MDR counted without waking up the MR each time would be sufficient for the LP-WUS test in terms of verifying the RF requirements.

Proposal 1: For LP-WUS RF performance verification, the MDR can be counted without waking up the MR each time.

The testability issue of LP-WUS BLER/MDR should be well resolved otherwise the requirements can not be properly verified, according to the LP-WUR mode, there could be two general directions to go, e.g., MR idle mode and MR connection mode.

For MR connection mode, the testability issue would be easy to resolve, the tested wake-up signal “ACK/NACK” results can be reported to gNB via MR, then the BLER/MDR can be calculated accordingly. It should be noted that MR should not receive and detect LP-WUS signal during testing, otherwise there might be a risk that the UE could cheat gNB by using MR detection results.

However, for MR idle mode, the REFSENS will be verified under a no close-loop condition, a new UE test mode is needed. Besides, each detection results as “ACK/NACK” in LR should be recorded and then MR can be wakened up after LP-WUR testing period finished, then the detection results can be feedback to gNB via MR.

Most likely, RAN1 will specify a UE capability for LP-WUR supports idle and/or connection mode. Then based on this UE capability, the test conditions can be set as different.

Proposal 2: Based on UE capability, the two options can be generally considered for LP-WUS RF verification:

- Option1: Using MR connection mode, but MR should not receive and detect LP-WUS signal. The miss detection ratio can be calculated based on “ACK/NACK” results of LP-WUR which is feedback to gNB by MR.

- Option2: Using MR idle mode, there is no LP-WUS feedback to gNB during testing, a new UE test mode is needed. MR should be waken-up after testing of all LP-WUS signal and connected to gNB to feedback the LP-WUS detection results.

If a single approach is selected, then idle mode testing for REFSENS should be prioritized.

Proposal 3: If single test condition is selected, then idle mode testing for RF requirements is prioritized.

- Conclusion

In this contribution, we share our views on LP-WUR testability issue.

Observation 1: LR to MR wakeup can be a functionality test.

Observation 2: For REFSENS and other RX requirements, we just need to verify LR output results via test mode, no wakeup action is needed at each OOK-ON signal, during testing period.

Proposal 1: For LP-WUS RF performance verification, the MDR can be counted without waking up the MR each time.

Proposal 2: Based on UE capability, the two options can be generally considered for LP-WUS RF verification:

- Option1: Using MR connection mode, but MR should not receive and detect LP-WUS signal. The miss detection ratio can be calculated based on “ACK/NACK” results of LP-WUR which is feedback to gNB by MR.

- Option2: Using MR idle mode, there is no LP-WUS feedback to gNB during testing, a new UE test mode is needed. MR should be waken-up after testing of all LP-WUS signal and connected to gNB to feedback the LP-WUS detection results.

Proposal 3: If single test condition is selected, then idle mode testing for RF requirements is prioritized.

# References

- R4-2414309, WF on LP-WUS UE RF requirements, vivo, RAN4#112

- R4-2414909, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1