3GPP TSG-RAN WG4 Meeting #112bis	R4-2415783

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.23.3

Source:	 vivo

Title:	Further discussions on LP-WUS BS RF requirements

Document for:	 Approval

- Introduction

In last RAN4 meeting, the BS RF requirements was discussed, and several potential options have been captured in the WF [1]. This contribution shares our views on LP-WUS BS RF requirements.

- LP-WUS BS RF requirements

Regarding manufacture declaration on LP-WUS, there are two options in WF [1].

## Manufacture declaration on LP-WUS

WF

FFS on the following options:

- Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

- Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

Similar to NB-IoT, a minimum power boosting level is valuable to be specified to ensure that at least the min value can be achieved when this feature is optionally declared by BS. If no value will be specified, then actually RAN4 do not need to discuss this power boosting feature at all.

Proposal 1: RAN4 should define a min power boosting level in core specification which is based on optional manufacture declaration.

Proposal 2: RAN4 further discuss whether the min power boosting level could be different value for different CBW.

## Concept of LP-WUS dynamic range/power boosting

WF

FFS on the following options:

- Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs.

  - Option 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

  - Option 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs).

- Adopt following definition rather than introduce EPRE ratio.

  - Option 3: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

For LP-WUS power boosting definition, the simulation assumption in RAN1 is always the EPRE ratio between LP-WUS and NR signal at different CBW. Therefore, we think the LP-WUS power boosting requirement should clearly reflect this aspect no matter which concept is finally adopted in RAN4.

Proposal 3: No matter which Option is selected as RAN4 LP-WUS power boosting definition, the requirement should consider the assumed case in RAN1 evaluation, i.e., the assumption in RAN1 of 3dB or 6dB means the EPRE ratio.

## Whether to preclude small CBW for consideration of LP-WUS power boosting

WF

- FFS on following options:

  - Option 1: Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting.

    - Option 1-1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting.

    - Option 1-2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded.

    - Option 1-3: other option does not excluded.

  - Option 2: No need to preclude any CBW, only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement.

Given there would be potential large power degradation on NR signal at 5MHz/10MHz CBW, thus LP-WUS with 30kHz at those CBW can be precluded. However, if 11 PRBs is selected for 15kHz SCS case, then potentially these two CBW can also support power boosting.

In short, LP-WUS power boosting can preclude 5MHz and 10MHz case for 30kHz SCS operation. But, for 15kHz SCS LP-WUS, the applicable CBW should be further discussed and analyzed based on decisions of LP-WUS RBs.

Proposal 4: For 30kHz SCS LP-WUS operation, power boosting within 5MHz and 10MHz CBW can be precluded. For 15kHz SCS LP-WUS operation, no CBW should be precluded.

## On minimum value for LP-WUS power boosting

WF

- FFS on the following options:

  - Option 1: If power boosting will be defined as EPRE ratio, use 3dB as minimum requirement.

  - Option 2: If power boosting will be defined as dynamic range, based on existing specification for OOK-1 waveform, the core requirement for the dynamic range should be limited to 3dB.

  - Option 3: Not to introduce minimum value for LP-WUS power boosting.

For min value for LP-WUS power boosting, how much power can be boosted for LP-WUS depends on analyzed impacts on NR performance. Based on our preliminary analysis of NR throughput performance impacts, the NR throughput loss is minor, under in-band LP-WUS boosting condition.

Part of the reason is that the LP-WUS REFSENS is potentially worse than NR, at least OOK-based receiver would be several dBs worse, thus the in-band LP-WUS operation is somehow making NR signal operation at a higher SNR condition, and the NR power degradation do not have big impacts on NR DL throughput performance.

Proposal 5: RAN4 consider 3dB as a minimum requirement for LP-WUS power boosting requirement.

## Transmitted signal quality

- WF

  - Option 1: Legacy transmitted signal quality requirements apply, no impact when BS transmitting LP-WUS signal.

  - Option 2: The OOK-based EVM should be defined for LP-WUS. FFS on how to specify this requirement with consideration of RAN1 waveform design.

The transmitted LP-WUS signal quality (e.g., ON/OFF characteristic) from BS side should be guaranteed and verified, traditional EVM requirement can not be applied to OOK waveform directly. RAN4 should further discuss how to ensure the LP-WUS signal performance.

Proposal 6: OOK-based EVM can be defined for LP-WUS. How to map OOK on-off performance to OOK-based EVM is FFS.

- Conclusion

In this contribution, we share our views on LP-WUR BS RF:

Proposal 1: RAN4 should define a min power boosting level in core specification which is based on optional manufacture declaration.

Proposal 2: RAN4 further discuss whether the min power boosting level could be different value for different CBW.

Proposal 3: No matter which Option is selected as RAN4 LP-WUS power boosting definition, the requirement should consider the assumed case in RAN1 evaluation, i.e., the assumption in RAN1 of 3dB or 6dB means the EPRE ratio.

Proposal 4: For 30kHz SCS LP-WUS operation, power boosting within 5MHz and 10MHz CBW can be precluded. For 15kHz SCS LP-WUS operation, no CBW should be precluded.

Proposal 5: RAN4 consider 3dB as a minimum requirement for LP-WUS power boosting requirement.

Proposal 6: OOK-based EVM can be defined for LP-WUS. How to map OOK on-off performance to OOK-based EVM is FFS.

# References

- R4-2413512, Way Forward for [112][309] NR_LPWUS, Huawei, RAN4#112