3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415784

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	MSD rules and duty cycle related Issues for CA HPUE

Agenda Item:	6.1.1.1.1

Document for:	Approval

- Introduction

The discussion of how to extend the MSD requirements to High power combinations has been existed in multiple agendas even before Rel-19 WI. In Rel-19, this was mainly discussed in Rel-19 UE RF and spec improvement. In RAN4#112, this discussion was mainly taken place in Rel-19 UE RF agenda, and a WF was reached in [1].

The duty cycle solution related contributions have been presented for a few meetings. However, this topic has not been discussed online or in the adhoc yet, thus no progress has been made.

In RAN4#112bis, a dedicated agenda was provided for these two topics is set up. And this contribution is designed for them.

- Discussion

  - MSD Issues

Background

In the WF [1], the following WF has been reached:

Way forward:

- RAN4 can discuss the MSD general rules, and strive to down-select among the following options

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

- Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD

- Other solutions are not precluded

NOTE: RAN4 continues normal procedure specifying MSD for all PC’s band combination supports until down-selection is done

It should be noted that this is basically the same to the earlier agreements in [2] which is as following:

For all types of MSDs:

- Option 1: RAN 4 defines only the default power class inter-band CA and intra-band CA MSD requirements, and the new HPUE MSD requirements are no longer specified in the TS. The legacy agreed HPUE MSD test points are not impacted and are kept in the TS.

- Option 2: RAN 4 defines the default power class inter-band CA and intra-band CA MSD requirements, and RAN4 specifies equations or simplified guidelines (like in [3,4]) or lookup tables “LUT” (like in [5]) that define the relationship between the default power class MSD and the HPUE MSD requirements:

  - Option 2a: Equations/LUT are captured in the RAN4 TS,

  - Option 2b: Equations/LUT are not captured in the RAN4 TS and are provided to RAN5.

- Option 3:  Keep the existing way of working, i.e. RAN4 analyses case by case all HPUE MSD requirements and experts flag TP for TRs for which MSD needs corrections/re-evaluations.

- Other options are not precluded.

It seems that we are still not having too much progress, since the main contents in these two WFs are almost identical. Here, we would like to make some analysis on those options and notes.

On defining only default power class MSD

There is a lot of limitations on MSD. Though this is a sign of receiver capability, this requirement has a lot of limitations, and there is a doubt on whether this is really needed, and a doubt on how much value can be added for reference on whether the network would be benefitted. The paper [3] has a complete summary on this and can serve as a reference.

Pros: This solution is the cleanest solution. It will solve the issues that results from the complexity almost once and for all with almost no, if any, actual losses for the verification coverage. This solution is also simple enough to be implemented, without the need to discuss further detailed solution,

Cons: The main problem for this solution is it is a fairly radical one, which is usually not so preferred by 3GPP. This is after all not so aligned with the tradition.

Observation 1: Although seemingly radical, defining only default power class MSD is the cleanest and actually simplest solution.

On specifying equations and/or LUP tables

This can be regarded as a compromising solution which honour more on legacy procedure and methods.

Pros: This solution would still keep the MSD requirements for hight power combinations, but the requirement values derivations would be more simpler. This can be regarded as a compromise.

Cons: Specifying equations and/or look up tables are not simple. Although there were already contributions such as [4][55], it seems that reaching agreements on different scenarios, including 2Tx/harmonic/IMD etc. are not easy, since different scenarios have different

Observation 2: Specifying equations and/or LUP tables can keep more current procedure, but is not an easy task for all the MSD scenarios.

On “temporarily” keeping legacy procedure

Currently, since no more agreements and conclusion, the “normal procedure” is still effective. There is actually a risk that the “temporary” condition may last forever, if no more conclusions can be made. Even if the principle of specifying equations and/or LUP tables can be reached, this may also meaningless if a complete solution can be reached.

Observation 3: There is a risk that the current “temporary” solution of keep defining MSD would be continue.

However, MSD definition involves more spectral Work Items, and it is a RAN level agreement.

Proposal 1: Prefer defining only default power class MSD.

Proposal 2: If specifying equations and/or LUP tables, the current way of MSD is better suspended, which may involve RAN level discussion.

  - Duty cycle solution

Since this is not discussed, the contents from [6] and [7] are re-submitted.

Scenarios

As the MOP keep increasing with power class, not just SAR compliance will be an issue, the heat dissipation of the device and the stability of RF components in the high-power mode will also be a challenge. If there is no duty cycle limit, the device may keep working in the high-power mode for 100% duty cycle.

PC1.5 of intra-band CA, PC1.5 of inter-band CA, PC1.5 of inter band EN-DC, PC2 of Inter-band EN-DC are proposed to compatible with the legacy duty cycle solution.

PC1.5 of intra-band CA

Considering the duty cycle solution for PC2 of intra-band CA is mandatory and the duty cycle restriction is default, when maxUplinkDutyCycle-PC2-FR1 is absent, the default threshold is 50%. If the duty cycle exceeds the threshold, the power reduction is expected.

Solution for PC1.5 of intra-band CA is extend for the similar way and compatible with PC2, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 3: For PC1.5 of intra-band CA, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of inter-band CA

The duty cycle solution for PC2 of inter-band CA is optional and also specified for 3Tx. When maxUplinkDutyCycle-interBandCA-PC2 is absent, there is no default threshold. The duty cycle solution is only applied when maxUplinkDutyCycle-interBandCA-PC2 is reported. If the capacity maxUplinkDutyCycle-interBandCA-PC2 is reported the duty cycle exceeds it, the power reduction is expected.

The solution for PC1.5 of inter-band CA is proposed to follow the similar way and compatible with PC2. There is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 4a: To compatible with PC2 of inter-band CA, for PC1.5 of inter-band CA, there is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of Inter-band EN-DC(TDD+TDD)

The duty cycle solution for PC2 of Inter-band EN-DC(TDD+TDD) is mandatory and the duty cycle restriction is default, when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, the default threshold is 30%. If the duty cycle exceeds the threshold, the power reduction is expected.

The solution for PC1.5 of Inter-band EN-DC(TDD+TDD) could be extended for the similar way and compatible with PC2, the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

Proposal 4b: To compatible with PC2 of inter-band EN-DC(TDD+TDD), for PC1.5 of Inter-band EN-DC(TDD+TDD) , the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

PC2 of inter-band EN-DC(FDD+FDD)

As PC2 of inter-band EN-DC(FDD+TDD) PC2 is already specified in 38.101-3, the scenario is FDD on LTE and TDD on NR.  On LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2. These two capabilities could be easily reused for FDD+FDD with editorial changes.

The solution for PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

Proposal 4c: For PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

- Conclusion

In this paper, the following observations and proposals are provided:

MSD rules

Observation 1: Although seemingly radical, defining only default power class MSD is the cleanest and actually simplest solution.

Observation 2: Specifying equations and/or LUP tables can keep more current procedure, but is not an easy task for all the MSD scenarios.

Observation 3: There is a risk that the current “temporary” solution of keep defining MSD would be continue.

Proposal 1: Prefer defining only default power class MSD.

Proposal 2: If specifying equations and/or LUP tables, the current way of MSD is better suspended, which may involve RAN level discussion.

Duty Cycle solution (Re-submitssion):

Proposal 3: For PC1.5 of intra-band CA, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 4a: To compatible with PC2 of inter-band CA, for PC1.5 of inter-band CA, there is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 4b: To compatible with PC2 of inter-band EN-DC(TDD+TDD), for PC1.5 of Inter-band EN-DC(TDD+TDD) , the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

Proposal 4c: For PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

- References

- R4-2414277, WF on HPUE for CA in TN, Samsung, RAN4#112

- R4-2410605, WF on HPUE MSD Simplification, Skyworks, Murata, Qualcomm, Samsung, Nokia, RAN4#111

- R4-2411147, Simplifying or removing MSD tables, Apple, RAN4#112

- R4-2413068, Update on Harmonic MSD simplification, Skyworks, RAN4#112

- R4-2408131, Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx, vivo, RAN4#111

- R4-2412092, Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx, vivo, RAN4#112

- R4-2412073, Further discussion on HPUE for intra-band contiguous and non-contiguous CA, vivo, RAN4#112