3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2415804

Hefei, China, October 14th – October 18th, 2024

Source: 	Huawei, HiSilicon

Title: 	On methods for reducing the number of UE Rx chain

Agenda Item:	6.6.2

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104 [1] with following objectives.

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In this contribution, we would like to discuss some general aspects related to this feature.

Discussion

Reference RF architecture

The motivation of current study item is to reduce the number of UE Rx chains that are used to receive the fragmented carriers. To serve this motivation, different RF architectures were mentioned during early discussions, e.g. from separate RF chains per CC to shared RF chains.

The reference RF architecture of non-contiguous CA was initially introduced in LTE Rel-11, and captured in TR 36.823 as below.

Figure 1	Reference RF architecture of non-contiguous CA (Figure 6.2.3.1-1 of TR 36.823)

According to this reference RF architecture, the LNA is shared by two non-contiguous carriers, while the mixer, LPF and A/D are independent for each carrier. This reference RF architecture continues to be used in deriving the NR requirements for non-contiguous CA. Therefore, the current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on this partially shared architecture.

Observation 1: The current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on the partially shared architecture. i.e. the antenna and LNA are shared by both carriers.

Comparing with the fully separated RF architecture, the partially shared architecture saves the cost and on-chip space of antenna and LNA, therefore could be considered as one option of handling the fragmented carriers. The existing requirements could be referred to and no extra requirements need to be studied.

To further reduce the Rx chains, the fully shared architecture is worth thorough study. Below figure is the comparison of fully shared, partially shared and separated architectures. We could tell from the figure that the fully shared architecture could handle the non-contiguous CCs with less Rx chain, or provide additional DL MIMO layer capability with the same number of Rx chains.

Figure 2	Comparison of Fully Shared, Partially Shared and Separated RF architectures

Observation 2: Comparing with partially shared or separated architecture, the fully shared architecture could handle the non-contiguous CCs with less Rx chain, or provide additional DL MIMO layer capability with the same number of Rx chains.

On the other hand, the fully shared architecture will face extra challenges due to shared LNA, LPF and A/D. The pass-band of LPF increases to receive the non-contiguous carriers simultaneously, therefore the out-band rejection might be worse. In FDD bands, it means the Tx leakage will have more impact on the receiver performance, and leads to larger REFSENS relaxation. In addition, the ACS and IBB blocker falling in between the non-contiguous carries can’t be filtered out by the LPF, which might cause the switching of AGC state, and lead to performance degradation.

Observation 3: The RF receiving performance might be degraded with the fully shared architecture.

Proposal 1: Current study item focuses on the fully shared architecture to identify what the major impacts on the RF requirements are, and how much the impacts could be.

In RAN4#112 meeting, ‘supporting only 2Rx in the mandatory 4Rx band’ was also raised as a potential option for RF architecture.

  - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz

This option assumes that UE is allowed to support only 2Rx per CC in the mandatory 4Rx band when intra-band CA is configured. With fully shared RF architecture, UE could receive both CCs on 2Rx, and the remaining 2Rx could be used in another band. We think the assumption needs first to be confirmed in RAN4. As in TS 38.101-1 section 7.2, the test cases other than single carrier REFSENS needs to be verified with four Rx antenna ports, which include the intra-band CA test cases. It’s not very clear if it includes

Observation 4: Clarification might be needed on the assumption that UE is allowed to support only 2Rx per CC in the mandatory 4Rx band when intra-band CA is configured.

If the assumption could be confirmed by RAN4, to our understanding the UE still needs to support 4Rx in the mandatory 4Rx band when there is only one carrier configured on this band. The ability of supporting higher order CA configurations is limited by both the number of antennas and the number of PLLs. For example, a UE supporting inter-band CA between band nX (4Rx mandatory) + B (2Rx band) needs to be equipped with at least 6 Rx antennas and 2 PLLs (Figure 3a). To support NCCA on band nX, both PLL would be used with partially shared or separated architecture, therefore the inter-band CA can’t be supported (Figure 3b).

| a) CA_nXA-nYA | b) CA_nX(2A) |
| --- | --- |

Figure 3	CA ability with partially shared or separated architecture

If fully shared architecture is adopted, the UE could support NCCA on band nX with only one PLL, therefore the higher order inter-band CA configuration could also be supported (Figure 4a). While at the same time it’s not necessary to switch to 2Rx on band nX as suggested in the Option 2 (Figure 4b).

| - CA_nX(2A)-nYA | - CA_nX(2A)-nYA |
| --- | --- |

Figure 4	CA ability with fully shared architecture

Observation 5: The option of ‘supporting only 2Rx in the mandatory 4Rx band’ is not necessary and brings no benefit.

Baseband assumption

In RAN4#102 meeting, companies raised the question on how the baseband assumption will impact the RF requirement, including the assumption on digital filter, and FFT design, etc. Below figure shows the signal flow in the RF chain and baseband.

Figure 5	Signal flow in RF chain and Baseband

To our understanding, the digital filter design might impact the RF requirements when there is in-gap interference between wanted carriers. When the fully shared architecture is adopted, the in-gap interference will not be filtered by LPF therefore will go through the A/D. The digital filter will play the key role of suppressing the in-gap interference. Both single digital filter and separate digital filter are feasible options depending on UE implementation, but the separate digital filter might provide better interference suppressing ability.

After the digital filter, FFT could be executed for each wanted carrier separately, or for the both carriers jointly. In both cases, the wanted carriers will suffer from co-channel noise, which is caused by the spreading in-gap blocker spectrum due to non-orthogonal SCS between blocker and wanted carriers. Below Figure 6 shows the spreading spectrum of the in-gap blocker. If the SCS of in-gap blocker is not aligned with the wanted carrier, the co-channel noise is ~40dB lower than the interference signal. Taking the IBB case2 as an example, the in-gap interference is -44dBm, therefore the co-channel noise would be ~-84dBm. The expected power level of wanted carrier is ~-90dBm (-96.5dBm REFSENS + 6dB for band n25). If the digital filter could provide 10dB interference suppressing ratio, the -1dB SNR (demodulation target) could be easily achieved.

Figure 6	Spreading spectrum of the in-gap blocker due to non-orthogonal SCS

Observation 6: If the digital filter could be designed properly, the baseband assumption has very little impact on the RF requirements.

Proposal 2: Baseband assumption could be up to UE implementation assuming the digital filter could be designed properly, i.e. single/separate digital filter, single/separate FFT are up to UE implementation.

Inter-operator collocated scenario

In the objective of current SI, the inter-operator deployment is limited to co-located scenario, while the PSD difference between operators is for further study.

According the discussion in RAN4#102, it’s very difficult to ensure different operators are always co-located. Even in some cases the operators share the same site, it’s very common that the infrastructures are not shared. As a result, the coverage of different operators is not fully aligned. Below Figure 7 shows a possible multi-operator deployment case.

Figure 7	Multi-operator deployment case

In this example, the UE is at the edge of its serving cell, while almost in the center of the neighbouring operator cell. Taking the BS antenna pattern from TR 38.769 (fixed beam direction) as the assumption for both cells, the UE’s maximum receiving RSRP difference is equal to the antenna gain difference between horizontal angle = 0 degree and +/-60 degree. The Figure 8 shows the RSRP difference could achieve 10dB in this case.

Figure 8	Delta RSRP Multi-operator deployment case

Observation 7: The RSRP difference between operators could achieve 10dB assuming fixed beam direction.

If the deployment includes both macro cell and micro cell, the possible RSRP difference will increase. Also the fast fading will bring more fluctuation to the RSRP difference. For the purpose of study, we could consider a higher level RSRP difference, e.g. 20dB.

Proposal 3: For inter-operator deployment scenario, study the PSD difference of 10dB and 20dB.

Fallback solution

In RAN4#112, there were documents discussing the UE behaviour when fully shared architecture is not applicable due to degraded RF requirements, e.g. large in-gap interference, severe self-interference in FDD bands, etc. We want to provide some initial thoughts on this point.

One of the fallback option could be the legacy CA behaviour, i.e. partially shared or separated RF architecture. However, the UE’s complexity would increase to support dynamically switching between fully shared and partially shared RF architecture. UE needs to be equipped with spare mixers, and the routing would be more complicated.

- Intra-band CA with Fully shared architecture

- Intra-band CA with Partially shared architecture

Figure 9	Dynamic switching between Fully shared and Partially shared architecture

Dynamically switching between fully shared and separated RF architecture would be less complicated, but it leads to reduction of DL MIMO layers on each CC.

- Intra-band CA with Fully shared architecture

- Intra-band CA with Separated architecture

Figure 10	Dynamic switching between Fully shared and Separated architecture

Another fall back option could be switching to non-CA on the relevant band. This option requires the lowest UE implementation complexity, and could reuse the existing signalling scheme.

- CA with Fully shared architecture

- Non-CA

Figure 11	Dynamic switching between CA with Fully shared architecture and Non-CA

Below table summaries the three fallback options. We could tell that the Option 1 of switching to Partially shared architecture will lead to high UE complexity and hardware cost. We prefer not to consider this option.

Comparing with Option 2, the option 3 of switching to Non-CA could provide more DL MIMO layers with low complexity regarding both UE implementation and Network Signalling.

|  | UE Complexity | Signalling Complexity | Hardware cost | DL MIMO layers | DL carriers |
| --- | --- | --- | --- | --- | --- |
| Option 1: CA with Partially shared architecture | High | High | High | 10 | 3 |
| Option 2: CA with Separated architecture | Mid | High | Low | 4 | 2 |
| Option 3: Non-CA | Low | Low | Low | 6 | 2 |

Proposal 4: As fallback solution, don’t consider option 1, i.e. dynamic switching between Fully shared architecture and Partially shared architecture.

Proposal 5: As fallback solution, prioritize option 3, i.e. dynamic switching between CA with Fully shared architecture and Non-CA.

Conclusion

In this contribution we discussed the general aspects of the fragmented carriers. According to the analysis, we have the following observations and proposals:

Observation 1: The current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on the partially shared architecture. i.e. the antenna and LNA are shared by both carriers.

Observation 2: Comparing with partially shared or separated architecture, the fully shared architecture could handle the non-contiguous CCs with less Rx chain, or provide additional DL MIMO layer capability with the same number of Rx chains.

Observation 3: The RF receiving performance might be degraded with the fully shared architecture.

Proposal 1: Current study item focuses on the fully shared architecture to identify what the major impacts on the RF requirements are, and how much the impacts could be.

Observation 4: RAN4 to clarify the assumption that UE is allowed to support only 2Rx per CC in the mandatory 4Rx band when intra-band CA is configured.

Observation 5: The option of ‘supporting only 2Rx in the mandatory 4Rx band’ is not necessary and brings no benefit.

Observation 6: If the digital filter could be designed properly, the baseband assumption has very little impact on the RF requirements.

Proposal 2: Baseband assumption could be up to UE implementation assuming the digital filter could be designed properly, i.e. single/separate digital filter, single/separate FFT are up to UE implementation.

Observation 7: The RSRP difference between operators could achieve 10dB assuming fixed beam direction.

Proposal 3: For inter-operator deployment scenario, study the PSD difference of 10dB and 20dB.

Proposal 4: As fallback solution, don’t consider option 1, i.e. dynamic switching between Fully shared architecture and Partially shared architecture.

Proposal 5: As fallback solution, prioritize option 3, i.e. dynamic switching between CA with Fully shared architecture and Non-CA.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024