3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2415805

Hefei, China, October 14th – October 18th, 2024

Source: 	Huawei, HiSilicon

Title: 	On RF requirements of fragmented carriers

Agenda Item:	6.6.3

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104 [1] with following objectives.

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4];

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In this contribution, we would like to discuss the potentially impacted RF requirements.

Discussion

General

For handling non-contiguous carriers, RF architectures could be categorized into fully shared, partially shared and separated architectures according to the level of shared RF components. Below figure is the comparison of these architectures.

The partially shared architecture is the reference architecture of existing RAN4 requirements in TS 38.101-1 and TS 38.133. To meet the SI objective of reducing Rx chains we propose to focus on studying the performance of fully shared architecture. More detailed analysis could be found in [2].

Proposal 1: RAN4 needs to study the performance of fully shared architecture, taking into account that the baseline reference architecture of existing RAN4 requirements is the partially shared architecture.

Figure 1	Comparison of Fully Shared, Partially Shared and Separated RF architectures

Based on our analysis, at least following RF receiving requirements will be impacted when adopting fully shared architecture: REFSENS, ACS and IBB.

REFSENS

As per discussion in previous meetings, the REFSENS performance of non-contiguous CA will depend on the selection of RF architecture, especially the ΔRIBNC value. Currently the ΔRIBNC is specified in TS 38.101-1 to reflect the required REFSENS regression due to self-interference on FDD bands. Taking band n25 as an example, the ΔRIBNC is specified with three representative configurations.

Figure 2	ΔRIBNC on band n25

The existing ΔRIBNC requirements are derived based on partially shared architecture assumption and the major contributor of the MSD is co-channel interference generated by the non-linearity of PA. When adopting fully shared architecture, the increased bandwidth of LPF, A/D and baseband will introduce additional performance degradation. As illustrated in below Figure 3, the out-band suppression of LPF becomes worse when its passband is adjusted to larger bandwidth to handle the non-contiguous carriers simultaneously. More Tx leakage will fall inside the passband of LPF and go further into the A/D and baseband. To keep reasonable performance of A/D, the AGC might need to switch its work mode and the noise floor will be increased accordingly. In this case, the receiving performance of both PCC and SCC will be deteriorated by the increased noise floor. The final MSD should be decided as MSD=maximum(MSDtx-leakage,MSDco-channel), where MSDtx-leakage is the MSD caused by Tx leakage, and MSDco-channel is the existing MSD caused by co-channel interference.

Figure 3	REFSENS regression in Partially shared and Fully shared architecture

Proposal 2: With fully shared RF architecture, study the degradation of REFSENS on both PCC and SCC caused by the potentially increased noise floor.

ACS/IBB

Similar as REFSENS, the fully shared architecture might cause extra performance degradation to ACS/IBB requirements. The ACS/IBB blocker falls inside the LPF and might trigger the switching of AGC mode, and the noise floor will be increased accordingly. The ACS/IBB requirements for PCC and SCC need to be further relaxed due to the increase of noise floor.

Figure 4	ACS/IBB regression in Partially shared and Fully shared architecture

Taking band n1 as an example, the current ACS/IBB requirements of some representative configurations are summarized as below table. The power level of in-band blocker could be up to -25dBm, and there might be impact on both PCC and SCC due to increased noise floor.

Proposal 3: With fully shared RF architecture, study the degradation of ACS/IBB on both PCC and SCC caused by the potentially increased noise floor.

As mentioned by several companies in RAN4#112, another important scenario worth attention is when the Rx image of in-gap blocker falls into the wanted carrier. Based on some initial evaluation, we think this scenario might face large performance degradation. It will be useful to treat the image scenario as a dedicate sub-test for ACS/IBB.

Proposal 4: For ACS/IBB, the test could be split to sub-test cases for configurations without Rx image and with Rx image.

NBB

In RAN4#112 meeting, companies proposed to further discuss the narrow band blocking requirement. According to TS 38.101-1, the narrow band blocker is placed very close to the wanted carrier, which is less than the nominal channel spacing. Even with the partially shared or separated architecture, the LPF could provide quite limited rejection ability on the NBB blocker. The AGC scheme needs to take the NBB blocker into account. When fully shared architecture is adopted, the performance is not expected to change too much.

Proposal 5: With fully shared RF architecture, the narrow band blocking does not need requirement adjustment.

Conclusion

In this contribution we proposed to study the fully shared architecture, and provided our view on the potentially impacted RF requirements. According to the analysis, we have the following proposals:

Proposal 1: RAN4 needs to study the performance of fully shared architecture, taking into account that the baseline reference architecture of existing RAN4 requirements is the partially shared architecture.

Proposal 2: With fully shared RF architecture, study the degradation of REFSENS on both PCC and SCC caused by the potentially increased noise floor.

Proposal 3: With fully shared RF architecture, study the degradation of ACS/IBB on both PCC and SCC caused by the potentially increased noise floor.

Proposal 4: For ACS/IBB, the test could be split to sub-test cases for configurations without Rx image and with Rx image.

Proposal 5: With fully shared RF architecture, the narrow band blocking does not need requirement adjustment.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- RP-2411691, “On general aspects of fragmented carriers”, ”, Huawei, RAN4#112 Meeting, August 2024